APL SSUSI SDR File

Version 1.16.1

Data version 0116

| Revision | Date | Changes |
| --- | --- | --- |
| 1.16.1 | 3/28/11 | DQI_NIGHT_GAIM and DQI_NIGHT variables now have a bit for the LBH contamination flag. |
| 1.16.0 | 8/23/10 | Added Red Leak correction flag |
| 1.15.1 | 9/21/09 | Added DQI bit for unknown mirror position |
| 1.15.0 | 4/29/09 | Added DQI and EXPOSURE for all grids |
| 1.14.1 | 9/24/08 | Fixed minor typos. Added SOFTWARE_VERSION_NUMBER to header. |
| 1.14.0 | 8/13/08 | Added CALIBRATION_PERIOD_VERSION, SCAN_SDR_THRESHOLD, SCAN_SDR2_THRESHOLD and SAA_427_PHOT_COUNT_THRESHOLD to header. Changed file naming to reflect new AFWA standard naming scheme |
| 1.13.2 | 8/1/08 | Fixed minor typos and word spacing. |
| 1.13.1 | 8/1/08 | Changed data product filenaming convention to include a calibration number, added a section for filenaming conventions and realigned document to data product |
| 1.12.0 | 3/25/07 | Added rebinned low resolution fields for GAIM |
| 1.11.0 | 9/17/07 | Added 630 Photometer data to all disk SDR2 (change made to code as per CalVal Recommendation). Added entire table for new very low resolution (grid cells 9x larger than regular SDR2 grid pixels) for GAIM background Ionosphere model. |
| 1.10.1 | 3/27/07 | Fixed typos. |
| 1.10.0 | 3/22/07 | SOLAR ZENITH ANGLE and IN_SAA now are part of the product |
| 1.9.1 | 3/19/07 | Added proposed format for solar zenith angle |
| 1.9.0 | 2/5/04 | Added LIMB_CALIBRATION_UNCERTAINTY, DISK_CALIBRATION_UNCERTAINTY (for _DAY and _NIGHT), SAA_COUNT, SAA_COUNT_DAY, SAA_COUNT NIGHT, and IN_SAA flags for LIMB, DAY, and NIGHT, added SCAN_MODE |
| 1.8.0 | 1/18/07 | Changed names of geophysical parameters and added calibration flags to support EDR |
| 1.7.1 | 1/4/07 | Fixed description in Introduction |
| 1.7.0 | 11/30/06 | Added effective look angles for day and night, added “geographic” adjective and UNITS attribute to latitude, longitude, and altitude. Added title and units to PIXLESIZE variables to match the current implementation. Added geophysical indices to global attributes. Added description of “*” and color index to Introduction. |
| 1.6.0 | 10/3/06 | Separated out common global attributes. Put SDR2 file into document. Added multiple new DMSP_ fields from L1B. Moved Calibration file parameters to Global attributes. Added NODAL_CROSSING_EPOCH, NODAL_DAY, NODAL_MONTH, and NODAL_YEAR. Added NO_DATA_IN_BIN_VALUE. Restructured description to consolidate common information. Distinguished day and night array sizes. Added DMSP_NADIR_ECI, DMSP_SOLAR_ECI, DMSP_SOLAR_LAT, and DMSP_SOLAR_LON. Removed DMSP_COORDS_ECEF |
| 1.5.0 | 9/14/06 | Added disk fields for orbit number (ORBIT_DAY, ORBIT_NIGHT), day of year (DOY_DAY, DOY_NIGHT), year (YEAR_DAY, YEAR_NIGHT) and changed ACROSSPIXELSIZE and ALONGPIXELSIZE to be actual arclengths on the disk. On the limb added fields for orbit number (ORBIT), day of year (DOY), and year (YEAR) |
| 1.4.0 | 8/21/06 | Removed Time_ephem, replaced with DMSP_COORDS_TIME. Simplified disk day and night grid names to just day or night. Replaced photometer time and time_ephem with photometer_dmsp_time_offset (scalar). Changed data product version to MMmm = 2 digit major version, 2 digit minor version of data format document. |
| 1.3.0 | 7/23/2006 | Commented out fields not filled in, added additional data fields for ephemeris and photometer, added additional day and night satellite time and positions. Removed redundant Limb table. |
| 1.0 | 7/21/2006 | Well we could go to version 1, but we will up the version to 1.3 just as in the L1B case to avoid any confusion with current data versions. |
| 0.9 | 7/19/2006 | Added limb sdr, added separate night and day grids., data version to document |
| 0.8 | 3/9/2006 | Removed DMSP Mission #, doesn’t seem to be used. |
| 0.7 | 1/24/2006 | Initial version of data files. |

Table of Contents

Introduction	4

SDR File Naming Conventions	5

SDR Global File Attributes	6

SDR Limb File Format	8

GAIM-Specific Very Low Resolution Radiance and Geolocation Data in the Limb SDR file	11

SDR (and SDR2/GAIM) Disk File Format	15

Radiances, times, and geolocations in SDR/SDR2 files	15

GAIM-Specific Very Low Resolution Radiance and Geolocation Data in the Disk SDR2 file	23

Auxilary Data in the Disk SDR file	29

# Introduction

The following tables describe the contents of the SSUSI SDR (Sensor Data Record) files produced by IDL code supplied by APL.  The information content of the current IDL produced SDR files is very similar to that created by the older ADA code.  There are two types of SDR file: limb and disk.  Both result from a rebinning of data in the L1B files onto a nearly regularly spaced grid in orbit angle (time) and scan mirror angle.  On the corresponding Pierce Point surface the grid spacing along the time/orbit angle axis is regular, but the grid spacing along scan mirror angle axis can be non-uniform because of the projection of the scan angles onto the Pierce point surface. However the scan angle gridding is such that on the day altitude grid, the surface binsizes are constant, except near the earth’s limb, where the cross-track spacing increases because the SSUSI data density will not support the smaller size pixels.  Similarly, the rebinned scan mirror angle spacing on the limb is designed to produce a uniform spacing in tangent altitude. In the following tables, ‘M’ and ‘N’ are used to designate the array size of the rebinned grid in the across- and along-track directions, respectively.  To distinguish between day and night grids on the disk the array sizes will be ‘Md’, and ‘Nd’ on the day grid and ‘Mn’ and ‘Nn’ on the night grid.

Parameters in the following tables not generated by the current build of the SDR software are marked with a ‘*’.  They will not appear in the  current version of the data files, but they are being considered for inclusion in a later version of the file.  In the tables below, NetCDF attributes (i.e., TITLE and UNITS) associated with variables are listed together with the variable.

There is a dimension (NetCDF name: nchan) for the waveband or “color” of the counts and radiance data.  Currently there are 5 colors (121.6, 130.4, 135.6 nm and LBH short and LBH long) and the channel index goes in that order over the range 0-4.  However there is nothing specific to color in SDR processing and if the order changes, the indices will match that specified in the Level 1B description document.

There are a set of global attributes common to all SDR files; these are described in the next section.  The content of the SDR limb files is described in the section after that.  The description of the fields in the disk files comes last.  It is worth noting that the fields describing the gridded radiances are the same in the two types of disk (sdr and the lower resolution sdr2) files; only the grid sizes are different.  In the high resolution disk sdr file there is also photometer and ephemeris information carried along from the L1B to facilitate EDR processing.  This auxiliary data is described in a separate table in the disk section.

# SDR File Naming Conventions

| SSUSI SDR Data Product | SSUSI EDR Data Product Name |
| --- | --- |
| SSUSI SDR Limb | PS.AFWA_SC.U_DI.A_GP.FXX-SSUSI_PA.APL-SDR-LIMB_DD.YYYYMMDD_SN.oooo-rr_DF.NC FXX denotes bird number, e.g., F16 DD>YYYYMDD: YYYY= 4 digit year MM – 2 digit Month DD - 2 digit day of month SN.ooooo-rr: ooooo – 5 digit orbit number rr – 2 digit data occurrence number. |
| SSUSI SDR Disk | PS.AFWA_SC.U_DI.A_GP.F16-SSUSI_PA.APL-SDR-DISK_DD.20060403_SN.00085-01_DF.NC FXX denotes bird number, e.g., F16 DD>YYYYMDD: YYYY= 4 digit year MM – 2 digit Month DD - 2 digit day of month SN.ooooo-rr: ooooo – 5 digit orbit number rr – 2 digit data occurrence number. |
| SSUSI SDR2/GAIM Disk | PS.AFWA_SC.U_DI.A_GP.F16-SSUSI_PA.APL-SDR2-DISK_DD.20060403_SN.00085-01_DF.NC FXX denotes bird number, e.g., F16 DD>YYYYMDD: YYYY= 4 digit year MM – 2 digit Month DD - 2 digit day of month SN.ooooo-rr: ooooo – 5 digit orbit number rr – 2 digit data occurrence number. |

# SDR Global File Attributes

These attributes are common to sdr, sdr2 and Disk and limb files.

| Variable Name | Type | Dimensions | Value/Comment |
| --- | --- | --- | --- |
| Global file attributes |  |  |  |
| FILENAME | STRING | - | e.g., “PS.APL_V0114S010CD031_SC.U_DI.A_GP.F16-SSUSI_PA.APL-SDR-DISK_DD.20050919_SN.09913-01_DF.NC” |
| MISSION | STRING | - | e.g., “F16” |
| DATA_PRODUCT_TYPE | STRING | - | “SDR Imaging Data” |
| SOURCE | STRING | [#files] | Names of the L1B File(s) used to generate this file. |
| SCAN_TYPE | STRING | - | “DISK” or “LIMB” |
| SCAN_MODE | STRING | - | “REDUCED” or “FULL” |
| DATA_PRODUCT_VERSION | STRING | - | e.g., “0106” |
| DATA_PRODUCT_REVISION | STRING | - | e.g., “001” |
| SOFTWARE_VERSION | STRING | - | e.g., “devel” |
| SOFTWARE_VERSION_NUMBER | STRING | - | CVS package tag of SDR software e.g., “sdr-1-1-1” |
| SOFTWARE_NAME | STRING | - | “APL Level1B to SDR” |
| CALIBRATION_TABLES_NAMES | STRING | - | ‘;’ separated list of calibration files that were used to generate this SDR file. |
| CALIBRATION_TABLES_CREATED | STRING | - | Date when calibration table was created, e.g., “Thu Oct 13 16:15:05 2005” If more than one file is used, a semi-colon separated list is used. |
| CALIBRATION_PERIOD_VERSION | STRING | - | Letter indicating calibration period along with 3 digit number of the calibration version in relation to all possible calibration version files, e.g. D031 implies calibration period D and the calibration table is the 31st version of the calibration file. |
| DESCRIPTION | STRING | - | e.g., “SDR Imaging Mode Data – Reduced Scan” |
| COMMENT | STRING | - | e.g., “No comment” |
| HISTORY | STRING | - | e.g., “No revisions” |
| DATE_GENERATED | STRING | - | e.g., “2005094213027” |
| STARTING_TIME | STRING | - | e.g., “2004150100727” |
| STOPPING_TIME | STRING | - | e.g., “2004150114908” |
| STARTING_ORBIT_NUMBER | FLOAT | - | Orbit number of the starting grid cell e.g., “3159.0” |
| STOPPING_ORBIT_NUMBER | FLOAT | - | Orbit number of the ending grid cell e.g., “3160.0” |
| NODAL_CROSSING_EPOCH | DOUBLE | - | The CDF EPOCH of the Nodal crossing data in the GWC ephemeris from the Prep file. e.g., “63273228090000.” |
| NODAL_DAY | INTEGER | - | The day of month of the Nodal Crossing |
| NODAL_MONTH | INTEGER | - | The month of the Nodal Crossing |
| NODAL_YEAR | INTEGER | - | The year of the Nodal Crossing |
| NO_DATA_IN_BIN_VALUE | FLOAT | - | The value placed in SDR grid cells with no L1B pixels. |
| SCAN_SDR_THRESHOLD | SHORT | - | The minimum number of scans in the L1B file needed in order to successfully generate an SDR file |
| SCAN_SDR2_THRESHOLD | SHORT | - | The minimum number of scans in the L1B file needed in order to successfully generate an SDR2/GAIM disk file |
| SAA_427_PHOT_COUNT_THRESHOLD | INTEGER | - | The number of counts per second seen in the 427 nm photometer that is the threshold for declaring that the spacecraft is the South Atlantic Anomaly. |
| Geophysical information (if available) |  |  |  |
| GEOPHYSICAL_INFO_UPDATE | STRING | - | Last update time, e.g., “20010418:0000” |
| DAY_81_F107 | STRING | - | 81 day average of F10.7 with current day as day 41,e.g., “ 0.000000” |
| DAILY_F107 | STRING | - | Daily average of F10.7 e.g., “ 0.000000” |
| F10_7_SOURCE | STRING | - | Where the F10.7 parameter originated, e.g., “DYN_PARAMS_01108.TXT” |
| HOUR_3_KP | STRING | - | 3 hour average of Kp, e.g., “ 2.00000” |
| DAILY_KP | STRING | - | Daily average of Kp, e.g., “ 2.00000” |
| KP_AP_SOURCE | STRING | - | Source of Kp, Ap indices, e.g., “DYN_PARAMS_01108.TXT” |
| DAILY_AP | STRING | - | Daily average of Ap, e.g., “ 2.00000” |

# SDR Limb File Format

The following table describes the data contents of the SDR Limb data files.  Note that the pixels are geolocated by the tangent point, so there is not a day/night distinction as in the disk files.  Here the NetCDF dimensions (nCross, nAlong) are represented by (M,N).

| Variable Name | Type | Dimensions | Value/Comment |
| --- | --- | --- | --- |
| Time and Position Information |  |  |  |
| TIME TIME_TITLE TIME_UNITS | DOUBLE STRING STRING | [N] - - | The effective time of each rebinned pixel. “Time of each newly rebinned along track pixel” “Seconds since the start of the day” |
| TIME_EPOCH TIME_EPOCH_TITLE TIME_EPOCH_UNITS | DOUBLE STRING STRING | [N] - - | The effective time of each rebinned pixel in milliseconds since 01-Jan-0000 (CDF Epoch value) “Time of each newly rebinned along track pixel” “Epoch milliseconds” |
| YEAR YEAR_TITLE | INT STRING | [N] - | The year of each rebinned pixel “Year of each rebinned pixel” |
| DOY DOY_TITLE | INT STRING | [N] - | The day of year of each rebinned pixel. “Day of year of each rebinned pixel” |
| ORBIT ORBIT TITLE | INT String | [N] - | Orbit numbers for each grid row. “Orbit number of each newly rebinned along track pixel” |
| LATITUDE LATITUDE_TITLE LATITUDE_UNITS | FLOAT STRING STRING | [N] - - | DMSP spacecraft latitude “S/C geographic latitude at the grid centers” “degrees” |
| LONGITUDE LONGITUDE_TITLE LONGITUDE_UNITS | FLOAT STRING STRING | [N] - | DMSP spacecraft longitude “S/C geographic longitude at the grid centers” “degrees” |
| ALTITUDE ALTITUDE_TITLE ALTITUDE_UNITS | FLOAT STRING STRING | [N] - | DMSP spacecraft altitude “S/C geographic altitude at the grid centers” “kilometers” |
| TANGENTPOINT_LATITUDE TANGENTPOINT_LATITUDE_TITLE TANGENTPOINT_LATITUDE_UNITS | FLOAT STRING STRING | [M,N] - | Rebinned pixel tangent point latitude “Tangent point geographic latitude” “degrees” |
| TANGENTPOINT_LONGITUDE TANGENTPOINT_LONGITUDE_TITLE TANGENTPOINT_LONGITUDE UNITS | FLOAT STRING STRING | [M,N] - | Rebinned pixel tangent point longitude “Tangent point geographic longitude” “degrees” |
| TANGENTPOINT_ALTITUDE TANGENTPOINT_ALTITUDE_TITLE TANGENTPOINT_ALTITUDE_UNITS | FLOAT STRING STRING | [M,N] - | Rebinned pixel tangent point altitude “Tangent point geographic altitude” “kilometers” |
| TANGENTPOINT_SZA TANGENTPOINT_SZA.TITLE TANGENTPOINT_SZA.UNITS | FLOAT STRING STRING | [M,N] - - | Tangent point solar zenith angle “solar zenith angle for the center of new limb grid cell.” “degrees” |
| IN_SAA IN_SAA_TITLE | INT STRING | [M,N] | Indicator of whether any limb pixels in the bin may have SAA contamination ‘SAA flag: 1 any pixels in bin were observed while the S/C was in SAA, 0 if not” |
| ACROSSPIXELSIZE | INT | - | Number of across track pixels for the rebinned grid (M). |
| ALONGPIXELSIZE | INT | - | Number of along track pixels for the rebinned grid (N). |
| EFFECTIVELOOKANGLE EFFECTIVELOOKANGLE_TITLE EFFECTIVELOOKANGLE_UNITS | FLOAT STRING STRING | [M,N] - - | Effective scan mirror look angles for the rebinned DAY grid (in degrees). “Effective cross track look angle from S/C nadir to grid tangent points” “Degrees” |
| *ALONGTRACKANGLE | FLOAT | [N] | Effective along-track (orbit) angles for the rebinned grid (in radians, beginning at the equator or the ascending crossing). |
| Re-binned Data |  |  |  |
| *LIMBPIXELDATA LIMBPIXELDATA_TITLE | FLOAT STRING | [M, N, 5] - | Rebinned raw pixel counts for the five SSUSI colors “Raw rebinned limb counts” |
| LIMBCOUNTSDATA LIMBCOUNTSDATA_TITLE | FLOAT STRING | [M, N, 5] - | Rebinned raw pixel counts for the five SSUSI colors “Raw rebinned limb counts” |
| LIMBDECOMP_UNCERTAINTY LIMBDECOMP_ UNCERTAINTY _TITLE | FLOAT STRING | [M, N, 5] - | Decompression uncertainty associated with rebinned counts “Rebinned limb pixel decompression uncertainty” |
| EXPOSURE EXPOSURE.TITLE EXPOSURE.UNITS | FLOAT STRING STRING | [M, N] - - | How many L1B pixel exposures are contained in this grid cell "Number of L1B scan pixels in each limb grid bin" "Count” |
| *LIMBDETECTORRATIO LIMBDETECTORRATIO_TITLE | FLOAT STRING | [M, N] - | Rebinned limb output/input detector ratio “Rebinned limb O/I ratio” |
| *LIMB_PIXELS_IN_BIN LIMB_PIXEL_IN_BIN_TITLE | FLOAT STRING | [M, N] - | Number of raw pixels in each bin “Number of pixels/bin” |
| SAA_COUNT SAA_COUNT_TITLE | INT STRING | [M, N] - | Number of raw pixels in each bin having high 4278 photometer counts (a signature of the South Atlantic Anomaly) “South Atlantic Anomaly count” |
| Calibration parameters |  |  |  |
| DARK_COUNT_CORRECTION DARK_COUNT_CORRECTION_TITLE | INT STRING | - - | Flag indicating whether dark count correction calibration step was performed “Corrected for dark counts (0/1-No/Yes)” |
| SCATTER_LIGHT_1216_CORRECTION SCATTER_LIGHT_1216_CORRECTION_TITLE | INT STRING | - - | Flag indicating whether 1216 scattered light correction was performed “Corrected for 1216 scattered light (0/1-No/Yes)” |
| SCATTER_LIGHT_1304_CORRECTION SCATTER_LIGHT_1304_CORRECTION_TITLE | INT STRING | - - | Flag indicating whether 1304 scattered light correction was performed “Corrected for 1304 scattered light (0/1-No/Yes)” |
| OVERLAP_1304_1356_CORRECTION OVERLAP_1304_1356_CORRECTION_TITLE | INT STRING | - - | Flag indicating whether 1304/1356 overlap correction was performed “Corrected for 1304/1356 overlap (0/1-No/Yes)” |
| LONGWAVE_SCATTER_CORRECTION LONGWAVE_SCATTER_CORRECTION_TITLE | INT STRING | - - | Flag indicating whether long wavelength scattered light correction was performed “Corrected for long  scattered light 0/1-No/Yes)” |
| RED_LEAK_CORRECTION RED_LEAK_CORRECTION_TITLE | INT STRING | - - | Flag indicating whether red leak light correction was performed “Corrected for red leaked light 0/1-No/Yes)” |
| *LIMB_FACTOR LIMB_FACTOR_TITLE | FLOAT STRING | [M, N] - | Rebinned responsivity ratios used in step 7 of calibration (post-9/21/05 data only) “Rebinned responsivity ratio for 1216 correction” |
| *LIMB_RESPONSIVITY LIMB_RESPONSIVITY_TITLE | FLOAT STRING | [M, N, 5] - | Rebinned detector responsivity “Rebinned detector responsivity” |
| *LIMB_1216_BACKGROUND_MASK LIMB_1216_BACKGROUND_MASK_TITLE | FLOAT STRING | [M, N, 5] - | Rebinned 1216 scattered light mask for the limb “Rebinned 1216 background mask” |
| *LIMB_1304_BACKGROUND_MASK LIMB_1304_BACKGROUND_MASK_TITLE | FLOAT STRING | [M, N] - | Rebinned 1304 scattered light mask for the limb “Rebinned 1304 background mask” |
| Calibrated, background-corrected data |  |  |  |
| LIMB_ INTENSITY LIMB_ INTENSITY_TITLE LIMB_ INTENSITY_UNITS | DOUBLE STRING STRING | [M, N, 5] - - | Rebinned limb radiances (in Rayleighs) “Rebinned imaging mode limb radiances” “Rayleighs” |
| LIMB_RADIANCE_UNCERTAINTY LIMB_RADIANCE_UNCERTAINTY_TITLE LIMB_RADIANCE_UNCERTAINTY_UNITS | DOUBLE STRING STRING | [M, N, 5] - - | Uncertainty in rebinned limb radiances due to counting statistics and decompression (in Rayleighs) “Rebinned limb radiance statistical uncertainty” “Rayleighs” |
| LIMB_CALIBRATION_UNCERTAINTY LIMB_CALIBRATION_UNCERTAINTY_TITLE | DOUBLE STRING | [M, N, 5] | Calibration uncertainty in rebinned limb radiances (in Rayleighs) “Rebinned limb radiance calibration uncertainty” |
| *LIMB_INTENSITY_SAA LIMB_INTENSITY_SAA_TITLE | FLOAT STRING | [M, N,3] - | Rebinned limb radiances (in Rayleighs) with the SAA removed. “Imaging Mode Limb Radiance data - corrected for background and SAA, re-binned to new grid” |
| *LIMB_COUNTING_UNCERTAINTY_SAA LIMB_COUNTING_UNCERTAINTY_SAA_TITLE | FLOAT STRING | [M, N, 3] - | Uncertainty in rebinned limb radiances due to counting statistics and decompression (in Rayleighs), with the SAA removed. “Limb Counting Statistical Error; corrected for SAA and rebinned to new grid” |
| DQI DQI.TITLE | INT STRING | [M,N,5] | Data Quality Indicator bitmask showing data condition. Data Quality bit set means: - 0:MeV noise in pixel, - 1: SAA, - 2: unkown mirror position |

## GAIM-Specific Very Low Resolution Radiance and Geolocation Data in the Limb SDR file

This is the same sort of radiance, time, and geolocation information as provided above; however, this is a lower resolution gridding of the data for a better impedance match with the GAIM global grid size.  The pixels here are binned at 3 times longer along track (~300 km along track instead of ~100 km along track).   Here the cross track dimension (NetCDF name: nCross_G) represented by Mg in the table is the same size as nCross above, but the along track dimension (NetCDF name: nAlong_G) represented by Ng is different.

| Variable Name | Type | Dimensions | Value/Comment |
| --- | --- | --- | --- |
| Time and Position Information |  |  |  |
| TIME_GAIM TIME_GAIM.TITLE TIME_GAIM.UNITS | DOUBLE STRING STRING | [Ng] - - | The effective time of each rebinned pixel. “Time of each newly rebinned along track GAIM pixel” “Seconds since the start of the day” |
| TIME_EPOCH_GAIM TIME_EPOCH_GAIM.TITLE TIME_EPOCH_GAIM.UNITS | DOUBLE STRING STRING | [Ng] - - | The effective time of each rebinned pixel. “Time of each newly rebinned along track GAIM pixel” “Epoch Milliseconds” (CDF epoch value) |
| YEAR_GAIM YEAR_GAIM.TITLE | INT STRING | [Ng] - | The year of each rebinned pixel “Year of each newly rebinned GAIM pixel” |
| DOY_GAIM DOY_GAIM.TITLE | INT STRING | [Ng] - | The day of year of each rebinned pixel. “Day of year of each newly rebinned GAIM pixel” |
| ORBIT_GAIM ORBIT GAIM.TITLE | INT String | [Ng] - | Orbit numbers for each grid row. “Orbit number of each newly rebinned along track GAIM pixel” |
| LATITUDE_GAIM LATITUDE_GAIM.TITLE LATITUDE_GAIM.UNITS | FLOAT STRING STRING | [Ng] - - | DMSP spacecraft latitude “S/C geographic latitude at the GAIM grid centers” “degrees” |
| LONGITUDE_GAIM LONGITUDE_GAIM.TITLE LONGITUDE__GAIM.UNITS | FLOAT STRING STRING | [Ng] - | DMSP spacecraft longitude “S/C geographic longitude at the GAIM grid centers” “degrees” |
| ALTITUDE_GAIM ALTITUDE_GAIM.TITLE ALTITUDE_GAIM.UNITS | FLOAT STRING STRING | [Ng] - - | DMSP spacecraft altitude “S/C geographic altitude at the GAIM grid centers” “kilometers” |
| TANGENTPOINT_LATITUDE_GAIM TANGENTPOINT_LATITUDE_GAIM.TITLE TANGENTPOINT_LATITUDE_GAIM.UNITS | FLOAT STRING STRING | [Mg,Ng] - | Rebinned pixel tangent point latitude “Tangent point GAIM geographic latitude” “degrees” |
| TANGENTPOINT_LONGITUDE_GAIM TANGENTPOINT_LONGITUDE_GAIM.TITLE TANGENTPOINT_LONGITUDE_GAIM.UNITS | FLOAT STRING STRING | [Mg,Ng] - | Rebinned pixel tangent point longitude “Tangent point GAIM geographic longitude” “degrees” |
| TANGENTPOINT_ALTITUDE_GAIM TANGENTPOINT_ALTITUDE_GAIM.TITLE TANGENTPOINT_ALTITUDE_GAIM.UNITS | FLOAT STRING STRING | [Mg,Ng] - | Rebinned pixel tangent point altitude “Tangent point GAIM geographic altitude” “kilometers” |
| TANGENTPOINT_SZA_GAIM TANGENTPOINT_SZA_GAIM.TITLE TANGENTPOINT_SZA_GAIM.UNITS | FLOAT STRING STRING | [Mg,Ng] - - | Tangent point solar zenith angle “solar zenith angle for the center of new limb GAIM grid cell.” “degrees” |
| IN_SAA_GAIM IN_SAA__GAIM.TITLE | INT STRING | [Mg,Ng] | Indicator of whether any limb pixels in the bin may have SAA contamination ‘SAA flag: 1 any pixels in GAIM bin were observed while the S/C was in SAA, 0 if not” |
| ACROSSPIXELSIZE_GAIM | INT | - | Number of across track pixels for the rebinned grid (M). |
| ALONGPIXELSIZE_GAIM | INT | - | Number of along track pixels for the rebinned grid (N). |
| EFFECTIVELOOKANGLE_GAIM EFFECTIVELOOKANGLE_GAIM.TITLE EFFECTIVELOOKANGLE_GAIM.UNITS | FLOAT STRING STRING | [Mg,Ng] - - | Effective scan mirror look angles for the rebinned DAY grid (in degrees). “Effective cross track look angle from S/C nadir to GAIM grid tangent points” “Degrees” |
| Re-binned Data |  |  |  |
| LIMBCOUNTSDATA_GAIM LIMBCOUNTSDATA_GAIM.TITLE | FLOAT STRING | [Mg, Ng, 5] - | Rebinned raw pixel counts for the five SSUSI colors “Raw instrument counts in the limb; rebinned to the new GAIM grid” |
| LIMBDECOMP_UNCERTAINTY_GAIM LIMBDECOMP_ UNCERTAINTY_GAIM.TITLE | FLOAT STRING | [Mg, Ng, 5] - | Decompression uncertainty associated with rebinned counts “Decompression errors in the limb; rebinned to the new GAIM grid” |
| EXPOSURE_GAIM EXPOSURE_GAIM.TITLE EXPOSURE_GAIM.UNITS | FLOAT STRING STRING | [Mg, Ng] - - | How many L1B pixel exposures are contained in this grid cell "Number of L1B scan pixels in each limb GAIM grid bin" "Count” |
| SAA_COUNT_GAIM SAA_COUNT_GAIM.TITLE | INT STRING | [Mg, Ng] - | Number of raw pixels in each bin having high 4278 photometer counts (a signature of the South Atlantic Anomaly) “GAIM South Atlantic Anomaly count” |
| Calibrated, background-corrected data |  |  |  |
| LIMB_INTENSITY_GAIM LIMB_INTENSITY_GAIM.TITLE LIMB_INTENSITY_GAIM.UNITS | DOUBLE STRING STRING | [Mg, Ng, 5] - - | Rebinned limb radiances (in Rayleighs) “Imaging mode limb radiance data; rebinned to the new GAIM grid” “Rayleighs” |
| LIMB_RADIANCE_UNCERTAINTY_GAIM LIMB_RADIANCE_UNCERTAINTY_GAIM.TITLE LIMB_RADIANCE_UNCERTAINTY_GAIM.UNITS | DOUBLE STRING STRING | [Mg, Ng, 5] - - | Uncertainty in rebinned limb radiances due to counting statistics and decompression (in Rayleighs) “Limb Radiance error; rebinned to the new GAIM grid” “Rayleighs” |
| LIMB_CALIBRATION_UNCERTAINTY_GAIM LIMB_CALIBRATION_UNCERTAINTY_GAIM.TITLE | DOUBLE STRING | [Mg, Ng, 5] | Calibration uncertainty in rebinned limb radiances (in Rayleighs) “Rebinned limb radiance calibration uncertainty on GAIM grid” |
| DQI_GAIM | INT | [Mg,Ng,5] | Data Quality Indicator for data in bins “Data Quality bitflag: - 0: MeV noise in cell, - 1: SAA - 2: unknown bit position |

# SDR (and SDR2/GAIM) Disk File Format

The following table describes the data content of the SDR and SDR2 (GAIM) disk data files.  All L1B detector pixels are binned and geolocated at two different pierce point altitudes, designated “NIGHT” and “DAY”.  It is up to the user to decide which geolocation value to use.  These grids can be completely independent so that two sets of grid variables are used.  In addition to the cross track and along track pixel numbers M and N, we also need S, the number of instrument scans in order to carry forward some L1B information.  (Md,Nd) are the array sizes for the day (cross, along) track arrays and (Mn,Nn) are the array sizes for the night (cross, along) track arrays. There are two differences between the sdr and sdr2 files: 1) they have different grid resolutions, and 2) the sdr files contain addition photometer, housekeeping, and ephemeris information carried along from the L1B.  The additional data is in a separate table following the common sdr disk data table.

## Radiances, times, and geolocations in SDR/SDR2 files

| Variable Name | Type | Dimensions |  | Value/Comment |  |
| --- | --- | --- | --- | --- | --- |
| Time and Position Information |  |  |  |  |  |
| TIME_DAY TIME_DAY_TITLE TIME_DAY_UNITS | DOUBLE STRING STRING | [Nd] - - | The time of each newly re-binned along track day altitude pixel. ‘Time of each newly re-binned along track pixel’ ‘Seconds since the start of the day’ |  |  |
| TIME_EPOCH_DAY TIME_EPOCH_DAY_TITLE TIME_EPOCH_DAY_UNITS | DOUBLE STRING STRING | [Nd] - - | The time of each newly re-binned along track day altitude pixel. ‘Time of each newly re-binned along track pixel’ ‘Epoch milliseconds’ |  |  |
| YEAR_DAY YEAR_DAY TITLE | INT STRING | [Nd] - | The year of each rebinned day pixel “Year of each rebinned day pixel” |  |  |
| DOY_DAY DOY_DAY_TITLE | INT STRING | [Nd] - | The day of year of each rebinned day pixel. “Day of year of each rebinned day pixel” |  |  |
| ORBIT_DAY ORBIT_DAY TITLE | INT String | [Nd] - | Orbit numbers for each day grid row. “Orbit number of each newly rebinned along track day pixel” |  |  |
| LATITUDE_DAY LATITUDE_DAY_TITLE LATITUDE_DAY_UNITS | FLOAT STRING STRING | [Nd] - - | DMSP S/C Latitude. “S/C geographic latitude, rebinned to the new day grid” “ degrees” |  |  |
| LONGITUDE_DAY LONGITUDE_DAY_TITLE LONGITUDE_DAY_UNITS | FLOAT STRING STRING | [Nd] - - | DMSP S/C Longitude. “S/C geographic longitude, rebinned to the new day grid” “degrees” |  |  |
| ALTITUDE_DAY ALTITUDE_DAY_TITLE ALTITUDE_DAY_UNITS | FLOAT STRING STRING | [Nd] - - | DMSP S/C Altitude. “S/C geographic altitude, rebinned to the new day grid” “km” |  |  |
| TIME_NIGHT TIME_NIGHT_TITLE TIME_NIGHT_UNITS | DOUBLE STRING STRING | [Nn] - - | The time of each newly re-binned along track night altitude pixel. ‘Time of each newly re-binned along track pixel’ ‘Seconds since the start of the day’ |  |  |
| TIME_EPOCH_NIGHT TIME_EPOCH_NIGHT_TITLE TIME_EPOCH_NIGHT_UNITS | DOUBLE STRING STRING | [Nn] - - | The time of each newly re-binned along track night altitude pixel. ‘Time of each newly re-binned along track pixel’ ‘Epoch seconds’ |  |  |
| YEAR_NIGHT YEAR_NIGHT_TITLE | INT STRING | [Nn] - | The year of each rebinned night pixel “Year of each rebinned night pixel” |  |  |
| DOY_NIGHT DOY_NIGHT_TITLE | INT STRING | [Nn] - | The day of year of each rebinned night pixel. “Day of year of each rebinned night pixel” |  |  |
| ORBIT_NIGHT ORBIT NIGHT_TITLE | INT String | [Nn] - | Orbit numbers for each night grid row. “Orbit number of each newly rebinned along track night pixel” |  |  |
| LATITUDE_NIGHT LATITUDE_NIGHT_TITLE LATITUDE_NIGHT_UNITS | FLOAT STRING STRING | [Nn] - - | DMSP S/C Latitude. “S/C Latitude, rebinned to the new night grid” “degrees” |  |  |
| LONGITUDE_NIGHT LONGITUDE_NIGHT_TITLE LONGITUDE_NIGHT_UNITS | FLOAT STRING STRING | [Nn] - - | DMSP S/C Longitude. “S/C Longitude, rebinned to the new night grid” “degrees” |  |  |
| ALTITUDE_NIGHT ALTITUDE_NIGHT_TITLE ALTITUDE_NIGHT_UNITS | FLOAT STRING STRING | [Nn] - - | DMSP S/C Altitude. “S/C Altitude, rebinned to the new night grid” ‘km” |  |  |
| PIERCEPOINT_NIGHT_LATITUDE PIERCEPOINT_NIGHT_LATITUDE_TITLE PIERCEPOINT_NIGHT_LATITUDE_UNITS | FLOAT STRING STRING | [Nn,,Mn] - - | Night Piercepoint latitude of each rebinned pixel center “Nightside latitude of the pierce point; rebinned to the new grid.” ‘degrees” |  |  |
| PIERCEPOINT_NIGHT_LONGITUDE PIERCEPOINT_NIGHT_LONGITUDE_TITLE PIERCEPOINT_NIGHT_LONGITUDE_UNITS | FLOAT STRING STRING | [Nn, Mn] - - | Night Piercepoint longitude of each rebinned pixel center “Nightside longitude of the pierce point; rebinned to the new grid.” “degrees” |  |  |
| PIERCEPOINT_NIGHT_ALTITUDE PIERCEPOINT_NIGHT_ALTITUDE_TITLE PIERCEPOINT_NIGHT_ALTITUDE _UNITS | FLOAT STRING STRING | [Nn,Mn] - - | Night Piercepoint altitude of each rebinned pixel center “Nightside altitude from pierce point location calculations.” “km” |  |  |
| PIERCEPOINT_NIGHT_SZA PIERCEPOINT_NIGHT_SZA.TITLE PIERCEPOINT_NIGHT_SZA.UNITS | FLOAT STRING STRING | [Nd,Md] - - | Night piercepoint solar zenith angle “Nightside solar zenith angle for the center of new grid cell.” “degrees” |  |  |
| PIERCEPOINT_DAY_LATITUDE PIERCEPOINT_DAY_LATITUDE_TITLE PIERCEPOINT_DAY_LATITUDE_UNITS | FLOAT STRING STRING | [Nd,Md] - | Day piercepoint latitude of each rebinned pixel center “Dayside latitude of the pierce point; rebinned to the new grid.” “degrees” |  |  |
| PIERCEPOINT_DAY_LONGITUDE PIERCEPOINT_DAY_LONGITUDE_TITLE PIERCEPOINT_DAY_LONGITUDE_UNITS | FLOAT STRING STRING | [Nd,Md] - | Day Piercepoint longitude of each rebinned pixel center “Dayside longitude of the pierce point; rebinned to the new grid.” “degrees” |  |  |
| PIERCEPOINT_DAY_ALTITUDE PIERCEPOINT_DAY_ALTITUDE_TITLE PIERCEPOINT_DAY_ALTITUDE_UNITS | FLOAT STRING STRING | [Nd,Md] - | Day piercepoint altitude of newly rebinned pixel center “Dayside altitude for all pierce point locations calculations.” “km” |  |  |
| PIERCEPOINT_DAY_SZA PIERCEPOINT_DAY_SZA.TITLE PIERCEPOINT_DAY_SZA.UNITS | FLOAT STRING STRING | [Nd,Md] - - | Day piercepoint solar zenith angle “Dayside solar zenith angle for the center of new grid cell.” “degrees” |  |  |
| IN_SAA_DAY IN_SAA_DAY_TITLE | INT STRING | [Md,Nd] | Indicator of whether any day pixels in the bin may have SAA contamination ‘SAA flag: 1 any pixels in bin were observed while the S/C was in SAA, 0 if not” |  |  |
| IN_SAA_NIGHT IN_SAA_NIGHT_TITLE | INT STRING | [Mn,Nn] | Indicator of whether any night pixels in bin may be contaminated by SAA ‘SAA flag: 1 if any binned pixels were observed while S/C was in SAA, 0 if not” |  |  |
| ACROSSPIXELSIZE_DAY ACROSSPIXELSIZE_DAY_TITLE ACROSSPIXELSIZE_DAY_UNITS | FLOAT STRING STRING | [Md] - - | Arclength on the dayside piercepoint surface of across track pixels. “Piercepoint surface arclengths of across track day pixels" “kilometers” |  |  |
| ALONGPIXELSIZE_DAY ALONGPIXELSIZE_DAY_TITLE ALONGPIXELSIZE_DAY_UNITS | FLOAT STRING STRING | - - - | Arclength on the dayside piercepoint surface of along track pixels "Piercepoint surface arclength of along track day pixels" “kilometers” |  |  |
| ACROSSPIXELSIZE_NIGHT ACROSSPIXELSIZE_NIGHT_TITLE ACROSSPIXELSIZE_NIGHT_UNITS | FLOAT STRING STRING | [Mn] - - | Arclength on the nightside piercepoint surface of across track pixels. “Piercepoint surface arclengths of across track night pixels" “kilometers” |  |  |
| ALONGPIXELSIZE_NIGHT ALONGPIXELSIZE_NIGHT_TITLE ALONGPIXELSIZE_NIGHT_UNITS | FLOAT STRING STRING | - - - | Arclength on the nightside Piercepoint surface of along track pixels. "Piercepoint surface arclength of along track night pixels" “kilometers” |  |  |
| EFFECTIVELOOKANGLE_DAY EFFECTIVELOOKANGLE_DAY_TITLE EFFECTIVELOOKANGLE_DAY_UNITS | FLOAT STRING STRING | [M,N] - - | Effective scan mirror look angles for the rebinned DAY grid (in degrees). “Effective cross track look angle to day grid centers” “Degrees” |  |  |
| EFFECTIVELOOKANGLE_NIGHT EFFECTIVELOOKANGLE_NIGHT_TITLE EFFECTIVELOOKANGLE_NIGHT_UNITS | FLOAT STRING STRING | [M,N] - - | Effective scan mirror look angles for the rebinned NIGHT grid (in degrees). “Effective cross track look angle to night grid centers” “Degrees” |  |  |
| *ACROSSTRACKANGLE | FLOAT | [M] | Across track angle (theta) on the newly created grid. |  |  |
| *ALONGTRACKANGLE | FLOAT | [N] | Along track angle (alpha) on the newly created grid. |  |  |
| Re-binned Data |  |  |  |  |  |
| *DISKPIXELDATA_DAY DISKPIXELDATA_DAY_TITLE | FLOAT STRING | [Md, Nd,5] - |  | Raw pixel data that was re-binned onto the new grid. “Dayside decompressed IO rate corrected counts in the disk; rebinned to the new grid.” |  |
| DISKCOUNTSDATA_DAY DISKCOUNTSDATA_DAY_TITLE DISKCOUNTSDATA_DAY_UNITS | FLOAT STRING STRING | [Md, Nd,5] - - |  | Raw pixel data that was re-binned onto the new dayside grid. “Dayside uncorrected counts in the disk; rebinned to the new grid.” “Uncorrected decompressed counts” |  |
| DISKDECOMP_ UNCERTAINTY _DAY DISKDECOMP_ UNCERTAINTY _DAY_TITLE DISKDECOMP_ UNCERTAINTY _DAY_UNITS | FLOAT STRING STRING | [Md, Nd,5] - - |  | Decompression error uncertainty that was re-binned onto the new grid. “Dayside decompression uncertainty for the disk; rebinned to the new grid.” “Uncorrected decompressed counts” |  |
| EXPOSURE_DAY EXPOSURE_DAY.TITLE EXPOSURE_DAY.UNITS | FLOAT STRING STRING | [Md, Nd, 5] - - |  | How many L1B pixel exposures are contained in this grid cell "Number of L1B scan pixels in each dayside grid bin" "Count” |  |
| *DISKPIXELDATA_NIGHT DISKPIXELDATA_NIGHT_TITLE | FLOAT STRING | [Mn, Nn,5] - |  | Raw pixel data that was re-binned onto the new nightside grid. “Nightside decompressed IO rate corrected counts in the disk; rebinned to the new grid.” |  |
| DISKCOUNTSDATA_NIGHT DISKCOUNTSDATA_NIGHT_TITLE DISKCOUNTSDATA_NIGHT_UNITS | FLOAT STRING STRING | [Mn, Nn,5] - - |  | Raw pixel data that was re-binned onto the new nightside grid. “Nightside uncorrected counts in the disk; rebinned to the new grid.” “Uncorrected decompressed counts” |  |
| DISKDECOMP_UNCERTAINTY_NIGHT DISKDECOMP_ UNCERTAINTY _NIGHT_TITLE DISKDECOMP_ UNCERTAINTY _NIGHT_UNITS | FLOAT STRING STRING | [Mn, Nn,5] - - |  | Decompression error uncertainty that was re-binned onto the new grid. “Nightside decompression uncertainty for the disk; rebinned to the new grid.” “Uncorrected decompressed counts” |  |
| EXPOSURE_NIGHT EXPOSURE_NIGHT.TITLE EXPOSURE_NIGHT.UNITS | FLOAT STRING STRING | [Mn,Nn,5] - - |  | How many L1B pixel exposures are contained in this grid cell "Number of L1B scan pixels in each nightside grid bin" "Count” |  |
| SAA_COUNT_DAY SAA_COUNT_DAY_TITLE | INT STRING | [Md, Nd] - |  | Number of raw pixels in each day bin having high 4278 photometer counts (a signature of the South Atlantic Anomaly) “South Atlantic Anomaly count” |  |
| SAA_COUNT_NIGHT SAA_COUNT_NIGHT TITLE | INT STRING | [Mn, Nn] - |  | Number of raw pixels in each night bin having high 4278 photometer counts (a signature of the South Atlantic Anomaly) “South Atlantic Anomaly count” |  |
| Calibration parameters |  |  |  |  |  |
| *DISKDETECTORRATIO_NIGHT DISKDETECTORRATIO_NIGHT_TITLE | FLOAT STRING | [Mn, Nn] - |  | Disk OI detector ratio that was re-binned onto the new grid. “Nightside O/I ratio data; rebinned in recalibration from L1B processing” |  |
| *DISKDETECTORRATIO_DAY DISKDETECTORRATIO_DAY_TITLE | FLOAT STRING | [Mn, Nn] - |  | Disk OI detector ratio that was re-binned onto the new grid. “Dayside O/I ratio data; rebinned in recalibration from L1B processing” |  |
| DARK_COUNT_CORRECTION DARK_COUNT_CORRECTION_TITLE | INT STRING | - - |  |  | Correct counts for dark counts. “Corrected for Dark counts (1-Yes, 0-No).” |
| SCATTER_LIGHT_1216_CORRECTION SCATTER_LIGHT_1216_CORRECTION_TITLE | INT STRING | - - |  |  | Correct counts for 1216 scattered lights. “Corrected for 1216 scattered light (1-Yes, 0-No).” |
| SCATTER_LIGHT_1304_CORRECTION SCATTER_LIGHT_1304_CORRECTION_TITLE | INT STRING | - - |  |  | Correct counts for 1304 scattered lights. “Corrected for 1304 scattered light (1-Yes, 0-No).” |
| OVERLAP_1304_1356_CORRECTION OVERLAP_1304_1356_CORRECTION_TITLE | INT STRING | - - |  |  | Correct counts for 1304/1356 overlap. “Corrected for 1304/1356 overlap (1-Yes, 0-No).” |
| LONGWAVE_SCATTER_CORRECTION LONGWAVE_SCATTER_CORRECTION_TITLE | INT STRING | - - |  |  | Correct counts for long-wave scattered light. “Corrected for long-wave scattered light (1-Yes, 0-No).” |
| RED_LEAK_CORRECTION RED_LEAK_CORRECTION_TITLE | INT STRING | - - |  |  | Flag indicating whether red leak light correction was performed “Corrected for red leaked light 0/1-No/Yes)” |
| *DISK_FACTOR_NIGHT DISK_FACTOR_NIGHT_TITLE | FLOAT STRING | [M, N] - |  | Responsivity ratios that are used for step 7 of recalibration, for post-9/21/05 even correction when setting the SCATTER_LIGHT_1216_CORRECTION flag. “Nightside responsivity ratio for R / R+1; 1216 correction ” |  |
| *DISK_FACTOR_DAY DISK_FACTOR_DAY _TITLE | FLOAT STRING | [M, N] - |  |  | Responsivity ratios that are used for step 7 of recalibration, for post-9/21/05 even correction when setting the SCATTER_LIGHT_1216_CORRECTION flag. “Dayside responsivity ratio for R / R+1; 1216 correction ” |
| *DISK_1216_BACKGROUND_MASK_NIGHT DISK_1216_BACKGROUND_MASK_NIGHT _TITLE | FLOAT STRING | [5, M, N - |  |  | Nightside 1216 scattered light mask for the disk, rebinned for the new grid. “Disk 1216 Background Mask” |
| *DISK_1304_BACKGROUND_MASK_NIGHT DISK_1304_BACKGROUND_MASK_NIGHT_TITLE | FLOAT STRING | [5, M, N] - |  |  | Nightside 1304 scatter light mask for the disk, rebinned for the new grid. “Disk 1304 Background Mask” |
| *DISK_1216_BACKGROUND_MASK_DAY DISK_1216_BACKGROUND_MASK_DAY_TITLE | FLOAT STRING | [5, M, N - |  |  | Dayside 1216 scattered light mask for the disk, rebinned for the new grid. “Disk 1216 Background Mask” |
| *DISK_1304_BACKGROUND_MASK_DAY DISK_1304_BACKGROUND_MASK_DAY_TITLE | FLOAT STRING | [5, M, N] - |  |  | Dayside 1304 scatter light mask for the limb, rebinned for the new grid. “Limb 1304 Background Mask” |
| Calibrated, background-corrected data |  |  |  |  |  |
| DISK_INTENSITY_NIGHT DISK_INTENSITY_NIGHT_TITLE DISK_INTENSITY_NIGHT_UNITS | DOUBLE STRING STRING | [Mn, Nn, 5] - - |  |  | Re-calibrated disk radiances, re-binned onto the new grid. “Nightside Imaging Mode Disk Radiance data re-binned to new grid” “Rayleighs” |
| DISK_INTENSITY_DAY DISK_INTENSITY_DAY_TITLE DISK_INTENSITY_DAY_UNITS | DOUBLE STRING STRING | [Md, Nd, 5] - |  |  | Re-calibrated disk radiances, re-binned onto the new grid. “Dayside Imaging Mode Disk Radiance data re-binned to new grid” “Rayleighs” |
| *DISK_INTENSITY_NIGHT_SAA DISK_INTENSITY_NIGHT_SAA_TITLE | DOUBLE STRING | [3, M, N] - |  |  | Re-calibrated disk radiances, corrected for the SAA, re-binned onto the new grid. “Nightside Imaging Mode Disk Radiance data – corrected for background and SAA, re-binned to new grid” |
| *DISK_INTENSITY_DAY_SAA DISK_INTENSITY_DAY_SAA_TITLE | DOUBLE STRING | [3, M, N] - |  |  | Re-calibrated disk radiances, corrected for the SAA, re-binned onto the new grid. “Dayside Imaging Mode Disk Radiance data – corrected for background and SAA, re-binned to new grid” |
| DISK_RADIANCE_UNCERTAINTY_NIGHT DISK_RADIANCE_UNCERTAINTY_NIGHT_TITLE DISK_RADIANCE_UNCERTAINTY_NIGHT_UNITS | DOUBLE STRING STRING | [Mn, Nn, 5] - |  |  | Uncertainty in disk values due to counting statistics and decompression, re-binned onto the new grid. “Nightside disk Counting Statistical Error; rebinned to new grid” “Rayleighs” |
| DISK_RADIANCE_UNCERTAINTY_DAY DISK_RADIANCE_UNCERTAINTY_DAY_TITLE DISK_RADIANCE_UNCERTAINTY_DAY_UNITS | DOUBLE STRING STRING | [Md, Nd, 5] - - |  |  | Uncertainty in disk values due to counting statistics and decompression, re-binned onto the new grid. “Dayside disk Counting Statistical Error; rebinned to new grid” “Rayleighs” |
| DISK_RECTIFIED_INTENSITY_NIGHT DISK_RECTIFIED_INTENSITY_NIGHT_TITLE DISK_RECTIFIED_INTENSITY_NIGHT_UNITS | DOUBLE STRING STRING | [Mn, Nn, 5] - |  |  | Re-calibrated and rectified disk radiances, re-binned onto the new grid. “Nightside Imaging Mode Disk Radiance data – corrected for background and look angle, re-binned to new grid” “Rayleighs” |
| DISK_RECTIFIED_INTENSITY_DAY DISK_RECTIFIED_INTENSITY_DAY_TITLE DISK_RECTIFIED_INTENSITY_DAY_UNITS | DOUBLE STRING STRING | [Md, Nd, 5] - |  |  | Re-calibrated and rectified disk radiances, re-binned onto the new grid. “Dayside Imaging Mode Disk Radiance data – corrected for background and look angle, re-binned to new grid” “Rayleighs” |
| DISK_RECTIFIED_RADIANCE_UNCERTAINTY_NIGHT DISK_RECTIFIED_RADIANCE_UNCERTAINTY_NIGHT_TITLE DISK_RECTIFIED_RADIANCE_UNCERTAINTY_NIGHT_UNITS | DOUBLE STRING STRING | [Mn, Nn,5] - |  |  | Uncertainty in rectified disk values due to counting statistics and decompression, re-binned onto the new grid. “Nightside rectified disk Counting Statistical Error; rebinned to new grid” “Rayleighs” |
| DISK_RECTIFIED_RADIANCE_UNCERTAINTY_DAY DISK_RECTIFIED_RADIANCE_UNCERTAINTY_DAY_TITLE DISK_RECTIFIED_RADIANCE_UNCERTAINTY_DAY_UNITS | DOUBLE STRING STRING | [Md, Nd, 5] - |  |  | Uncertainty in rectified disk values due to counting statistics and decompression, re-binned onto the new grid. “Dayside rectified disk Counting Statistical Error; rebinned to new grid” “Rayleighs” |
| DISK_CALIBRATION_UNCERTAINTY_NIGHT DISK_CALIBRATION_UNCERTAINTY_NIGHT_TITLE | DOUBLE STRING | [5, M, N] - |  |  | Disk calibration uncertainty, re-binned onto the new grid. “Nightside Disk Calibration Error; rebinned to new grid” |
| DISK_CALIBRATION_UNCERTAINTY_DAY DISK_CALIBRATION_UNCERTAINTY_DAY_TITLE | DOUBLE STRING | [5, M, N] - |  |  | Disk calibration uncertainty, re-binned onto the new grid. “Dayside Disk Calibration Error; rebinned to new grid” |
| DQI_DAY DQI_DAY.TITLE | SHORT STRING | [Md,Nd,5] - |  |  | Data Quality Indicator for data in dayside grid bins “Dayside Data Quality bitflag. - 0:MeV noise in cell, - 1: SAA, - 2: unknown mirror position” |
| DQI_NIGHT DQI_NIGHT.TITLE | SHORT STRING | [Mn,Nn,5] - |  |  | Data Quality Indicator for data in nightside grid bins “Nightside Data Quality bitflag: - 0:MeV noise in cell, - 1: SAA, - 2: unknown mirror position - 3: LBH Thresh exceeded” |
| Photometer Data |  |  |  |  |  |
| PHOTOMETER_DMSP_TIME_OFFSET PHOTOMETER_DMSP_TIME_OFFSET.TITLE PHOTOMETER_DMSP_TIME_OFFSET.UNITS | FLOAT STRING STRING | - - - |  |  | Scan start time offset from DMSP_COORDS_TIME " Offset between ephemeris time and photometer scan start" “Seconds” |
| TIME_PHOTOMETER TIME_PHOTOMETER.TITLE TIME_PHOTOMETER.UNITS | FLOAT STRING STRING | [S] |  |  | GWC Ephemeris time on Scan nadir cadence “Nadir Time of each scan” “Seconds” |
| PHOTOMETER630_RADIANCE | FLOAT | [S,22] |  |  | Calibrated Radiances from the 630 nm photometer in Rayleighs |
| PHOTOMETER630_VARIANCE |  | [S,22] |  |  | Variances of the 630 nm photometer radiances in Rayleighs**2 |

## GAIM-Specific Very Low Resolution Radiance and Geolocation Data in the Disk SDR2 file

This is the same sort of radiance, time, and geolocation information as provided above; however, this is a much lower resolution gridding of the data for a better impedance match with the GAIM global grid size.  The pixels here are 9 times larger (3 times longer and 3 times wider) than the SSUSI “EDR” sized pixels which are used in the SDR2 table above.

| Variable Name | Type | Dimensions |  | Value/Comment |  |
| --- | --- | --- | --- | --- | --- |
| Time and Position Information |  |  |  |  |  |
| TIME_GAIM_DAY TIME GAIM_DAY_TITLE TIME GAIM_DAY_UNITS | DOUBLE STRING STRING | [Nd] - - | The time of each newly re-binned along track day altitude pixel. ‘Time of each newly re-binned along track GAIM pixel’ ‘Seconds since the start of the day’ |  |  |
| YEAR GAIM_DAY YEAR_GAIM_DAY TITLE | INT STRING | [Nd] - | The year of each rebinned day pixel “Year of each newly rebinned for GAIM day pixel” |  |  |
| DOY_GAIM_DAY DOY GAIM_DAY_TITLE | INT STRING | [Nd] - | The day of year of each rebinned day pixel. “Day of year of each newly rebinned for GAIM day pixel” |  |  |
| ORBIT GAIM_DAY ORBIT GAIM_DAY TITLE | INT String | [Nd] - | Orbit numbers for each day grid row. “Orbit number of each newly rebinned for GAIM along track day pixel” |  |  |
| LATITUDE_GAIM_DAY LATITUDE_GAIM_DAY_TITLE LATITUDE_GAIM_DAY_UNITS | FLOAT STRING STRING | [Nd] - - | DMSP S/C Latitude. “S/C geographic latitude, rebinned to the new GAIM day grid” “ degrees” |  |  |
| LONGITUDE_GAIM_DAY LONGITUDE_GAIM_DAY_TITLE LONGITUDE_GAIM_DAY_UNITS | FLOAT STRING STRING | [Nd] - - | DMSP S/C Longitude. “S/C geographic longitude, rebinned to the new GAIM day grid” “degrees” |  |  |
| ALTITUDE_GAIM_DAY ALTITUDE_GAIM_DAY_TITLE ALTITUDE_GAIM_DAY_UNITS | FLOAT STRING STRING | [Nd] - - | DMSP S/C Altitude. “S/C geographic altitude, rebinned to the new GAIM day altitudegrid” “km” |  |  |
| TIME_GAIM_NIGHT TIME GAIM_NIGHT_TITLE TIME GAIM_NIGHT_UNITS | DOUBLE STRING STRING | [Nn] - - | The time of each newly re-binned along track night altitude pixel. ‘Time of each newly re-binned for GAIM along track pixel’ ‘Seconds since the start of the day’ |  |  |
| YEAR GAIM_NIGHT YEAR GAIM_NIGHT_TITLE | INT STRING | [Nn] - | The year of each rebinned night pixel “Year of each newly rebinned for GAIM night pixel” |  |  |
| DOY GAIM_NIGHT DOY GAIM_NIGHT_TITLE | INT STRING | [Nn] - | The day of year of each rebinned night pixel. “Day of year of each newly rebinned for GAIM night pixel” |  |  |
| ORBIT GAIM_NIGHT ORBIT GAIM NIGHT_TITLE | INT String | [Nn] - | Orbit numbers for each night grid row. “Orbit number of each newly rebinned for GAIM along track night pixel” |  |  |
| LATITUDE GAIM_NIGHT LATITUDE GAIM_NIGHT_TITLE LATITUDE GAIM_NIGHT_UNITS | FLOAT STRING STRING | [Nn] - - | DMSP S/C Latitude. “S/C Latitude, rebinned to the new GAIM night grid” “degrees” |  |  |
| LONGITUDE GAIM_NIGHT LONGITUDE GAIM_NIGHT_TITLE LONGITUDE GAIM_NIGHT_UNITS | FLOAT STRING STRING | [Nn] - - | DMSP S/C Longitude. “S/C Longitude, rebinned to the new GAO< night grid” “degrees” |  |  |
| ALTITUDE GAIM_NIGHT ALTITUDE GAIM_NIGHT_TITLE ALTITUDE GAIM_NIGHT_UNITS | FLOAT STRING STRING | [Nn] - - | DMSP S/C Altitude. “S/C Altitude, rebinned to the new GAIM night altitude grid” ‘km” |  |  |
| PIERCEPOINT GAIM_NIGHT_LATITUDE PIERCEPOINT GAIM_NIGHT_LATITUDE_TITLE PIERCEPOINT GAIM_NIGHT_LATITUDE_UNITS | FLOAT STRING STRING | [Mn, Nn] - - | Night Piercepoint latitude of each rebinned pixel center “Nightside latitude of the pierce point; rebinned to the new GAIM grid.” ‘degrees” |  |  |
| PIERCEPOINT GAIM_NIGHT_LONGITUDE PIERCEPOINT GAIM_NIGHT_LONGITUDE_TITLE PIERCEPOINT GAIM_NIGHT_LONGITUDE_UNITS | FLOAT STRING STRING | [Mn, Nn] - - | Night Piercepoint longitude of each rebinned pixel center “Nightside longitude of the pierce point; rebinned to the new GAIM grid.” “degrees” |  |  |
| PIERCEPOINT GAIM_NIGHT_ALTITUDE PIERCEPOINT GAIM_NIGHT_ALTITUDE_TITLE PIERCEPOINT GAIM_NIGHT_ALTITUDE _UNITS | FLOAT STRING STRING | [Mn, Nn] - - | Night Piercepoint altitude of each rebinned pixel center “Nightside pierce point altitude for the new GAIM night grid.” “km” |  |  |
| PIERCEPOINT GAIM_NIGHT_SZA PIERCEPOINT GAIM_NIGHT_SZA.TITLE PIERCEPOINT GAIM_NIGHT_SZA.UNITS | FLOAT STRING STRING | [Md, Nd] - - | Night piercepoint solar zenith angle “Nightside solar zenith angle for the center of new GAIM grid cell.” “degrees” |  |  |
| PIERCEPOINT GAIM_DAY_LATITUDE PIERCEPOINT GAIM_DAY_LATITUDE_TITLE PIERCEPOINT GAIM_DAY_LATITUDE_UNITS | FLOAT STRING STRING | [Md, Nd] - | Day piercepoint latitude of each rebinned pixel center “Dayside latitude of the pierce point; rebinned to the new GAIM grid.” “degrees” |  |  |
| PIERCEPOINT GAIM_DAY_LONGITUDE PIERCEPOINT GAIM_DAY_LONGITUDE_TITLE PIERCEPOINT GAIM_DAY_LONGITUDE_UNITS | FLOAT STRING STRING | [Md, Nd] - | Day Piercepoint longitude of each rebinned pixel center “Dayside longitude of the pierce point; rebinned to the new GAIM grid.” “degrees” |  |  |
| PIERCEPOINT GAIM_DAY_ALTITUDE PIERCEPOINT GAIM_DAY_ALTITUDE_TITLE PIERCEPOINT GAIM_DAY_ALTITUDE_UNITS | FLOAT STRING STRING | [Md, Nd] - | Day piercepoint altitude of newly rebinned pixel center “Dayside pierce point altitudes for the new GAIM grid” “km” |  |  |
| PIERCEPOINT GAIM_DAY_SZA PIERCEPOINT GAIM_DAY_SZA.TITLE PIERCEPOINT GAIM_DAY_SZA.UNITS | FLOAT STRING STRING | [Md, Nd] - - | Day piercepoint solar zenith angle “Dayside solar zenith angle for the center of new GAIM grid cell.” “degrees” |  |  |
| IN_SAA GAIM_DAY IN_SAA GAIM_DAY_TITLE | INT STRING | [Md,Nd] | Indicator of whether any day pixels in the bin may have SAA contamination ‘SAA GAIM grid cell flag: 1 any pixels in bin were observed while the S/C was in SAA, 0 if not” |  |  |
| IN_SAA GAIM_NIGHT IN_SAA GAIM_NIGHT_TITLE | INT STRING | [Mn,Nn] | Indicator of whether any night pixels in bin may be contaminated by SAA ‘SAA GAIM grid cell flag: 1 if any binned pixels were observed while S/C was in SAA, 0 if not” |  |  |
| ACROSSPIXELSIZE GAIM_DAY ACROSSPIXELSIZE GAIM_DAY_TITLE ACROSSPIXELSIZE GAIM_DAY_UNITS | FLOAT STRING STRING | [Md] - - | Arclength on the dayside piercepoint surface of across track pixels. “Piercepoint surface arclengths of across track GAIM day pixels" “kilometers” |  |  |
| ALONGPIXELSIZE GAIM_DAY ALONGPIXELSIZE GAIM_DAY_TITLE ALONGPIXELSIZE GAIM_DAY_UNITS | FLOAT STRING STRING | - - - | Arclength on the dayside piercepoint surface of along track pixels "Piercepoint surface arclength of along track GAIM day pixels" “kilometers” |  |  |
| ACROSSPIXELSIZE GAIM_NIGHT ACROSSPIXELSIZE GAIM_NIGHT_TITLE ACROSSPIXELSIZE GAIM_NIGHT_UNITS | FLOAT STRING STRING | [Mn] - - | Arclength on the nightside piercepoint surface of across track pixels. “Piercepoint surface arclengths of across track GAIM night pixels" “kilometers” |  |  |
| ALONGPIXELSIZE GAIM_NIGHT ALONGPIXELSIZE GAIM_NIGHT_TITLE ALONGPIXELSIZE GAIM_NIGHT_UNITS | FLOAT STRING STRING | - - - | Arclength on the nightside Piercepoint surface of along track pixels. "Piercepoint surface arclength of along track GAIM night pixels" “kilometers” |  |  |
| EFFECTIVELOOKANGLE GAIM_DAY EFFECTIVELOOKANGLE GAIM_DAY_TITLE EFFECTIVELOOKANGLE GAIM_DAY_UNITS | FLOAT STRING STRING | [M,N] - - | Effective scan mirror look angles for the rebinned DAY grid (in degrees). “Effective cross track look angle to day GAIM grid centers” “Degrees” |  |  |
| EFFECTIVELOOKANGLE GAIM_NIGHT EFFECTIVELOOKANGLE GAIM_NIGHT_TITLE EFFECTIVELOOKANGLE GAIM_NIGHT_UNITS | FLOAT STRING STRING | [M,N] - - | Effective scan mirror look angles for the rebinned NIGHT grid (in degrees). “Effective cross track look angle to night GAIM grid centers” “Degrees” |  |  |
| Re-binned Data |  |  |  |  |  |
| DISKCOUNTSDATA GAIM_DAY DISKCOUNTSDATA GAIM_DAY_TITLE DISKCOUNTSDATA GAIM_DAY_UNITS | FLOAT STRING STRING | [Md, Nd,5] - - |  | Raw pixel data that was re-binned onto the new dayside grid. “Dayside uncorrected decompressed counts in the disk; rebinned to the new GAIM grid.” “Uncorrected decompressed counts” |  |
| DISKDECOMP_ UNCERTAINTY GAIM _DAY DISKDECOMP_ UNCERTAINTY GAIM_DAY_TITLE DISKDECOMP_ UNCERTAINTY GAIM_DAY_UNITS | FLOAT STRING STRING | [Md, Nd,5] - - |  | Decompression error uncertainty that was re-binned onto the new grid. “Dayside decompression uncertainty for the disk; rebinned to the new grid.” “Uncorrected decompressed counts” |  |
| EXPOSURE_DAY_GAIM EXPOSURE_DAY_GAIM.TITLE EXPOSURE_DAY_GAIMUNITS | FLOAT STRING STRING | [Md,Nd,5] - - |  | How many L1B pixel exposures are contained in this grid cell "Number of L1B scan pixels in each dayside GAIM grid bin" "count” |  |
| DISKCOUNTSDATA GAIM_NIGHT DISKCOUNTSDATA GAIM_NIGHT_TITLE DISKCOUNTSDATA GAIM_NIGHT_UNITS | FLOAT STRING STRING | [Mn, Nn,5] - - |  | Raw pixel data that was re-binned onto the new nightside grid. “Nightside uncorrected decompressed counts in the disk; rebinned to the new GAIM grid.” “Uncorrected decompressed counts” |  |
| DISKDECOMP_UNCERTAINTY GAIM_NIGHT DISKDECOMP_ UNCERTAINTY GAIM _NIGHT_TITLE DISKDECOMP_ UNCERTAINTY GAIM_NIGHT_UNITS | FLOAT STRING STRING | [Mn, Nn,5] - - |  | Decompression error uncertainty that was re-binned onto the new grid. “Nightside decompression uncertainty for the disk; rebinned to the new GAIM grid.” “Uncorrected decompressed counts” |  |
| EXPOSURE_NIGHT_GAIM EXPOSURE_NIGHT_GAIM.TITLE EXPOSURE_NIGHT._GAIMUNITS | FLOAT STRING STRING | [Mn,Nn,5] - - |  | How many L1B pixel exposures are contained in this grid cell "Number of L1B scan pixels in each nightside GAIM grid bin" "Count” |  |
| SAA_COUNT GAIM_DAY SAA_COUNT GAIM_DAY_TITLE | INT STRING | [Md, Nd] - |  | Number of raw pixels in each day bin having high 4278 photometer counts (a signature of the South Atlantic Anomaly) “South Atlantic Anomaly count GAIM day grid” |  |
| SAA_COUNT GAIM_NIGHT SAA_COUNT GAIM_NIGHT TITLE | INT STRING | [Mn, Nn] - |  | Number of raw pixels in each night bin having high 4278 photometer counts (a signature of the South Atlantic Anomaly) “South Atlantic Anomaly count GAIM night grid” |  |
| Calibrated, background-corrected data |  |  |  |  |  |
| DISK_INTENSITY GAIM_NIGHT DISK_INTENSITY GAIM_NIGHT_TITLE DISK_INTENSITY GAIM_NIGHT_UNITS | DOUBLE STRING STRING | [Mn, Nn, 5] - - |  |  | Re-calibrated disk radiances, re-binned onto the new grid. “Nightside Imaging Mode Disk Radiance data re-binned to new GAIM grid” “Rayleighs” |
| DISK_INTENSITY GAIM_DAY DISK_INTENSITY GAIM_DAY_TITLE DISK_INTENSITY GAIM_DAY_UNITS | DOUBLE STRING STRING | [Md, Nd, 5] - |  |  | Re-calibrated disk radiances, re-binned onto the new grid. “Dayside Imaging Mode Disk Radiance data re-binned to new GAIM grid” “Rayleighs” |
| DISK_RADIANCE_UNCERTAINTY GAIM_NIGHT _TITLE _UNITS | DOUBLE STRING STRING | [Mn, Nn, 5] - |  |  | Uncertainty in disk values due to counting statistics and decompression, re-binned onto the new grid. “Nightside disk Counting Statistical Error; rebinned to new GAIM grid” “Rayleighs” |
| DISK_RADIANCE_UNCERTAINTY GAIM_DAY DISK_RADIANCE_UNCERTAINTY GAIMDAY_TITLE DISK_RADIANCE_UNCERTAINTY GAIM_DAY_UNITS | DOUBLE STRING STRING | [Md, Nd, 5] - - |  |  | Uncertainty in disk values due to counting statistics and decompression, re-binned onto the new grid. “Dayside disk Counting Statistical Error; rebinned to new GAIM grid” “Rayleighs” |
| DISK_RECTIFIED_INTENSITY GAIM_NIGHT DISK_RECTIFIED_INTENSITY GAIM_NIGHT_TITLE DISK_RECTIFIED_INTENSITY GAIM_NIGHT_UNITS | DOUBLE STRING STRING | [Mn, Nn, 5] - |  |  | Re-calibrated and rectified disk radiances, re-binned onto the new grid. “Nightside Imaging Mode Disk Radiance data – corrected for background and look angle, re-binned to new GAIM grid” “Rayleighs” |
| DISK_RECTIFIED_INTENSITY GAIM_DAY DISK_RECTIFIED_INTENSITY GAIM_DAY_TITLE DISK_RECTIFIED_INTENSITY GAIM_DAY_UNITS | DOUBLE STRING STRING | [Md, Nd, 5] - |  |  | Re-calibrated and rectified disk radiances, re-binned onto the new grid. “Dayside Imaging Mode Disk Radiance data – corrected for background and look angle, re-binned to new GAIM grid” “Rayleighs” |
| DISK_RECTIFIED_RADIANCE_UNCERTAINTY_ GAIM_NIGHT DISK_RECTIFIED_RADIANCE_UNCERTAINTY_ GAIM_NIGHT_TITLE DISK_RECTIFIED_RADIANCE_UNCERTAINTY_ GAIM_NIGHT_UNITS | DOUBLE STRING STRING | [Mn, Nn,5] - - |  |  | Uncertainty in rectified disk values due to counting statistics and decompression, re-binned onto the new grid. “Nightside rectified disk Counting Statistical Error; rebinned to new GAIM grid” “Rayleighs” |
| DISK_RECTIFIED_RADIANCE_UNCERTAINTY_ GAIM_DAY DISK_RECTIFIED_RADIANCE_UNCERTAINTY_ GAIM_DAY_TITLE DISK_RECTIFIED_RADIANCE_UNCERTAINTY_ GAIM_DAY_UNITS | DOUBLE STRING STRING | [Md, Nd, 5] - - |  |  | Uncertainty in rectified disk values due to counting statistics and decompression, re-binned onto the new grid. “Dayside rectified disk Counting Statistical Error; rebinned to new GAIM grid” “Rayleighs” |
| DISK_CALIBRATION_UNCERTAINTY GAIM_NIGHT DISK_CALIBRATION_UNCERTAINTY_ GAIM_NIGHT_TITLE | DOUBLE STRING | [M, N, 5] - |  |  | Disk calibration uncertainty, re-binned onto the new grid. “Nightside Disk Calibration Error; rebinned to new grid” |
| DISK_CALIBRATION_UNCERTAINTY GAIM_DAY DISK_CALIBRATION_UNCERTAINTY GAIM_DAY_TITLE | DOUBLE STRING | [M, N, 5] - |  |  | Disk calibration uncertainty, re-binned onto the new grid. “Dayside Disk Calibration Error; rebinned to new GAIM grid” |
| DQI_DAY_GAIM DQI_DAY.TITLE | SHORT STRING | [Md,Nd,5] - |  |  | Data Quality Indicator for data in dayside GAIM grid bins “Dayside GAIM Data Quality bitflag: - 0:MeV noise in cell, - 1: SAA, - 2: unknown mirror position” |
| DQI_NIGHT_GAIM DQI_NIGHT.TITLE | SHORT STRING | [Mn,Nn,5] - |  |  | Data Quality Indicator for data in nightside GAIM grid bins “Nightside GAIM Data Quality bitflag: - 0:MeV noise in cell, - 1: SAA, - 2: unknown mirror position - 3: LBH Thresh exceeded” |

## Auxiliary Data in the Disk SDR file

The following table describes data fields in the disk sdr file that is passed along from the L1B in order to be used in EDR processing. The number of raw scans in the file is specified by the letter ‘S’.

GAIM

| Variable Name | Type | Dimensions | Value/Comment |
| --- | --- | --- | --- |
| Ephemeris Information from RSDR |  |  |  |
| COSCROSSINGANGLE | FLOAT | [S] | The cosine of crossing angle of subsatellite track with respect to local meridian. See p. 25 of DMSP Data Specification (IS-YD-821) Rev C. |
| COSLUNARAZIMUTH | FLOAT | [S] | Azimuth of moon relative to direction of satellite flight. However, this parameter seems to have been improperly scaled as it has a value greater than 1. It also exhibits discrete “stair step” behavior. |
| COSSOLARAZIMUTH | FLOAT | [S] | Azimuth of sun relative to direction of satellite flight. It has a value of close to 1 for F16, which doesn’t seem quite right. |
| EPHEMCLOCKPT1 | INT | [S] | Divided by 16 to get a time-like variable in units of seconds that rolls over every 512 seconds. This variable has zeros every ~53 seconds. |
| EPHEMCLOCKPT2 | INT | [S] | Multiply by 8 and then add (EPHEMCLOCKPT1 mod 128) divided by 16 to get the time in elapsed seconds since 0000Z (I think this is the right algorithm). This variable has zeros every ~53 seconds. |
| HEIGHTEARTHRADIUSRATIO | FLOAT | [S] | Ratio of altitude of spacecraft above subsatellite point at mean sea level to the Earth’s radius (R = 6378.145 km). This has a value of ~4400. |
| LUNARELEVATION | FLOAT | [S] | Elevation of moon with respect to local orbit tangent plane (in degrees). |
| LUNARPHASEANGLE | FLOAT | [S] | Angle between lunar and solar position vectors. Units should be degrees, but this has an integer value between 5 and 7, so it does not seem right. |
| SOLARELEVATION | FLOAT | [S] | Elevation of sun with respect to local orbit tangent plane (in degrees). |
| SUBPOINTLAT | FLOAT | [S] | Geodetic latitude of subsatellite point, WGS-72 (degrees). The values are very close to but not quite the same as those in LAT_1. |
| SUBPOINTLONG | FLOAT | [S] | Longitude of subsatellite point, WGS-72 (degrees, from –180 to 180). The values are very close to but not quite the same as those in LONGITUDE.. |
| TIME TIME_TITLE | DOUBLE STRING | [S] - | The time of each scan in seconds since the start of the day. “Time of each scan” |
| JULDAY | INT | [S] | Precomputed GWC ephemeris: Julian day. Previously named JULDAY_1. |
| SATH | FLOAT | [S] | Precomputed GWC ephemeris: ?. Not sure what this is, but it runs from 0 to 360 degrees within one file, so it looks like an orbit angle. Previously named SATH_1. |
| Ephemeris Global Attributes |  |  |  |
| *EPHEMERIS_CODE | STRING | - | e.g., “SGP4 VERSION 1.1” |
| *TLE_LINE1 | STRING | - | First line of the TLE |
| *TLE_LINE2 | STRING | - | Second line of the TLE |
| *TLE_SOURCE | STRING | - | Origin of TLE file |
| *TLE_DATE | STRING | - | Date of TLE file creation |
| *TLE_FILE_NAME | STRING | - | Name of TLE file as stored at APL |
| *EPHEMERIS_CREATION_DATE | STRING | - | e.g., “12/25/2004 23:59” |
| Ephemeris data generated in L1B code |  |  |  |
| DMSP_LATITUDE DMSP_LATITUDE_TITLE DMSP_LATITUDE_UNITS | FLOAT STRING STRING | [S,22] - - | DMSP latitude. For first delivery, using smoothed and filtered GWC ephemeris data. "Geodetic latitude of the DMSP spacecraft at 1 second resolution" "Degrees” |
| DMSP_LONGITUDE DMSP_LONGITUDE_TITLE DMSP_LONGITUDE_UNITS | FLOAT STRING STRING | [S,22] - - | DMSP east longitude. For first delivery, using smoothed and filtered GWCephem data. "Geodetic longitude of the DMSP spacecraft at 1 second resolution" "Degrees” |
| DMSP_RADIAL_DISTANCE DMSP_RADIAL_DISTANCE_TITLE DMSP_RADIAL_DISTANCE_UNITS | FLOAT STRING STRING | [S,22] - - | DMSP radial distance. For first delivery, using smoothed and filtered GWCephem data. “Radial distance of the DMSP spacecraft from center of earth at 1 second resolution” “km” |
| DMSP_ALTITUDE DMSP_ALTITUDE_UNITS DMSP_ALTITUDE_UNITS | FLOAT STRING STRING | [S,22] - - | DMSP altitude. For first delivery, using smoothed and filtered GWC ephemeris data. "Altitude of the DMSP spacecraft from the surface of the earth at 1 second resolution" “km” |
| DMSP_COORDS_ECI DMSP_COORDS_ECI_TITLE DMSP_COORDS_ECI_UNITS | FLOAT STRING STRING | [S,3,22] - - | DMSP location in ECI coordinates. For first delivery, derived using smoothed and filtered GWCephem data. "ECI coordinates of the DMSP spacecraft at 1 second resolution" “km” |
| DMSP_NADIR_ECI DMSP_NADIR_ECI_TITLE DMSP_NADIR_ECI_UNITS | FLOAT STRING STRING | [S,3,22] - - | DMSP nadir vector in ECI coordinates. For first delivery, derived using smoothed and filtered GWCephem data. "ECI coordinates of the DMSP spacecraft nadir position at 1 second resolution” “km” |
| DMSP_SOLAR_ECI DMSP_SOLAR_ECI_TITLE DMSP_SOLAR_ECI_UNITS | FLOAT STRING STRING | [S,3,22] - - | Solar location in ECI coordinates. For first delivery, based smoothed and filtered GWCephem data. "ECI position of the sun at 1 second resolution" “km” |
| DMSP_SOLAR_LAT DMSP_SOLAR_LAT_TITLE DMSP_SOLAR_LAT_UNITS | FLOAT STRING STRING | [S,22] - - | Subsolar latitude. For first delivery, derived using smoothed and filtered GWCephem data. "Subsolar latitude at 1 second resolution" “degrees” |
| DMSP_SOLAR_LON DMSP_SOLAR_LON_TITLE DMSP_SOLAR_LON_UNITS | FLOAT STRING STRING | [S,22] - - | Subsolar longitude. For first delivery, derived using smoothed and filtered GWCephem data. "Subsolar longitude at 1 second resolution" “degrees” |
| DMSP_COORDS_TIME DMSP_COORDS_TIME_TITLE DMSP_COORDS_TIME_UNITS | FLOAT STRING STRING | [S,22] - - | 1 second time steps used to generate the “DMSP_” tagged coordinates "Time corresponding to each DMSP coordinate at 1 second resolution” “Seconds” |
| Photometer Data |  |  |  |
| PHOTOMETER427COUNT | LONG | [S,22] |  |
| PHOTOMETER629COUNT | LONG | [S,22] |  |
| PHOTOMETER630COUNT | LONG | [S,22] |  |
| PHOTOMETERSTATUS | INT | [S] |  |
| ILLUMSENSOR1 | INT | [S] |  |
| ILLUMSENSOR2 | INT | [S] |  |
| LAT_PHOTOMETER | FLOAT | [S,22] | Latitude of each photometer radiance |
| LON_PHOTIMETER | FLOAT | [S,22] | Longitude of each photometer radiance |
| ALT_PHOTOMETER | FLOAT | [S,22] | Altitude of the photometer for each radiance |
| PULSEHEIGHTDATA | INT | [S,22] |  |
| PHOTOMETER427_RADIANCE | FLOAT | [S,22] | Calibrated Radiances from the 427 nm photometer in Rayleighs |
| PHOTOMETER629_RADIANCE | FLOAT | [S,22] | Calibrated Radiances from the 629 nm photometer in Rayleighs |
| PHOTOMETER427_VARIANCE |  | [S,22] | Variances of the 427 nm photometer radiances in Rayleighs**2 |
| PHOTOMETER629_VARIANCE |  | [S,22] | Variances of the 629 nm photometer radiances in Rayleighs**2 |