APL SSUSI EDR FILE FORMAT DOCUMENT

Document Version 1.4.5

# Change Log

| Revision | Date | Changes Made |
| --- | --- | --- |
| 1.4.5 | 5/2/2012 | Updated documentation for 3D Ionosphere EDR |
| 1.4.4 | 10/11/11 | Made separate title page, table of contents page and change log pages. |
| 1.4.3 | 10/10/11 | Added Day Auroral GAIM data to edr-gaim disk files |
| 1.4.2 | 9/29/11 | Put back Bubble EDR for now until it is phased out next year. |
| 1.4.1 | 9/14/11 | Added new dayside topside limb variables to EDR-DAY-LIMB product |
| 1.4.0 | 8/8/11 | Added section for EDR-GAIM description |
| 1.3.0 | 6/15/11 | Added documentation for the 3D Ionosphere EDR, and removed documentation for the Ionosphere Bubble EDR |
| 1.2.3 | 7/6/10 | Fixed definitions of nadir and disk quality bits for night disk EDR |
| 1.2.2 | 3/5/10 | Added new fields to Ionosphere Bubble EDR for bubble characterization. These include NMF2, HMF2, MEAN_BUBBLE_ERROR, STD_BUBBLE_ERR, MEAN_BUBBLE, and STD_BUBBLE. |
| 1.2.1 | 9/23/09 | Reorganized Data quality flag bit order to make it more uniform. Dayside limb now has a standard data_quality flag. Separate integer flags for HMF2, etc, will eventually be phased out. |
| 1.2.0 | 9/22/09 | Added documentation for the Ionosphere Bubble EDR. |
| 1.1.3 | 6/15/09 | Added PROTON_FLAG_NORTH/SOUTH and HEMISPHERE_POWER_NORTH/SOUTH to the auroral product. |
| 1.1.2 | 6/10/09 | Added MeV contamination flag to the data quality index |
| 1.1.1 | 10/3/08 | Added SAA data quality bits description for EDR night disk. Upped data product version numbers for full orbit |
| 1.1.0 | 9/24/08 | Added CALIBRATION_PERIOD_VERSION, and SOFTWARE_VERSION_NUMBER. Changed file naming to reflect new AFWA standard naming scheme. Fixed minor typos. |
| 1.0.10 | 8/1/08 | Changed filenaming conventions and realigned document to data products |
| 1.0.9 | 7/2/08 | Added new fields and flags to Nightside Limb product, now 0103 |
| 1.0.8 | 6/30/08 | Added new fields and flags to Nightside Disk Product, now 0103 |
| 1.0.7 | 11/16/07 | Filled in missing HmE, NmE fields in Auroral EDR description |
| 1.0.6 | 10/05/07 | Added Dayside limb description: added CalVal corrected NmF2 to nightside limb |
| 1.0.5 | 9/20/07 | Added CalVal Correction factor and values – fixed some typos |
| 1.0.4 | 8/23/07 | Modified Dayside Disk EDR description |
| 1.0.3 | 7/06/07 | Added Dayside Disk EDR table |
| 1.0.2 | 2/16/07 | Added magnetic local time to northern and southern auroral boundary. Added geographic lat/lon to northern and southern auroral boundary. |
| 1.0.1 | 1/29/07 | Changed names of geophysical parameters. Removed dimensions from global attribute sources. Updates to EDR Limb data product |
| 1.0.0 | 1/5/2007 | Initial Version of document. |

Table of Contents

Change Log	2

Introduction	5

EDR File Naming Conventions	6

EDR Global File Attributes	8

EDR Nightside Limb Data File	11

EDR Nightside Disk Data File	16

EDR Dayside Disk Data File	22

EDR Dayside Limb Data File	27

EDR Aurora Data File	33

EDR Ionosphere Bubble Data File	42

EDR Ionosphere Data File	45

EDR GAIM-LIMB Data File	49

EDR GAIM-DISK Data File	53

# Introduction

The following tables describe the contents of the SSUSI EDR (Environmental Data Record) files produced by IDL code supplied by APL.  The information content of the current IDL produced EDR file is very similar to that created by the older ADA code, with the exception of the newer the Ionosphere Bubble EDR.  This document describes the format of the data only.  For more information about how the quantities are derived, see the SSUSI Data Products Algorithms document.

There are four types of EDR files: limb (nightside and dayside), disk (nightside and disk), auroral, and ionospheric bubble.  All are ultimately derived from rebinned L1B radiance data.  To distinguish between day, night and auroral grids on the disk the array sizes, the indices for across track ‘M’ and along tack, ‘N’ will be annotated with letters to indicate whether they are for aurora (with an ‘a’) or for disk (‘d’) or limb (‘l’) and for night (‘n’) or day (‘d’). These are outlined in the table below.  At present there is no grid in the ionosphere EDR.

|  | Across Track Index | Along Track Index |
| --- | --- | --- |
| Nightside Disk | Mnd | Nnd |
| Nightside Limb | Mnl | Nnl |
| Dayside Disk | Mdd | Ndd |

| Dayside Disk Aurora | Mdda | Ndda |
| --- | --- | --- |

| Dayside Limb | Mdl | Ndl |
| --- | --- | --- |
|  | Geomagnetic Latitude | Geomagnetic Longitude |
| Aurora | Ma | Na |

There are a set of global attributes common to all EDR files; these are described in the next section.  This is followed by detailed descriptions of the remaining customized content of the EDR files: (Nightside Limb, Nightside Disk, Auroral, and Ionosphere).

The processing algorithms use the SSUSI SDR and L1B files previously generated to create separate limb and disk full orbit files for a given SSUSI orbit. The disk EDR files are re-binned onto a lower resolution grid, based on bin sizes proscribed by APL.

Those parameters not generated by the current build of the level EDR software marked by a ‘*’. They may be included in a later version of the file. NetCDF attributes associated with variables are listed together with the variable and are indented.

# EDR File Naming Conventions

| SSUSI EDR Data Product | SSUSI EDR Data Product Name |
| --- | --- |
| SSUSI EDR Nightside Disk | PS.AFWA_SC.U_DI.A_GP.FXX-SSUSI_PA.APL-EDR-NIGHT-DISK_DD.YYYYMMDD_SN.oooo-rr_DF.NC FXX denotes bird number, e.g., F16 DD.YYYYMDD: YYYY= 4 digit year MM – 2 digit Month DD - 2 digit day of month SN.ooooo-rr: ooooo – 5 digit orbit number rr – 2 digit data occurrence number. |
| SSUSI EDR Nightside Limb | PS.AFWA_SC.U_DI.A_GP.FXX-SSUSI_PA.APL-EDR-NIGHT-LIMB_DD.YYYYMMDD_SN.oooo-rr_DF.NC FXX denotes bird number, e.g., F16 DD.YYYYMDD: YYYY= 4 digit year MM – 2 digit Month DD - 2 digit day of month SN.ooooo-rr: ooooo – 5 digit orbit number rr – 2 digit data occurrence number. |
| SSUSI EDR Dayside Disk | PS.AFWA_SC.U_DI.A_GP.FXX-SSUSI_PA.APL-EDR-DAY-DISK_DD.YYYYMMDD_SN.oooo-rr_DF.NC FXX denotes bird number, e.g., F16 DD.YYYYMDD: YYYY= 4 digit year MM – 2 digit Month DD - 2 digit day of month SN.ooooo-rr: ooooo – 5 digit orbit number rr – 2 digit data occurrence number. |
| SSUSI EDR Dayside Limb | PS.AFWA_SC.U_DI.A_GP.FXX-SSUSI_PA.APL-EDR-DAY-LIMB_DD.YYYYMMDD_SN.oooo-rr_DF.NC FXX denotes bird number, e.g., F16 DD.YYYYMDD: YYYY= 4 digit year MM – 2 digit Month DD - 2 digit day of month SN.ooooo-rr: ooooo – 5 digit orbit number rr – 2 digit data occurrence number. |
| SSUSI EDR Aurora | PS.AFWA_SC.U_DI.A_GP.FXX-SSUSI_PA.APL-EDR-AURORA_DD.YYYYMMDD_SN.oooo-rr_DF.NC FXX denotes bird number, e.g., F16 DD.YYYYMDD: YYYY= 4 digit year MM – 2 digit Month DD - 2 digit day of month SN.ooooo-rr: ooooo – 5 digit orbit number rr – 2 digit data occurrence number. |
| SSUSI EDR Bubble | PS.AFWA_SC.U_DI.A_GP.FXX-SSUSI_PA.APL-EDR-IONO-BUBBLE_DD.YYYYMMDD_SN.oooo-rr_DF.NC FXX denotes bird number, e.g., F16 DD>YYYYMDD: YYYY= 4 digit year MM – 2 digit Month DD - 2 digit day of month SN.ooooo-rr: ooooo – 5 digit orbit number rr – 2 digit data occurrence number. |
| SSUSI EDR Ionosphere | PS.AFWA_SC.U_DI.A_GP.FXX-SSUSI_PA.APL-EDR-IONO _DD.YYYYMMDD_SN.oooo-rr_DF.NC FXX denotes bird number, e.g., F16 DD.YYYYMDD: YYYY= 4 digit year MM – 2 digit Month DD - 2 digit day of month SN.ooooo-rr: ooooo – 5 digit orbit number rr – 2 digit data occurrence number. |
| SSUSI EDR GAIM-LIMB | PS. AFWA_SC.U_DI.A_GP.FXX-SSUSI_PA.APL-EDR-GAIM-LIMB _DD.YYYYMMDD_SN.oooo-rr_DF.NC FXX denotes bird number, e.g., F16 DD.YYYYMDD: YYYY= 4 digit year MM – 2 digit Month DD - 2 digit day of month SN.ooooo-rr: ooooo – 5 digit orbit number rr – 2 digit data occurrence number. |
| SSUSI EDR GAIM-DISK | PS. AFWA_SC.U_DI.A_GP.FXX-SSUSI_PA.APL-EDR-GAIM-DISK _DD.YYYYMMDD_SN.oooo-rr_DF.NC FXX denotes bird number, e.g., F16 DD.YYYYMDD: YYYY= 4 digit year MM – 2 digit Month DD - 2 digit day of month SN.ooooo-rr: ooooo – 5 digit orbit number rr – 2 digit data occurrence number. |

# EDR Global File Attributes

These attributes are common to EDR disk, limb, aurora, GAIM, and ionosphere files.

| Variable Name | Type | Dimensions | Value/Comment |
| --- | --- | --- | --- |
| Global file attributes |  |  |  |
| FILENAME | STRING | - | e.g., “PS.APL_V0102S010CD031_SC.U_DI.A_GP.F16-SSUSI_PA.APL-EDR-DAY-DISK_DD.20050919_SN.09913-01_DF.NC” |
| MISSION | STRING | - | e.g., “F16” |
| DATA_PRODUCT_TYPE | STRING | - | e.g., “EDR Aurora” |
| SOURCE | STRING | - | Names of the SDR and/or L1B File(s) used to generate this file. |
| SCAN_TYPE | STRING | - | “DISK” or “LIMB” |
| REGION_TYPE | STRING | - | “NIGHT” or “AURORA” or “DAY” or “TWILIGHT” |
| DATA_PRODUCT_VERSION | STRING | - | e.g., “0106” |
| DATA_PRODUCT_REVISION | STRING | - | e.g., “001” |
| DATA_DOCUMENTATION_VERSION | STRING | - | Version number of the SSUSI File Definition Data Document utilized, e.g., “0106” |
| SOFTWARE_VERSION_NUMBER | STRING | - | CVS package tag of SDR software e.g., “sdr-1-1-1” |
| SOFTWARE_VERSION | STRING | - | e.g., “devel” |
| SOFTWARE_NAME | STRING | - | “APL SDR to EDR algorithm X” |
| CALIBRATION_TABLES_NAMES | STRING | - | ‘;’ separated list of calibration files that were used to generate this EDR file. |
| CALIBRATION_TABLES_CREATED | STRING | - | Date when calibration table was created, e.g., “Thu Oct 13 16:15:05 2005” If more than one file is used, a semi-colon separated list is used. |
| CALIBRATION_PERIOD_VERSION | STRING | - | Letter indicating calibration period along with 3 digit number of the calibration version in relation to all possible calibration version files, e.g. D031 implies calibration period D and the calibration table is the 31st version of the calibration file. |
| DESCRIPTION | STRING | - | e.g., “EDR Imaging Mode Data – Nightside Disk” |
| COMMENT | STRING | - | e.g., “No comment” |
| HISTORY | STRING | - | e.g., “No revisions” |
| DATE_GENERATED | STRING | - | e.g., “2005094213027” |
| STARTING_TIME | STRING | - | e.g., “2004150100727” |
| STOPPING_TIME | STRING | - | e.g., “2004150114908” |
| STARTING_ORBIT_NUMBER | FLOAT | - | Orbit number of the starting grid cell e.g., “3159.0” |
| STOPPING_ORBIT_NUMBER | FLOAT | - | Orbit number of the ending grid cell e.g., “3160.0” |
| NODAL_CROSSING_EPOCH | DOUBLE | - | The CDF EPOCH of the Nodal crossing data in the GWC ephemeris from the Prep file. e.g., “63273228090000.” |
| NODAL_DAY | INTEGER | - | The day of month of the Nodal Crossing |
| NODAL_MONTH | INTEGER | - | The month of the Nodal Crossing |
| NODAL_YEAR | INTEGER | - | The year of the Nodal Crossing |
| NO_DATA_IN_BIN_VALUE | FLOAT | - | The value placed in EDR grid cells with no L1B/SDR pixels, e.g., “fNaN” |
| Geophysical information (if available) |  |  |  |
| GEOPHYSICAL_INFO_UPDATE | STRING | - | Last update time, e.g., “20010418:0000” |
| DAY_81_F107 | STRING | - | 81 day average of F10.7 with current day as day 41,e.g., “ 0.000000” |
| DAILY_F107 | STRING | - | Daily average of F10.7 e.g., “ 0.000000” |
| F10_7_SOURCE | STRING | - | Where the F10.7 parameter originated, e.g., “DYN_PARAMS_01108.TXT” |
| HOUR_3_KP | STRING | - | 3 hour average of Kp, e.g., “ 2.00000” |
| DAILY_KP | STRING | - | Daily average of Kp, e.g., “ 2.00000” |
| KP_AP_SOURCE | STRING | - | Source of Kp, Ap indices, e.g., “DYN_PARAMS_01108.TXT” |
| DAILY_AP | STRING | - | Daily average of Ap, e.g., “ 2.00000” |

# EDR Nightside Limb Data File

PRODUCT VERSION 0104

| Revision | Date | Changes Made |
| --- | --- | --- |
| 0104 | 08/01/08 | Realigned document to data products |
| 0103 | 7/1/08 | Added new fields for max radiance and altitude of max, and new DQI |
| 0102 | 10/5/07 | Added CalVal corrected NmF2 to file |
| 0101 | 1/29/2007 | Removed acrosspixelsize, alongpixelsize, effectivelookangle, acrosstrackangle, alongtrackangle. Changed dimensions on tangentpoint_re and tangentpoint_altitude. |
| 0100 | 1/5/2007 | Initial Version of nightside limb definition table |

The table below details the EDR Nightside Limb data files.

| Variable Name | Type | Dimensions | Value/Comment |
| --- | --- | --- | --- |
| Global Nightside Limb data file attributes |  |  |  |
| Time and Position Information |  |  |  |
| TIME TIME_TITLE TIME_UNITS | DOUBLE STRING STRING | [Nnl] - - | The effective time of each scan. ”Time of each scan” ”Seconds since the start of the day” |
| TIME_EPOCH TIME_EPOCH_TITLE TIME_EPOCH_UNITS | DOUBLE STRING STRING | [Nnl] - - | The effective time of each scan. “Time of each scan” “Milli-seconds since 01-Jan-0000 00:00:00.000 (CDF epoch value)” |
| YEAR YEAR_TITLE | INT STRING | [Nnl] - | The year of each rebinned scan “Year of each scan” |
| DOY DOY_TITLE | INT STRING | [Nnl] - | The day of year of each rebinned scan. “Day of year of each scan” |
| LATITUDE LATITUDE_TITLE LATITUDE_UNITS | FLOAT STRING STRING | [Nnl] - - | DMSP spacecraft latitude “S/C geographic latitude at effective times” “Degrees” |
| LONGITUDE LONGITUDE_TITLE LONGITUDE_UNITS | FLOAT STRING STRING | [Nnl] - - | DMSP spacecraft longitude “S/C geographic longitude at effective times” “Degrees” |
| ALTITUDE ALTITUDE_TITLE ALTITUDE_UNITS | FLOAT STRING STRING | [Nnl] - - | DMSP spacecraft altitude “S/C altitude at effective times” “Kilometers” |
| TANGENTPOINT_LATITUDE_GEOGRAPHIC TANGENTPOINT_LATITUDE_GEOGRAPHIC _TITLE TANGENTPOINT_LATITUDE_GEOGRAPHIC _UNITS | FLOAT STRING STRING | [Nnl] - - | Pixel tangent point geographic latitude at approximately 300km “Tangent point geographic latitude” “Degrees” |
| TANGENTPOINT_LONGITUDE_GEOGRAPHIC TANGENTPOINT_LONGITUDE _GEOGRAPHIC _TITLE TANGENTPOINT_LONGITUDE_GEOGRAPHIC _UNITS | FLOAT STRING STRING | [Nnl] - - | Pixel tangent point geographic longitude “Tangent point geographic longitude” “Degrees” |
| TANGENTPOINT_RE TANGENTPOINT_RE_TITLE TANGENTPOINT_RE_UNITS | DOUBLE STRING STRING | [Nnl] - - | Radius of earth at tangent point latitude “Earth radius at tangent point latitude” “km” |
| ALTITUDE_MODEL_GRID ALTITUDE_ MODEL_GRID _TITLE ALTITUDE_ MODEL_GRID _UNITS | FLOAT STRING STRING | [Mnl] - - | Altitude at which MSIS is run “Model grid altitude” “km” |
| TANGENTPOINT_ALTITUDE TANGENTPOINT_ALTITUDE _TITLE TANGENTPOINT_ALTITUDE_UNITS | FLOAT STRING STRING | [Nnl] - - | Pixel tangent point altitude “Tangent point altitude” “Kilometers” |
| Calibration parameters |  |  |  |
| DARK_COUNT_CORRECTION DARK_COUNT_CORRECTION_TITLE | INT STRING | - - | Correct counts for dark counts. “Corrected for Dark counts (0/1 – No/Yes).” |
| SCATTER_LIGHT_1216_CORRECTION SCATTER_LIGHT_1216_CORRECTION_TITLE | INT STRING | - - | Correct counts for 1216 scattered lights. “Corrected for 1216 scattered light (0/1 – No/Yes).” |
| SCATTER_LIGHT_1304_CORRECTION SCATTER_LIGHT_1304_CORRECTION_TITLE | INT STRING | - - | Correct counts for 1304 scattered lights. “Corrected for 1304 scattered light (0/1 – No/Yes).” |
| OVERLAP_1304_1356_CORRECTION OVERLAP_1304_1356_CORRECTION_TITLE | INT STRING | - - | Correct counts for 1304/1356 overlap. “Corrected for 1304/1356 overlap (0/1 – No/Yes).” |
| LONGWAVE_SCATTER_CORRECTION LONGWAVE_SCATTER_CORRECTION_TITLE | INT STRING | - - | Correct counts for long-wave scattered light. “Corrected for long-wave scattered light (0/1 – No/Yes).” |
| Electron density profile data |  |  |  |
| NUM_VER_NONZERO_POINTS | INT | [Nnl] | Number of non-zero points in VER profile “Number of VER non-zero points” |
| VOLUME_EMISSION_RATE VOLUME_EMISSION_RATE_TITLE VOLUME_EMISSION_RATE_UNITS | FLOAT STRING STRING | [Nnl, Mnl] - - | Volume Emission Rate “Volume emission rate of monatomic oxygen”. “/sec/cc”. |
| VOLUME_EMISSION_RATE_UNCERTAINTY VOLUME_EMISSION_RATE_UNCERTAINTY_TITLE VOLUME_EMISSION_RATE_UNITS | FLOAT STRING STRING | [Nnl, Mnl] - - | Volume emission rate uncertainty. “Volume emission rate uncertainty”. “percent” |
| VOLUME_EMISSION_RATE_CHISQUARED VOLUME_EMISSION_RATE_CHISQUARED_TITLE VOLUME_EMISSION_RATE_CHISQUARED_UNITS | FLOAT STRING STRING | [Nnl] - - | Volume emission rate fit chi-squared “Volume emission rate chi-squared”. “” |
| CHAPMAN_CHISQUARED CHAPMAN _CHISQUARED_TITLE CHAPMAN _CHISQUARED_UNITS | FLOAT STRING STRING | [Nnl] - - | Chapman fit chi-squared “Chapman fit chi-squared”. “” |
| EDP EDP _TITLE EDP _UNITS | FLOAT STRING STRING | [Nnl, Mnl] - - | Electron density profile. “Electron density profile”. “Cm**-3” |
| EDP_UNCERTAINTY EDP_UNCERTAINTY_TITLE EDP_UNCERTAINTY_UNITS | FLOAT STRING STRING | [Nnl, Mnl] - | EDP uncertainty “Electron density profile uncertainty”. “percent” |
| NMF2 NMF2_TITLE NMF2_UNITS | FLOAT STRING STRING | [Nnl] - - | NMF2. “Maximum number density of electrons”. “cm**3” |
| NMF2_UNCERTAINTY NMF2_ UNCERTAINTY _TITLE NMF2_ UNCERTAINTY _UNITS | FLOAT STRING STRING | [Nnl] - - | NMF2 uncertainty “Uncertainty in the maximum number density of electrons”. “percent” |
| CALVAL_NMF2_CORRECTION_FACTOR | FLOAT | - | Factor to multiply NmF2 to match Ionosonde data in CalVal study |
| CALVAL_CORRECTED_NMF2 CALVAL_CORRECTED_NMF2_TITLE CALVAL_CORRECTED_NMF2_UNITS | FLOAT STRING STRING | [Nnl] - - | NMF2. “Maximum number density of electrons multiplied by the CALVAL correction factor”. “cm**3” |
| CALVAL_CORRECTED_NMF2_UNCERTAINTY CALVAL_CORRECTED_NMF2_ UNCERTAINTY _TITLE CALVAL_CORRECTED_NMF2_ UNCERTAINTY _UNITS | FLOAT STRING STRING | [Nnl] - - | NMF2 uncertainty “Uncertainty in the maximum number density of electrons multiplied by the CALVAL correction factor”. “percent” |
| HMF2 HMF2 _TITLE HMF2_UNITS | FLOAT STRING STRING | [Nnl] - - | HMF2. “Height of the peak F-region electron density”. “Km” |
| HMF2_UNCERTAINTY HMF2_ UNCERTAINTY _TITLE HMF2_ UNCERTAINTY _UNITS | FLOAT STRING STRING | [Nnl] - - | HMF2 uncertainty “Uncertainty in the height of the F-region electron density”. “percent” |
| TOPH TOPH_TITLE TOPH_UNITS | FLOAT STRING STRING | [Nnl] - - | Top-side scale height. “Top side scale height”. “Km” |
| TOPH _ UNCERTAINTY TOPH _ UNCERTAINTY _TITLE TOPH _ UNCERTAINTY _UNITS | FLOAT STRING STRING | [Nnl] - - | TOPH uncertainty “Uncertainty in the top side scale height”. “percent” |
| MAXRAD_1304 MAXRAD_1304.TITLE MAXRAD_1304.UNITS | FLOAT STRING STRING | [Nnl] - - | Maximum Limb 1304 Radiance "Maximum 1304 radiance (from SDR)" "Rayleigh" |
| MAXRAD_1356 MAXRAD_1356.TITLE MAXRAD_1356.UNITS | FLOAT STRING STRING | [Nnl] - - | Maximum Limb 1356 Radiance "Maximum 1356 radiance (from SDR)" "Rayleigh" |
| MAXRAD_LBHS MAXRAD_LBHS.TITLE MAXRAD_LBHS.UNITS | FLOAT STRING STRING | [Nnl] - - | Maximum Limb LBHS Radiance "Maximum LBHS radiance (from SDR)" "Rayleigh" |
| ZMAX_1304 ZMAXRAD_1304.TITLE ZMAXRAD_1304.UNITS | FLOAT STRING STRING | [Nnl] - - | Altitude of Maximum Limb 1304 Radiance "Altitude of maximum 1304 radiance (from SDR)" "Km" |
| ZMAX_1356.TITLE ZMAXRAD_1356.TITLE ZMAXRAD_1356.UNITS | FLOAT STRING STRING | [Nnl] - - | Altitude of Maximum Limb 1356 Radiance "Altitude of maximum 1356 radiance (from SDR)" "Km" |
| ZMAX_LBHS ZMAXRAD_LBHS.TITLE ZMAXRAD_LBHSUNITS | FLOAT STRING STRING | [Nnl] - - | Altitude of Maximum Limb LBHS Radiance "Altitude of maximum LBHS radiance (from SDR)" "Km" |
| TOP_LBHS TOP_LBHS.TITLE TOP_LBHS.UNITS | FLOAT STRING STRING | [Nnl] - - | Top of the limb LBHS radiance "LBHS radiance at top of limb (from SDR)" "Rayleigh" |
| DATA_QUALITY | UNSIGNED SHORT | [Nnl] | Bit # Meaning if set to true - VER fit unsuccessful (chi squared test: rchi2v>10) - Chapman fit unsuccessful (chi squared test: rchi2c>10) - NmF2 uncertainty > 50% - HmF2 < 0 km - HmF2 > 500 km - HmF2 uncertainty > 50% - NRP < 7 - SZA < 105 degrees - MeV contamination 9-15 Spare |

# EDR Nightside Disk Data File

PRODUCT VERSION 0104

| Revision | Date | Changes Made |
| --- | --- | --- |
| 0104 | 8/1/08 | Realigned document to data products; DQI arrays are unsigned shorts, not BYTEs. |
| 0103 | 6/30/08 | Added SZA and LBHS; added sza and LBHS flags |
| 0102 | 9/20/07 | Apparently went to 0102 with previous |
| 0101 | 9/20/07 | CalVal corrected Densities added to product. |
| 0100 | 1/5/2007 | Initial Version of nightside disk definition table |

The table below details the EDR Nightside Disk file.

| Variable Name | Type | Dimensions | Value/Comment |
| --- | --- | --- | --- |
| Global Nightside Disk data file attributes |  |  |  |
| ACROSSPIXELSIZE ACROSSPIXELSIZE_TITLE ACROSSPIXELSIZE_UNITS | INT STRING STRING | - - - | Across track pixel size for the grid. ”Across track pixel size” ”km” |
| ALONGPIXELSIZE ALONGPIXELSIZE_TITLE ALONGPIXELSIZE_UNITS | INT STRING STRING | - - - | Along track pixel size for the grid. ”Across track pixel size” ”km” |
| NIGHTSIDEDISK_LOOKUP_TABLE | UNSIGNED SHORT | - | Lookup table used for the nightside disk limb algorithm 0 = JDO; 1 = OO |
| Time and Position Information |  |  |  |
| TIME TIME_TITLE TIME_UNITS | DOUBLE STRING STRING | [Nnd] - - | The effective time of each rebinned scan. “Time of each rebinned scan” ”Seconds since the start of the day” |
| TIME_EPOCH TIME_EPOCH_TITLE TIME_EPOCH_UNITS | DOUBLE STRING STRING | [Nnd] - - | The effective time of each rebinned scan. ”Time of each rebinned scan” ”Milli-seconds since 01-Jan-0000 00:00:00.000 (CDF epoch value)” |
| YEAR YEAR_TITLE | INT STRING | [Nnd] - | The year of each rebinned scan. “Year of each rebinned scan” |
| DOY DOY_TITLE | INT STRING | [Nnd] - | The day of year of each rebinned scan. “Day of year of each rebinned scan” |
| LATITUDE LATITUDE_TITLE LATITUDE_UNITS | FLOAT STRING STRING | [Nnd] - - | DMSP spacecraft Latitude. “S/C geographic latitude at effective times” “Degrees” |
| LONGITUDE LONGITUDE_TITLE LONGITUDE_UNITS | FLOAT STRING STRING | [Nnd] - - | DMSP spacecraft Longitude. “S/C geographic longitude at effective times” “Degrees” |
| ALTITUDE ALTITUDE_TITLE ALTITUDE_UNITS | FLOAT STRING STRING | [Nnd] - - | DMSP spacecraft Altitude. “S/C altitude at effective times” “Kilometers” |
| PIERCEPOINT_NIGHT_LATITUDE PIERCEPOINT_NIGHT_LATITUDE _TITLE PIERCEPOINT_NIGHT_LATITUDE_UNITS | FLOAT STRING STRING | [Nnd, Mnd,] - - | Rebinned latitude of the pierce point, geolocated using the nightside reference altitude. “Nightside geographic latitude of the pierce point” “Degrees” |
| PIERCEPOINT_NIGHT_LONGITUDE PIERCEPOINT_NIGHT_LONGITUDE _TITLE PIERCEPOINT_NIGHT_LONGITUDE_UNITS | FLOAT STRING STRING | [Nnd, Mnd,] - - | Rebinned longitude of the pierce point, geolocated using the nightside reference altitude. “Nightside geographic longitude of the pierce point” “Degrees” |
| PIERCEPOINT_NIGHT_ALTITUDE PIERCEPOINT_NIGHT_ALTITUDE _TITLE PIERCEPOINT_NIGHT_ALTITUDE_UNITS | FLOAT STRING STRING | - - - | Reference altitude for nightside “Nightside reference altitude for geolocation” “Kilometers” |
| ACROSSTRACKANGLE | FLOAT | [Mnd, Nnd] | Across track angle (theta) on the grid. |
| ALONGTRACKANGLE | FLOAT | [Nnd] | Along track angle (alpha) on the grid. |
| Calibration parameters |  |  |  |
| DARK_COUNT_CORRECTION DARK_COUNT_CORRECTION_TITLE | INT STRING | - - | Correct counts for dark counts. “Corrected for Dark counts (0/1 – No/Yes).” |
| SCATTER_LIGHT_1216_CORRECTION SCATTER_LIGHT_1216_CORRECTION_TITLE | INT STRING | - - | Correct counts for 1216 scattered lights. “Corrected for 1216 scattered light (0/1 – No/Yes).” |
| SCATTER_LIGHT_1304_CORRECTION SCATTER_LIGHT_1304_CORRECTION_TITLE | INT STRING | - - | Correct counts for 1304 scattered lights. “Corrected for 1304 scattered light (0/1 – No/Yes).” |
| OVERLAP_1304_1356_CORRECTION OVERLAP_1304_1356_CORRECTION_TITLE | INT STRING | - - | Correct counts for 1304/1356 overlap. “Corrected for 1304/1356 overlap (0/1 – No/Yes).” |
| LONGWAVE_SCATTER_CORRECTION LONGWAVE_SCATTER_CORRECTION_TITLE | INT STRING | - - | Correct counts for long-wave scattered light. “Corrected for long-wave scattered light (0/1 – No/Yes).” |
| Electron density profile data |  |  |  |
| NMF2_DISK NMF2 _DISK_TITLE NMF2_DISK_UNITS | FLOAT STRING STRING | [Mnd, Nnd] - | NMF2 on the disk. “Peak F-region electron density on the disk”. “Cm**-3”. |
| NMF2_DISK_UNCERTAINTY NMF2_DISK_ UNCERTAINTY _TITLE NMF2_DISK_ UNCERTAINTY _UNITS | FLOAT STRING STRING | [Mnd, Nnd] - | Uncertainty of NMF2 on the disk. “Uncertainty in the peak F-region electron density on the disk”. “percent” |
| SZA_DISK SZA_DISK.TITLE SZA_DISK.UNITS | FLOAT STRING STRING | [Mnd, Nnd] - | Piercepoint Solar Zenith Angle on the disk. “Solar Zenith Angle at the night referenced disk piercepoint”. “degrees”. |
| LBHS_DISK LBHS_DISK.TITLE LBHS_DISK.UNITS | FLOAT STRING STRING | [Mnd, Nnd] - | LBH short on the disk. “LBH short radiance geolocated to the nightside referenced disk”. “Rayleighs”. |
| NMF2_NADIR NMF2_NADIR _TITLE NMF2_NADIR _UNITS | FLOAT STRING STRING | [Nnd] - | NMF2 on the nadir. “Peak F-region electron density on the nadir”. “Cm**-3” |
| NMF2_NADIR_UNCERTAINTY NMF2_NADIR_ UNCERTAINTY _TITLE NMF2_NADIR_ UNCERTAINTY _UNITS | FLOAT STRING STRING | [Nnd] - | Uncertainty of NMF2 on the nadir. “Uncertainty in the peak F-region electron density on the nadir”. “percent” |
| CALVAL_NMF2_CORRECTION_FACTOR | FLOAT | - | Correction factor to be applied to the NmF2 electron densities as recommended in the final CalVal report. |
| CALVAL_CORRECTED_DISK_NMF2 CALCAL_CORRECTED_DISK_NMF2.TITLE CALVAL_CORRECED_DISK_NMF2.UNITS | FLOAT STRING STRING | [Mnd,Nnd] | NmF2 with correction factor applied “CalVal corrected peak F-region electron density on the disk” “cm^-3" |
| CALVAL_CORRECTED_VDISK_NmF2 CALVAL_CORRECTED_VDISK_NmF2.TITLE CALVAL_CORRECTED_VDISK_NmF2.UNITS | FLOAT STRING STRING | [Mnd, Nnd] | Correction factor squared and applied to the NmF2 Variance "CalVal corrected variance in the peak F-region electron density on the disk" "cm^-6" |
| CALVAL_CORRECTED_NADIR_NmF2 CALVAL_CORRECTED_NADIR_NmF2.TITLE CALVAL_CORRECTED_NADIR_NmF2.UNITS | FLOAT STRING STRING | [Nnd] - - | Correction factor applied to nadir NmF2 densities "CalVal corrected Peak F-region electron density at nadir" "cm^-3" |
| CALVAL_CORRECTED_VNADIR_NmF2 CALVAL_CORRECTED_VNADIR_NmF2.TITLE CALVAL_CORRECTED_VNADIR_NmF2.UNITS | FLOAT STRING STRING | [Nnd] - - | Correction factor squared and applied to nadir NmF2 variance "CalVal corrected Variance in the peak F-region electron density at nadir" "cm^-6" |
| HMF2_NADIR HMF2_NADIR _TITLE HMF2_NADIR _UNITS | FLOAT STRING STRING | [Nnd - - | HMF2 on the nadir. “Height of the peak F-region electron density at nadir”. “Km” |
| HMF2_NADIR _ UNCERTAINTY HMF2_NADIR _ UNCERTAINTY _TITLE HMF2_NADIR _ UNCERTAINTY _UNITS | FLOAT STRING STRING | [Nnd] - - | Uncertainty of HMF2 on the nadir “Uncertainty in the height of the F-region electron density at nadir”. “percent” |
| SZA_NADIR SZA_NADIRTITLE SZA_NADIR.UNITS | FLOAT STRING STRING | [Nnd] - - | Nadir Piercepoint Solar Zenith Angle. “Solar Zenith Angle at the night referenced nadir piercepoint”. “degrees”. |
| LBHS_NADIR LBHS_NADIR.TITLE LBHS_NADIR.UNITS | FLOAT STRING STRING | [Nnd] - - | LBH short at Nadir. “LBH short radiance geolocated to the nightside referenced nadir”. “Rayleighs”. |
| FoF2_DISK FoF2_DISK_TITLE FoF2_DISK_UNITS | FLOAT STRING STRING | [Mnd, Nnd] - | FoF2 on the disk. “Plasma frequency on the disk”. “Hz” |
| FoF2_DISK_ UNCERTAINTY FoF2_DISK_ UNCERTAINTY _TITLE FoF2_DISK_ UNCERTAINTY _UNITS | FLOAT STRING STRING | [Mnd, Nnd] - | Uncertainty of FoF2 on the disk “Uncertainty in the plasma frequency on the disk”. “Hz**2” |
| FoF2_NADIR FoF2_ NADIR _TITLE FoF2_ NADIR _UNITS | FLOAT STRING STRING | [Nnd] - | FoF2 at nadir. “Plasma frequency at nadir”. “Hz” |
| FoF2_ NADIR _ UNCERTAINTY FoF2_ NADIR _ UNCERTAINTY _TITLE FoF2_ NADIR _ UNCERTAINTY _UNITS | FLOAT STRING STRING | [Nnd] - | Uncertainty of FoF2 at nadir “Uncertainty in the plasma frequency at nadir”. “percent” |
| DATA_QUALITY_GLOBAL | UNSIGNED SHORT | [1] | 1 byte global data quality “Global data quality Flags – see EDR Format document for details” Bit # Meaning if set to true - Northern Anomaly Crest Not Found - Southern Anomaly Crest Not Found - spare - spare - spare - spare - spare - spare |
| DATA_QUALITY_NADIR | UNSIGNED SHORT | [Nnd] | Bit # Meaning if set to true - Nadir 1356 A Radiance undefined (NaN) - Solar Zenith Angle too small - LBH short exceeds limit - SAA contaminated radiances - Nadir NmF2 uncertainty > 50% - Nadir HmF2 uncertainty > 50% - Nadir 6300 A Radiance undefined (NaN) - Spare - MeV Noise contamination - Mirror position unknown |
| DATA_QUALITY_DISK | UNSIGNED SHORT | [Mnd, Nnd] | Bit # Meaning if set to true - Nadir 1356 A Radiance undefined (NaN) - Solar Zenith Angle too small - LBH short exceeds limit - SAA Contaminated disk radiances - Disk NmF2 uncertainty > 50% - Spare - Spare - Spare - MeV Noise contamination - Mirror position unknown |

# EDR Dayside Disk Data File

PRODUCT VERSION  0103

| Revision | Date | Changes Made |
| --- | --- | --- |
| 0103 | 9/22/09 | Reordered data quality bits |
| 0102 | 08/1/2008 | Realigned document to data products |
| 0101 | 08/23/2007 | Updated to reflect file |
| 0100 | 1/5/2007 | Initial Version of nightside disk definition table |

The table below details the EDR Dayside Disk file.

| Variable Name | Type | Dimensions | Value/Comment |
| --- | --- | --- | --- |
| Global Nightside Disk data file attributes |  |  |  |
| ACROSSPIXELSIZE ACROSSPIXELSIZE_TITLE ACROSSPIXELSIZE_UNITS | INT STRING STRING | - - - | Across track pixel size for the grid. ”Across track pixel size” ”km” |
| ALONGPIXELSIZE ALONGPIXELSIZE_TITLE ALONGPIXELSIZE_UNITS | INT STRING STRING | - - - | Along track pixel size for the grid. ”Across track pixel size” ”km” |
| ON2_TABLE | STRING | - | e.g. “DTON2R_TABLE_V0100.sav” |
| QEUV_TABLE | STRING | - | e.g. “DTQEUV_TABLE_V0100.sav” |
| EDPP_TABLE | STRING | - | e.g. “EDPP_TABLE_V0100.sav” |
| Time and Position Information |  |  |  |
| TIME TIME_TITLE TIME_UNITS | DOUBLE STRING STRING | [Ndd] - - | Time of each newly rebinned along track day altitude pixel “Time of each newly rebinned along track day altitude pixel” ”Seconds since the start of the day” |
| TIME_EPOCH TIME_EPOCH_TITLE TIME_EPOCH_UNITS | DOUBLE STRING STRING | [Ndd] - - | Time of each newly rebinned along track day altitude pixel ” Time of each newly rebinned along track day altitude pixel” ”Milli-seconds since 01-Jan-0000 00:00:00.000 (CDF epoch value)” |
| YEAR YEAR_TITLE | INT STRING | [Ndd] - | Year of each newly rebinned along track day altitude pixel “Year of each newly rebinned along track day altitude pixel” |
| DOY DOY_TITLE | INT STRING | [Ndd] - | The day of year of each newly rebinned scan dayside along track pixel. “Day of year of each newly rebinned along track day altitude pixel” |
| LATITUDE LATITUDE_TITLE LATITUDE_UNITS | FLOAT STRING STRING | [Ndd] - - | DMSP spacecraft Latitude. “S/C geographic latitude, rebinned to the day altitude grid” “Degrees” |
| LONGITUDE LONGITUDE_TITLE LONGITUDE_UNITS | FLOAT STRING STRING | [Ndd] - - | DMSP spacecraft Longitude. “S/C geographic longitude, rebinned to the day altitude grid” “Degrees” |
| ALTITUDE ALTITUDE_TITLE ALTITUDE_UNITS | FLOAT STRING STRING | [Ndd] - - | DMSP spacecraft Altitude. “S/C altitude, rebinned to the day altitude grid” “Kilometers” |
| PIERCEPOINT_DAY_LATITUDE PIERCEPOINT_DAY_LATITUDE _TITLE PIERCEPOINT_DAY_LATITUDE_UNITS | FLOAT STRING STRING | [Ndd, Mdd] - - | Rebinned latitude of the pierce point, geolocated using the dayside reference altitude. “Geographic latitude of the pierce point; rebinned to the day altitude grid” “Degrees” |
| PIERCEPOINT_DAY_LONGITUDE PIERCEPOINT_DAY_LONGITUDE _TITLE PIERCEPOINT_DAY_LONGITUDE_UNITS | FLOAT STRING STRING | [Ndd, Mdd] - - | Rebinned longitude of the pierce point, geolocated using the dayside reference altitude. “Geographic longitude of the pierce point; rebinned to the day altitude grid” “Degrees” |
| PIERCEPOINT_DAY_ALTITUDE PIERCEPOINT_DAY_ALTITUDE _TITLE PIERCEPOINT_DAY_ALTITUDE_UNITS | FLOAT STRING STRING | - - - | Reference altitude for dayside “Dayside reference altitudes for pierce point location calculations” “Kilometers” |
| PIERCEPOINT_DAY_SZA PIERCEPOINT_DAY_ SZA _TITLE PIERCEPOINT_DAY_ SZA _UNITS | FLOAT STRING STRING | [Ndd, Mdd] - - | Rebinned solar zenith angle of the pierce point, geolocated using the dayside reference altitude. “Dayside solar zenith angle for center of the new limb grid cell” “Degrees” |
| EFFECTIVE_LOOK_ANGLE EFFECTIVE_LOOK_ANGLE_TITLE EFFECTIVE_LOOK_ANGLE_UNITS | FLOAT STRING STRING | [Ndd, Mdd] | Effective look angle. “Effective cross track look angle to day grid centers” “Degrees” |
| Calibration parameters |  |  |  |
| DARK_COUNT_CORRECTION DARK_COUNT_CORRECTION_TITLE | INT STRING | - - | Correct counts for dark counts. “Corrected for Dark counts (1-Yes, 0-No).” |
| SCATTER_LIGHT_1216_CORRECTION SCATTER_LIGHT_1216_CORRECTION_TITLE | INT STRING | - - | Correct counts for 1216 scattered lights. “Corrected for 1216 scattered light (1-Yes, 0-No).” |
| SCATTER_LIGHT_1304_CORRECTION SCATTER_LIGHT_1304_CORRECTION_TITLE | INT STRING | - - | Correct counts for 1304 scattered lights. “Corrected for 1304 scattered light (1-Yes, 0-No).” |
| OVERLAP_1304_1356_CORRECTION OVERLAP_1304_1356_CORRECTION_TITLE | INT STRING | - - | Correct counts for 1304/1356 overlap. “Corrected for 1304/1356 overlap (1-Yes, 0-No).” |
| LONGWAVE_SCATTER_CORRECTION LONGWAVE_SCATTER_CORRECTION_TITLE | INT STRING | - - | Correct counts for long-wave scattered light. “Corrected for long-wave scattered light (1-Yes, 0-No).” |
| Electron density profile data |  |  |  |
| NMF2 NMF2 _ TITLE NMF2_ UNITS | FLOAT STRING STRING | [Ndd, Mdd] - | NMF2 on the disk. “Peak F-region electron density on the disk”. “Cm**-3”. |
| NMF2_ UNCERTAINTY NMF2_ UNCERTAINTY _TITLE NMF2_ UNCERTAINTY _UNITS | FLOAT STRING STRING | [Ndd, Mdd] - | Error of NMF2 on the disk. “Error in the peak F-region electron density on the disk”. “Cm**-3” |
| HMF2 HMF2_TITLE HMF2_UNITS | FLOAT STRING STRING | [Ndd, Mdd] - | HMF2 on the disk. “Height of the peak F-region electron density on the disk”. “Km” |
| HMF2_UNCERTAINTY HMF2_UNCERTAINTY _TITLE HMF2_UNCERTAINTY _UNITS | FLOAT STRING STRING | [Ndd, Mdd] - | Error of HMF2 on the disk “Error in the height of the F-region electron density on the disk”. “Km” |
| QEUV_NADIR QEUV _ NADIR _TITLE QEUV _ NADIR _UNITS | FLOAT STRING STRING | [Ndd] - | QEUV at nadir. “Integrated solar irradiance from 5 to 45 nm at nadir”. “ergs^2 cm^-4 s^-2” |
| QEUV _ NADIR _ UNCERTAINTY QEUV _ NADIR _ UNCERTAINTY _TITLE QEUV _ NADIR _ UNCERTAINTY _UNITS | FLOAT STRING STRING | [Ndd] - | Error of QEUV at nadir “Error in integrated solar irradiance from 5 to 45 nm at nadir”. “ergs^2 cm^-4 s^-2” |
| ON2_NADIR ON2_ NADIR _TITLE ON2_ NADIR _UNITS | FLOAT STRING STRING | [Ndd] - | ON2 at nadir. “Ratio of the O to N2 vertical column densities at nadir”. “none” |
| ON2_ NADIR _ UNCERTAINTY ON2_ NADIR _ UNCERTAINTY _TITLE ON2_ NADIR _ UNCERTAINTY _UNITS | FLOAT STRING STRING | [Ndd] - | Error of ON2 at nadir “Error in ratio of the O to N2 vertical column densities at nadir”. “none” |
| ON2 ON2_ TITLE ON2_ UNITS | FLOAT STRING STRING | [Ndd] - | ON2.on the disk “Ratio of the O to N2 vertical column densities on the disk”. “none” |
| ON2_ UNCERTAINTY ON2_ UNCERTAINTY _TITLE ON2_ UNCERTAINTY _UNITS | FLOAT STRING STRING | [Ndd] - | Error of ON2 on the disk “Error in ratio of the O to N2 vertical column densities on the disk”. “none” |
| TEC TEC _TITLE TEC _UNITS | FLOAT STRING STRING | [Ndd] - | Total electron content “Total electron content”. “TECU (10^16 electron m^-2)” |
| TEC_ UNCERTAINTY TEC_ UNCERTAINTY _TITLE TEC_ UNCERTAINTY _UNITS | FLOAT STRING STRING | [Ndd] - | Error of ON2 at nadir “Error in total electron content”. “TECU (10^16 electron m^-2)” |
| DATA_QUALITY_DISK | UNSIGNED SHORT | [Ndd] | 2 bytes Bit # Meaning if set to true - Spare - Spare - NmF2 uncertainty > 100% signmf2/(1+nmf2) > 1.0 - HmF2 <= 0 km hmf2 <= 0.0 - HmF2 > 500 km hmf2 > 500.0 - HmF2 uncertainty > 50% sighmf2/(1+hmf2) > 1.0 - spare - spare - MeV noise contamination - Mirror pointing unknown |
| DATA_QUALITY_NADIR | UNSIGNED SHORT | [Ndd] | Dayside Nadir Products Bit # Meaning if set to true - Spare - Spare - NmF2 uncertainty > 100% signmf2/(1+nmf2) > 1.0 - HmF2 <= 0 km hmf2 <= 0.0 - HmF2 > 500 km hmf2 > 500.0 - HmF2 uncertainty > 50% sighmf2/(1+hmf2) > 1.0 - spare - spare - MeV noise contamination - Mirror pointing unkown |

# EDR Dayside Limb Data File

PRODUCT VERSION  0105

| Revision | Date | Changes Made |
| --- | --- | --- |
| 0105 | 9/20/11 | Added Topside Sunlit |
| 0104 | 10/25/11 | Added Red Leak correction flag |
| 0103 | 9/22/09 | Redefined Data Quality bits |
| 0102 | 8/1/2008 | Realigned document to data products |
| 0100 | 10/5/2007 | Initial Version of nightside limb definition table |

The table below details the EDR Dayside Limb data files.

The dimensions are:

- Nd (for the dayside alongtrack pixel dimension),

- Md (for the dayside altitude dimension), and

- Nz (for the dayside topside altitude dimension)

| Variable Name | Type | Dimensions | Value/Comment |
| --- | --- | --- | --- |
| Global Dayside Limb data file attributes |  |  |  |
| Time and Position Information |  |  |  |
| TIME TIME_TITLE TIME_UNITS | DOUBLE STRING STRING | [Ndl] - - | The effective time of each scan. ”Time of each scan” ”Seconds since the start of the day” |
| TIME_EPOCH TIME_EPOCH_TITLE TIME_EPOCH_UNITS | DOUBLE STRING STRING | [Ndl] - - | The effective time of each scan. “Time of each scan” “Milli-seconds since 01-Jan-0000 00:00:00.000 (CDF epoch value)” |
| YEAR YEAR_TITLE | INT STRING | [Ndl] - | The year of each rebinned scan “Year of each scan” |
| DOY DOY_TITLE | INT STRING | [Ndl] - | The day of year of each rebinned scan. “Day of year of each scan” |
| LATITUDE LATITUDE_TITLE LATITUDE_UNITS | FLOAT STRING STRING | [Ndl] - - | DMSP spacecraft latitude “S/C geographic latitude at effective times” “Degrees” |
| LONGITUDE LONGITUDE_TITLE LONGITUDE_UNITS | FLOAT STRING STRING | [Ndl] - - | DMSP spacecraft longitude “S/C geographic longitude at effective times” “Degrees” |
| ALTITUDE ALTITUDE_TITLE ALTITUDE_UNITS | FLOAT STRING STRING | [Ndl] - - | DMSP spacecraft altitude “S/C altitude at effective times” “Kilometers” |
| MEAN_TANGENTPOINT_LATITUDE_GEOGRAPHIC _TITLE _UNITS | FLOAT STRING STRING | [Ndl] - - | Pixel averaged tangent point geographic latitude “Mean cross track tangent point geographic latitude” “Degrees” |
| MEAN_TANGENTPOINT_LONGITUDE_GEOGRAPHIC _TITLE _UNITS | FLOAT STRING STRING | [Ndl] - - | Pixel averaged cross track tangent point geographic longitude “Mean cross track tangent point geographic longitude” “Degrees” |
| MEAN_TANGENTPOINT_LATITUDE_GEOMAGNETIC _TITLE _UNITS | FLOAT STRING STRING | [Ndl] - - | Pixel averaged tangent point geomagnetic latitude “Mean cross track tangent point geomagnetic latitude” “Degrees” |
| MEAN_TANGENTPOINT_LONGITUDE_GEOMAGNETIC _TITLE _UNITS | FLOAT STRING STRING | [Ndl] - - | Pixel averaged cross track tangent point geomagnetic longitude “Mean cross track tangent point geomagnetic longitude” “Degrees” |
| MEAN_GEOMAGNETIC_LOCAL_TIME _TITLE _UNITS | FLOAT STRING STRING | [Ndl] - - | Pixel averaged tangent point geomagnetic local time “Mean cross track tangent point geomagnetic local time” “hours” |
| TANGENTPOINT_LATITUDE_GEOGRAPHIC _TITLE _UNITS | FLOAT STRING STRING | [Ndl] | Pixel tangent point geographic latitude “Tangent point geographic latitude” “Degrees” |
| TANGENTPOINT_LONGITUDE_GEOGRAPHIC _TITLE _UNITS | FLOAT STRING STRING | [Ndl] | Pixel tangent point geographic longitude “Tangent point geographic longitude” “Degrees” |
| TANGENTPOINT_ALTITUDE TANGENTPOINT_ALTITUDE _TITLE TANGENTPOINT_ALTITUDE_UNITS | FLOAT STRING STRING | [Ndl] | Pixel tangent point altitude “Tangent point altitude” “Kilometers” |
| Calibration parameters |  |  |  |
| DARK_COUNT_CORRECTION DARK_COUNT_CORRECTION_TITLE | INT STRING | - - | Correct counts for dark counts. “Corrected for Dark counts (0/1 – No/Yes).” |
| SCATTER_LIGHT_1216_CORRECTION SCATTER_LIGHT_1216_CORRECTION_TITLE | INT STRING | - - | Correct counts for 1216 scattered lights. “Corrected for 1216 scattered light (0/1 – No/Yes).” |
| SCATTER_LIGHT_1304_CORRECTION SCATTER_LIGHT_1304_CORRECTION_TITLE | INT STRING | - - | Correct counts for 1304 scattered lights. “Corrected for 1304 scattered light (0/1 – No/Yes).” |
| OVERLAP_1304_1356_CORRECTION OVERLAP_1304_1356_CORRECTION_TITLE | INT STRING | - - | Correct counts for 1304/1356 overlap. “Corrected for 1304/1356 overlap (0/1 – No/Yes).” |
| LONGWAVE_SCATTER_CORRECTION LONGWAVE_SCATTER_CORRECTION_TITLE | INT STRING | - - | Correct counts for long-wave scattered light. “Corrected for long-wave scattered light (0/1 – No/Yes).” |
| RED_LEAK_CORRECTION RED_LEAK_CORRECTION.TITLE | INT STRING | - - | Flag to indicate whether red leak correction has been applied. "Corrected for red leaked light (0/1 - No/Yes)" |
| Electron and Neutral density profile data |  |  |  |
| QEUV QEUV_TITLE QEUV_UNITS | FLOAT STRING STRING | - | Mean Solar Qeuv averaged over all data in input SDR “Mean integrated solar irradiance from 5 to 45 nm” “ergs cm^-2 s^-1” |
| QEUV_UNCERTAINTY QEUV_UNCERTAINTY.TITLE QEUV_UNCERTAINTY.UNITS | FLOAT STRING STRING | - - - | Mean solar Qeuv uncertainty 'Uncertainty of integrated solar irradiance from 5 to 45 nm” “percent” |
| ON2 ON2.TITLE ON2.UNITS | FLOAT STRING STRING | [Ndl] - - | Oxygen to Nitrogen density ratio “Ratio of the O to N2 vertical column densities” “none” |
| ON2_UNCERTAINTY ON2_UNCERTAINTY.TITLE ON2_UNCERTAINTY..UNITS | FLOAT STRING STRING | [Ndl] - - | Uncertainty of the Oxygen to Nitrogen density ratio “Uncertainty in ratio of the O to N2 vertical column densities” “percent” |
| NMF2 NMF2.TITLE NMF2.UNITS | FLOAT STRING STRING | [Ndl] - - | Electron Number density at the F-region maximum “Maximum number density of electrons" "cm^-3" |
| NMF2_UNCERTAINTY NMF2_UNCERTAINTY.TITLE NMF2_UNCERTAINTY.UNITS | FLOAT STRING STRING | [Ndl] - - | Uncertainty of the electron number density variance at the F-region maximum “Uncertainty in the maximum number density of electrons" "percent" |
| HMF2 HMF2.TITLE HMF2.UNITS | FLOAT STRING STRING | [Ndl] - - | Altitude of the NmF2 peak density “Height of the peak F-region electron density" "km" |
| HMF2_UNCERTAINTY HMF2_UNCERTAINTY.TITLE HMF2_UNCERTAINTY.UNITS | FLOAT STRING STRING | [Ndl] - - | Uncertainty of the NmF2 peak density altitude "Uncertainty in the height of the F-region electron density" "percent" |
| TEC TEC.TITLE TEC.UNITS | FLOAT STRING STRING | [Ndl] - - | Total electron column density "Total electron content" "TECU (10^16 electron m^-2)" |
| TEC_UNCERTAINTY TEC_UNCERTAINTY.TITLE TEC_UNCERTAINTY.UNITS | FLOAT STRING STRING | [Ndl] - - | Total electron column density uncertainty "Uncertainty in total electron content" "percent" |
| N2DP N2DP.TITLE N2DP.UNITS | FLOAT STRING STRING | [Ndl, Mdl] - - | Molecular nitrogen density "N2 density profile" "cm^-3" |
| N2DP_UNCERTAINTY N2DP_UNCERTAINTY.TITLE N2DP_UNCERTAINTY.UNITS | FLOAT STRING STRING | [Ndl, Mdl] - - | Molecular nitrogen density uncertainty "Uncertainty in the N2 density profile" "percent" |
| O2DP O2DP.TITLE O2DP.UNITS | FLOAT STRING STRING | [Ndl, Mdl] - - | Molecular Oxygen density "O2 density profile" "cm^-3 |
| O2DP_UNCERTAINTY O2DP_UNCERTAINTY.TITLE O2DP_UNCERTAINTY.UNITS | FLOAT STRING STRING | [Ndl, Mdl] - - | Molecular Oxygen density uncertainty "Uncertainty in the O2 density profile" "percent" |
| ODP ODP.TITLE ODP.UNITS | FLOAT STRING STRING | [Ndl, Mdl] - - | Atomic Oxygen density "O density profile" "cm^-3" |
| ODP_UNCERTAINTY ODP_UNCERTAINTY.TITLE ODP_UNCERTAINTY.UNITS | FLOAT STRING STRING | [Ndl, Mdl] - - | Atomic Oxygen density uncertainty "Uncertainty in the O density profile" "percent" |
| NMF2_FLAG NMF2_FLAG.TITLE NMF2.UNITS | FLOAT STRING STRING | [Ndl] - - | Data quality flagged as 1 if NmF2 is good "Data quality flag for NMF2" "none" |
| HMF2_FLAG HMF2_FLAG.TITLE HMF2_FLAG.UNITS | FLOAT STRING STRING | [Ndl] - - | Data quality flagged as 1 if HmF2 is good "Data quality flag for HMF2" "none" |
| TEC_FLAG TEC_FLAG.TITLE TEC_FLAG.UNITS | FLOAT STRING STRING | [Ndl] - - | Data quality flagged as 1 if TEC is good "Data quality flag for TEC" "none" |
| TOPSIDE_Z0 TOPSIDE_Z0.TITLE TOPSIDE_Z0.UNITS | FLOAT STRING STRING | [Ndl] - - | Minimum altitude for fitting topside EDP “the minimum height acceptable for topside limb fitting” “km” |
| TOPSIDE_N_AT_Z0 TOPSIDE_N_AT_Z0.TITLE TOPSIDE_N_AT_Z0.UNITS | FLOAT STRING STRING | [Ndl] - - | The topside electron density at the minimum altitude “the electron density at Z0” “electrons per cc” |
| TOPSIDE_N_AT_Z0_UNCERTAINTY TOPSIDE_N_AT_Z0_UNCERTAINTY.TITLE TOPSIDE_N_AT_Z0_UNCERTAINTY.UNITS | FLOAT STRING STRING | [Ndl] - - | Estimated error in the topside electron density “uncertainty in Topside electron number density at Z0 “electrons per cc” |
| TOPSIDE_H TOPSIDE_H.TITLE TOPSIDE_H.UNITS | FLOAT STRING STRING | [Ndl] - - | atmospheric scale height “Scale height of exponential fit to the atmosphere” “km” |
| TOPSIDE_H_UNCERTAINTY TOPSIDE_H_UNCERTAINTY.TITLE TOPSIDE_H_UNCERTAINTY.UNITS | FLOAT STRING STRING | [Ndl] - - | Error estimate of the atmospheric scale height “Uncertainty in atmosphere scale height H” “km” |
| TOPSIDE_EDP | FLOAT STRING STRING | [Nd,Nz] | Electron densities at topside altitudes “Electron Densities at topside_altitudes” “electrons per cubic centimeter” |
| TOPSIDE_ALTITUDES | FLOAT STRING STRING | [Nd,Nz] | Altitudes where topside electron densities are fit. “altitudes where topside_edp are fit” “km” |
| TOPSIDE_CHISQ | FLOAT STRING | [Nd] | Chi-squared of the topside fit “The topside EDP fit chi-squared value” |
| TOPSIDE_FIT_Q | FLOAT STRING | [Nd] | Topside fit Q parameter “indicates the quality of the topside EDP fit – 0 means a poor fit while 1 is an excellent fit.: |
| DATA_QUALITY DATA_QUALITY.TITLE DATA_QUALITY.UNITS | FLOAT STRING STRING | [Ndl] - - | Bit # Meaning if set to true - Topside EDP fit unsuccessful – did not converge - Topside EDP fit failed (singular matrix or unbounded chisquared - NmF2 is bad (outside physical limits) - Topside_N_at_Z0 uncertatinty > Topside_N_at_Z0 - Topside_H <10 km or Topside_H > 200 km - HmF2 is bad ( < 0 or > 600 km) - Not enough points to fit topside EDP - Spare - MeV noise contamination in pixel - Mirror pointing is unknown, so results are bad. - Topside_H_uncertaity > Topside_H 11-15 Spare “See EDR data document for description” “none” |

# EDR Aurora Data File

PRODUCT VERSION  0104

| Revision | Date | Changes Made |
| --- | --- | --- |
| 0104 | 9/22/09 | Added global data quality flag |
| 1.0.3 | 6/15/09 | Added PROTON_FLAG_NORTH/SOUTH and HEMISPHERE_POWER_NORTH/SOUTH |
| 1.0.2 | 8/1/08 | Realigned document to data products |
| 1.0.1 | 11/16/07 | Missing documentation for NmE, HmE |
| 1.0.0 | 1/5/2007 | Initial Version of Aurora definition table |

The table below details the EDR Auroral file.

| Variable Name | Type | Dimensions | Value/Comment |
| --- | --- | --- | --- |
| Global Auroral data file attributes |  |  |  |
| AURORA_EDRS_MAXWELLIAN_TABLE_NAME | STRING | - | e.g. “SSUSIF16_AURORA_EDRS_MAXWELLIAN_TABLE_v0000.dat” |
| AURORA_EDRS_MAXWELLIAN_TABLE_CREATED | STRING | - | Date when Aurora Maxwellian table created |
| AURORA_EDRS_GAUSSIAN_TABLE_NAME | STRING | - | e.g. “SSUSIF16_AURORA_EDRS_GAUSSIAN_TABLE_v0000.dat” |
| AURORA_EDRS_GAUSSIAN_TABLE_CREATED | STRING | - | Date when Aurora Gaussian table created |
| DAYGLOW_TABLE_NAME | STRING | - | e.g. “SSUSIF16_DAYGLOW_TABLE_v0000.dat” |
| DAYGLOW_TABLE_CREATED | STRING | - | Date when Dayglow table created |
| GEO_MAG_TABLE_NAME | STRING | - | e.g. “SSUSIF16_GEO_MAG_TABLE_v0000.dat” |
| GEO_MAG_TABLE_CREATED | STRING | - | Date when Geo Mag table created |
| L1B_DAYGLOW_TREATED_TABLE_NAME | STRING | - | e.g. “SSUSIF16_L1B_DAYGLOW_TREATED_TABLE_v0000.dat” |
| L1B_DAYGLOW_TREATED _TABLE_CREATED | STRING | - | Date when L1B dayglow treated table created |
| MODEL_MLAT_MLT_TABLE_NAME | STRING | - | e.g. “SSUSIF16_MODEL_MLAT_MLT_TABLE_v0000.dat” |
| MODEL_MLAT_MLT _TABLE_CREATED | STRING | - | Date when Model MLat MLT table created |
| MODEL_BOUNDARY_KP_TABLE_NAME | STRING | - | e.g. “SSUSIF16_MODEL_BOUNDARY_KP_TABLE_v0000.dat” |
| MODEL_BOUNDARY_KP_TABLE_CREATED | STRING | - | Date when Model boundary Kp table created |
| QUIET_NO_NOISE_TABLE_NAME | STRING | - | e.g. “SSUSIF16_QUIET_NO_NOISE_TABLE_v0000.dat” |
| QUIET_NO_NOISE_TABLE_CREATED | STRING | - | Date when Quiet no noise table created |
| SCAN_TIME_TABLE_NAME | STRING | - | e.g. “SSUSIF16_SCAN_TIME_TABLE_v0000.dat” |
| SCAN_TIME_TABLE_CREATED | STRING | - | Date when Scan time table created |
| *STATISTICAL_AURORA_DATA_TABLE_NAME | STRING | - | e.g. “SSUSI_STATISTICAL_AURORA_TABLE_v0000.dat” |
| *STATISTICAL_AURORA_DATA_TABLE_CREATED | STRING | - | Date when Statistical Aurora data table created |
| PIXELSIZE_GEOMAGNETIC_LATITUDE PIXELSIZE_GEOMAGNETIC_LATITUDE_TITLE PIXELSIZE_GEOMAGNETIC_LATITUDE_UNITS | FLOAT STRING STRING | - - - | Geomagnetic latitude pixel size “Geomagnetic latitude pixel size” “Degrees” |
| PIXELSIZE_GEOMAGNETIC_LONGITUDE PIXELSIZE_GEOMAGNETIC_ LONGITUDE _TITLE PIXELSIZE_GEOMAGNETIC_ LONGITUDE _UNITS | FLOAT STRING STRING | - - - | Geomagnetic longitude pixel size “Geomagnetic longitude pixel size” “Degrees” |
| NORTH_DATA NORTH_DATA_TITLE NORTH_DATA_UNITS | BOOLEAN STRING STRING | - - - | Data availability check “Data available check for north”. “0=w/out data, 1=w/data” |
| SOUTH_DATA SOUTH _DATA_TITLE SOUTH _DATA_UNITS | BOOLEAN STRING STRING | - - - | Data availability check “Data available check for south”. “0=w/out data, 1=w/data” |
| NORTH_SWATH_BOUNDARY NORTH_SWATH_BOUNDARY_TITLE NORTH_SWATH_BOUNDARY_UNITS | BOOLEAN STRING STRING | - - - | Swath boundary data availability check “Data available check for north swath boundary”. “0=w/out data, 1=w/data” |
| SOUTH_SWATH_BOUNDARY SOUTH _SWATH_BOUNDARY_TITLE SOUTH _SWATH_BOUNDARY_UNITS | BOOLEAN STRING STRING | - - - | Swath boundary data availability check “Data available check for south swath boundary”. “0=w/out data, 1=w/data” |
| Time and Position Information |  |  |  |
| TIME TIME_TITLE TIME_UNITS | DOUBLE STRING STRING | - - - | The effective time of each rebinned scan. “Time of each rebinned scan” ”Seconds since the start of the day” |
| TIME_EPOCH TIME_EPOCH_TITLE TIME_EPOCH_UNITS | DOUBLE STRING STRING | - - - | The effective time of each rebinned scan. ”Time of each rebinned scan” ”Milli-seconds since 01-Jan-0000 00:00:00.000 (CDF epoch value)” |
| YEAR YEAR_TITLE | INT STRING | - - | The year of each rebinned scan. “Year of each rebinned scan” |
| DOY DOY_TITLE | INT STRING | - - | The day of year of each rebinned scan. “Day of year of each rebinned scan” |
| LATITUDE LATITUDE_TITLE LATITUDE_UNITS | FLOAT STRING STRING | [#scans] - - | DMSP spacecraft Latitude. “S/C geographic latitude at effective times” “Degrees” |
| LONGITUDE LONGITUDE_TITLE LONGITUDE_UNITS | FLOAT STRING STRING | [#scans] - - | DMSP spacecraft Longitude. “S/C geographic longitude at effective times” “Degrees” |
| ALTITUDE ALTITUDE_TITLE ALTITUDE_UNITS | FLOAT STRING STRING | [#scans] - - | DMSP spacecraft Altitude. “S/C altitude at effective times” “Kilometers” |
| GEOMAGNETIC_AURORAL_LATITUDE GEOMAGNETIC_ AURORAL _LATITUDE_TITLE GEOMAGNETIC_ AURORAL _LATITUDE_UNITS | FLOAT STRING STRING | [Na, Ma] - - | Rebinned geomagnetic latitude, geolocated using the auroral reference altitude “Auroral geomagnetic latitude” “Degrees” |
| GEOMAGNETIC_ AURORAL _LONGITUDE GEOMAGNETIC_ AURORAL _LONGITUDE_TITLE GEOMAGNETIC_ AURORAL _LONGITUDE_UNITS | FLOAT STRING STRING | [Na, Ma] - - | Rebinned geomagnetic longitude, geolocated using the auroral reference altitude “Auroral geomagnetic longitude” “Degrees” |
| ACROSSTRACKANGLE | FLOAT | [Na, Ma] | Across track angle (theta) on the grid. |
| ALONGTRACKANGLE | FLOAT | [Na] | Along track angle (alpha) on the grid. |
| Calibration parameters | Calibration parameters | Calibration parameters | Calibration parameters |
| DARK_COUNT_CORRECTION DARK_COUNT_CORRECTION_TITLE | INT STRING | - - | Correct counts for dark counts. “Corrected for Dark counts (0/1 – No/Yes).” |
| SCATTER_LIGHT_1216_CORRECTION SCATTER_LIGHT_1216_CORRECTION_TITLE | INT STRING | - - | Correct counts for 1216 scattered lights. “Corrected for 1216 scattered light (0/1 – No/Yes).” |
| SCATTER_LIGHT_1304_CORRECTION SCATTER_LIGHT_1304_CORRECTION_TITLE | INT STRING | - - | Correct counts for 1304 scattered lights. “Corrected for 1304 scattered light (0/1 – No/Yes).” |
| OVERLAP_1304_1356_CORRECTION OVERLAP_1304_1356_CORRECTION_TITLE | INT STRING | - - | Correct counts for 1304/1356 overlap. “Corrected for 1304/1356 overlap (0/1 – No/Yes).” |
| LONGWAVE_SCATTER_CORRECTION LONGWAVE_SCATTER_CORRECTION_TITLE | INT STRING | - - | Correct counts for long-wave scattered light. “Corrected for long-wave scattered light (0/1 – No/Yes).” |
| Auroral Boundary South Data |  |  |  |
| SOUTH_GEOMAGNETIC_LATITUDE SOUTH_GEOMAGNETIC_LATITUDE_TITLE SOUTH_GEOMAGNETIC_LATITUDE_UNITS | FLOAT STRING STRING | [1..1000] - - | Southern Auroral boundary geomagnetic latitude “Southern auroral boundary geomagnetic latitude” “Degrees” |
| SOUTH_GEOMAGNETIC_LONGITUDE SOUTH_GEOMAGNETIC_ LONGITUDE _TITLE SOUTH_GEOMAGNETIC_ LONGITUDE _UNITS | FLOAT STRING STRING | [1..1000] - - | Southern Auroral boundary geomagnetic longitude “Southern auroral boundary geomagnetic longitude” “Degrees” |
| SOUTH_GEOMAGNETIC_LOCAL_TIME SOUTH_GEOMAGNETIC_ LOCAL_TIME _TITLE SOUTH_GEOMAGNETIC_ LOCAL_TIME _UNITS | FLOAT STRING STRING | [1..1000] - - | Southern Auroral boundary magnetic local time “Southern auroral boundary magnetic local time” “Hours” |
| SOUTH_GEOGRAPHIC_LATITUDE SOUTH_ GEOGRAPHIC _LATITUDE_TITLE SOUTH_ GEOGRAPHIC _LATITUDE_UNITS | FLOAT STRING STRING | [1..1000] - - | Southern Auroral boundary geographic latitude “Southern auroral boundary geographic latitude” “Degrees” |
| SOUTH_ GEOGRAPHIC _LONGITUDE SOUTH_ GEOGRAPHIC _ LONGITUDE _TITLE SOUTH_ GEOGRAPHIC _ LONGITUDE _UNITS | FLOAT STRING STRING | [1..1000] - - | Southern Auroral boundary geographic longitude “Southern auroral boundary geographic longitude” “Degrees” |
| MODEL_ SOUTH_GEOMAGNETIC_LATITUDE MODEL _SOUTH_GEOMAGNETIC_LATITUDE_TITLE MODEL_ SOUTH_GEOMAGNETIC_LATITUDE_UNITS | FLOAT STRING STRING | [1..1000] - - | Model southern Auroral boundary geomagnetic latitude “Model southern auroral boundary geomagnetic latitude” “Degrees” |
| MODEL_ SOUTH_GEOMAGNETIC_LONGITUDE MODEL_ SOUTH_GEOMAGNETIC_ LONGITUDE _TITLE MODEL_ SOUTH_GEOMAGNETIC_ LONGITUDE _UNITS | FLOAT STRING STRING | [1..1000] - - | Model southern Auroral boundary geomagnetic longitude “Model southern auroral boundary geomagnetic longitude” “Degrees” |
| MODEL_ SOUTH_GEOMAGNETIC_LOCAL_TIME MODEL_ SOUTH_GEOMAGNETIC_ LOCAL_TIME _TITLE MODEL_ SOUTH_GEOMAGNETIC_ LOCAL_TIME _UNITS | FLOAT STRING STRING | [1..1000] - - | Model southern Auroral boundary magnetic local time “Model southern auroral boundary magnetic local time” “Hours” |
| MODEL_SOUTH_GEOGRAPHIC_LATITUDE MODEL_SOUTH_ GEOGRAPHIC _LATITUDE_TITLE MODEL_SOUTH_ GEOGRAPHIC _LATITUDE_UNITS | FLOAT STRING STRING | [1..1000] - - | Model southern Auroral boundary geographic latitude “Model southern auroral boundary geographic latitude” “Degrees” |
| MODEL_SOUTH_ GEOGRAPHIC _LONGITUDE MODEL_SOUTH_ GEOGRAPHIC _ LONGITUDE _TITLE MODEL_SOUTH_ GEOGRAPHIC _ LONGITUDE _UNITS | FLOAT STRING STRING | [1..1000] - - | Model southern Auroral boundary geographic longitude “Model southern auroral boundary geographic longitude” “Degrees” |
| SOUTH_DATA_QUALITY | UNSIGNED SHORT | [1..1000] | 1-3 bytes Byte 1 Southern Aurora Boundary Bit # Meaning if set to true - TBD - TBD - TBD - Spare - Spare - Spare - Spare - Spare |
| Auroral Boundary North Data |  |  |  |
| NORTH_GEOMAGNETIC_LATITUDE NORTH_GEOMAGNETIC_LATITUDE_TITLE NORTH _GEOMAGNETIC_LATITUDE_UNITS | FLOAT STRING STRING | [1..1000] - - | Northern Auroral boundary geomagnetic latitude “Northern auroral boundary geomagnetic latitude” “Degrees” |
| NORTH _GEOMAGNETIC_LONGITUDE NORTH _GEOMAGNETIC_ LONGITUDE _TITLE NORTH _GEOMAGNETIC_ LONGITUDE _UNITS | FLOAT STRING STRING | [1..1000] - - | Northern Auroral boundary geomagnetic longitude “Northern auroral boundary geomagnetic longitude” “Degrees” |
| NORTH _GEOMAGNETIC_LOCAL_TIME NORTH _GEOMAGNETIC_ LOCAL_TIME _TITLE NORTH _GEOMAGNETIC_ LOCAL_TIME _UNITS | FLOAT STRING STRING | [1..1000] - - | Northern Auroral boundary magnetic local time “Northern auroral boundary magnetic local time” “Hours” |
| NORTH _GEOGRAPHIC_LATITUDE NORTH _ GEOGRAPHIC _LATITUDE_TITLE NORTH _ GEOGRAPHIC _LATITUDE_UNITS | FLOAT STRING STRING | [1..1000] - - | Northern Auroral boundary geographic latitude “Northern auroral boundary geographic latitude” “Degrees” |
| NORTH _ GEOGRAPHIC _LONGITUDE NORTH _ GEOGRAPHIC _ LONGITUDE _TITLE NORTH _ GEOGRAPHIC _ LONGITUDE _UNITS | FLOAT STRING STRING | [1..1000] - - | Northern Auroral boundary geographic longitude “Northern auroral boundary geographic longitude” “Degrees” |
| MODEL_ NORTH _GEOMAGNETIC_LATITUDE MODEL_ NORTH _GEOMAGNETIC_LATITUDE_TITLE MODEL_ NORTH _GEOMAGNETIC_LATITUDE_UNITS | FLOAT STRING STRING | [1..1000] - - | Model northern Auroral boundary geomagnetic latitude “Model northern auroral boundary geomagnetic latitude” “Degrees” |
| MODEL_ NORTH _GEOMAGNETIC_LONGITUDE MODEL_ NORTH _GEOMAGNETIC_ LONGITUDE _TITLE MODEL_ NORTH _GEOMAGNETIC_ LONGITUDE _UNITS | FLOAT STRING STRING | [1..1000] - - | Model northern Auroral boundary geomagnetic longitude “Model northern auroral boundary geomagnetic longitude” “Degrees” |
| MODEL_NORTH _GEOMAGNETIC_LOCAL_TIME MODEL_NORTH _GEOMAGNETIC_ LOCAL_TIME _TITLE MODEL_NORTH _GEOMAGNETIC_ LOCAL_TIME _UNITS | FLOAT STRING STRING | [1..1000] - - | Model northern Auroral boundary magnetic local time “Model northern auroral boundary magnetic local time” “Hours” |
| MODEL_NORTH_GEOGRAPHIC_LATITUDE MODEL_ NORTH _ GEOGRAPHIC _LATITUDE_TITLE MODEL_ NORTH _ GEOGRAPHIC _LATITUDE_UNITS | FLOAT STRING STRING | [1..1000] - - | Model northern Auroral boundary geographic latitude “Model northern auroral boundary geographic latitude” “Degrees” |
| MODEL_ NORTH _ GEOGRAPHIC _LONGITUDE MODEL_ NORTH _ GEOGRAPHIC _ LONGITUDE _TITLE MODEL_ NORTH _ GEOGRAPHIC _ LONGITUDE _UNITS | FLOAT STRING STRING | [1..1000] - - | Model northern Auroral boundary geographic longitude “Model northern auroral boundary geographic longitude” “Degrees” |
| NORTH_DATA_QUALITY | UNSIGNED SHORT | [1..1000] | 2 bytes Byte 1 Northern Aurora Boundary Bit # Meaning if set to true - TBD - TBD - TBD - Spare - Spare - Spare - Spare - Spare |
| Auroral Data |  |  |  |
| ELECTRON_MEAN_NORTH_ENERGY_MAP ELECTRON_MEAN_NORTH_ENERGY_MAP_TITLE ELECTRON_MEAN_NORTH_ENERGY_MAP_UNITS | FLOAT STRING STRING | [Na, Ma] - - | Electron mean energy 2-D map for northern hemisphere in MLat-MLT “Electron mean northern energy map”. “keV” |
| ELECTRON_MEAN_SOUTH_ENERGY_MAP ELECTRON_MEAN_ SOUTH _ENERGY_MAP_TITLE ELECTRON_MEAN_ SOUTH _ENERGY_MAP_UNITS | FLOAT STRING STRING | [Na, Ma] - - | Electron mean energy 2-D map for southern hemisphere in MLat-MLT “Electron mean southern energy map”. “keV” |
| ENERGY_FLUX_NORTH_MAP ENERGY_FLUX_NORTH_MAP_TITLE ENERGY_FLUX_NORTH_MAP_UNITS | FLOAT STRING STRING | [Na, Ma] - - | Electron energy flux 2-D map for northern hemisphere in MLat-MLT “Electron energy flux northern map”. “ergs/s/cm2” |
| ENERGY_FLUX_SOUTH_MAP ENERGY_FLUX_ SOUTH _MAP_TITLE ENERGY_FLUX_ SOUTH _MAP_UNITS | FLOAT STRING STRING | [Na, Ma] - - | Electron energy flux 2-D map for southern hemisphere in MLat-MLT “Electron energy flux southern map”. “ergs/s/cm2” |
| ELECTRON_FLUX_NORTH_BOUNDARY_MAP ELECTRON_FLUX_NORTH_BOUNDARY_MAP_TITLE ELECTRON_FLUX_NORTH_BOUNDARY_MAP_UNITS | FLOAT STRING STRING | [Na, Ma] - - | Electron energy flux map with boundary for northern hemisphere “Electron energy flux northern map with boundary”. “ergs/s/cm2” |
| ELECTRON_FLUX_SOUTH_BOUNDARY_MAP ELECTRON_FLUX_ SOUTH_BOUNDARY_MAP_TITLE ELECTRON_FLUX_ SOUTH_BOUNDARY_MAP_UNITS | FLOAT STRING STRING | [Na, Ma] - - | Electron energy flux map with boundary for southern hemisphere “Electron energy flux southern map with boundary”. “ergs/s/cm2” |
| LATITUDE_GEOMAGNETIC_GRID_MAP LATITUDE_GEOMAGNETIC_GRID_MAP_TITLE LATITUDE_GEOMAGNETIC_GRID_MAP_UNITS | FLOAT STRING STRING | [Na, Ma] - - | Magnetic latitude for the above maps “Magnetic latitude grid map”. “degrees” |
| MLT_GRID_MAP MLT_GRID_MAP_TITLE MLT_GRID_MAP_UNITS | FLOAT STRING STRING | [Na, Ma] - - | Magnetic local time in hours for the above maps “Magnetic local time grid map”. “hours” |
| ELECTRON_ENERGY_FLUX_THRESHOLDS ELECTRON_ENERGY_FLUX_THRESHOLDS_TITLE ELECTRON_ENERGY_FLUX_THRESHOLDS_UNITS | FLOAT STRING STRING | [2] - - | Electron energy flux threshold (north, south) for boundary detection “Electron energy flux threshold”. “Ergs/s/cm2” |
| HME_NORTH HME_NORTH.TITLE HME_NORTH.UNITS | FLOAT STRING STRING | [Na, Ma] | Height of the E-region peak in the north auroral region “HmE North” “km” |
| NME_NORTH NME_NORTH.TITLE NME_NORTH.UNITS | FLOAT STRING STRING | [Na, Ma] | Peak electron density of the E-region in the north auroral region “NmE North” “cm^-3” |
| HME_SOUTH HME_SOUTH.TITLE HME_SOUTH.UNITS | FLOAT STRING STRING | [Na, Ma] | Height of the E-region peak in the south auroral region “HmE South” “km” |
| NME_SOUTH NME_SOUTH.TITLE NME_SOUTH.UNITS | FLOAT STRING STRING | [Na, Ma] | Peak electron density of the E-region in the south auroral region “NmE South” “cm^-3” |
| PROTON_FLAG_NORTH PROTON_FLAG_NORTH_TITLE PROTON_FLAG_NORTH_UNITS | FLOAT STRING STRING | [Na, Ma] - - | Proton flag for north auroral region (possible values = 0/1) “Proton flag north” “N/A” |
| PROTON_FLAG_SOUTH PROTON_FLAG_SOUTH_TITLE PROTON_FLAG_SOUTH_UNITS | FLOAT STRING STRING | [Na, Ma] - - | Proton flag for south auroral region (possible values = 0/1) “Proton flag south” “N/A” |
| HEMISPHERE_POWER_NORTH HEMISPHERE_POWER _NORTH_TITLE HEMISPHERE_POWER _NORTH_UNITS | FLOAT STRING STRING | - - - | Hemisphere power for north auroral region “Hemisphere power north” “GW” |
| HEMISPHERE_POWER_SOUTH HEMISPHERE_POWER _SOUTH_TITLE HEMISPHERE_POWER _SOUTH_UNITS | FLOAT STRING STRING | - - - | Hemisphere power for south auroral region “Hemisphere power south” “GW” |
| DATA_QUALITY_GLOBAL | UNSIGNED SHORT | [1] | Bit # Meaning if set to true - Spare - Spare - Spare - Spare - Spare - Spare - Spare - Spare - Spare - Pointing unknown 10-15 Spare |
| *DATA_QUALITY | UNSIGNED SHORT | [Ma] | 1-3 bytes Byte 1 Aurora Products Bit # Meaning if set to true 0; MeV noise. 1; Dayside. 2; Fair; 0.2<=Q<=2 & nightside & no MeV noise. 3; Poor; Q < 0.2 ergs/s/cm**2, or dayside, or MeV noise. 4; Spare. 5; Spare. 6; Spare. 7-15; Spare. |

# EDR Ionosphere Bubble Data File

PRODUCT VERSION  0101

| Revision | Date | Changes Made |
| --- | --- | --- |
| 1.0.1 | 3/5/2010 | Updated with bubble characterization parameters |
| 1.0.0 | 6/8/2009 | Initial version of ionosphere bubble definition table |

The table below details the EDR Ionosphere Bubble file.

| Variable Name | Type | Dimensions | Value/Comment |
| --- | --- | --- | --- |
| Global Ionosphere Bubble data file attributes |  |  |  |
| DATA_QUALITY_INDEX | UNSIGNED INTEGER | - | Byte 1 Bubble Products Bit # Meaning if set to true - Mev Noise - Mirror Pointing unknown - No equatorial data in file (within +/- 40 deg of mag eq) |
| NSINGVAL | INT | - | Number of singular values used for obtaining structured data in bubble identification code |
| PROB_FALSE_ALARM | FLOAT | - | Probability of false alarm parameter for bubble identification code |
| EPSR | FLOAT | - | Parameter for pocbresler inversion algorithm |
| LAMBDA | FLOAT | - | Parameter for pocbresler inversion algorithm |
| TPARAM | FLOAT | - | Parameter for pocbresler inversion algorithm |
| CONF_THRESHOLD | FLOAT | - | Minimum value of the Gaussian pdf function evaluated at the fraction of the difference between nmf2 and the tomographic projection and the error for a pixel to be considered part of a candidate bubble region. |
| PER_BUBBLE_THRESHOLD | FLOAT | - | Minimum percent of pixels of the xgrid that a candidate bubble must contain to be considered a bubble |
| Ionosphere Bubble Data |  |  |  |
| NBUBBLES | INT | - | The number of bubbles detected |
| CENTROID_LAT TITLE UNITS | DOUBLE STRING STRING | [nbubbles] - - | Centroid geographic latitude “Centroid geographic latitude” “Degrees” |
| CENTROID_LON TITLE UNITS | DOUBLE STRING STRING | [nbubbles] - - | Centroid geographic longitude “Centroid geographic longitude” “Degrees” |
| CENTROID_ALT TITLE UNITS | DOUBLE STRING STRING | [nbubbles] - - | Centroid geographic altitude “Centroid geographic altitude” “km” |
| DOY | STRING | [nbubbles] | Day of year |
| TIME TITLE DESCRIPTION | DOUBLE STRING STRING | [nbubbles] - - | Time “Time” “The nadir time of each scan in seconds since the start of the day” |
| PEAK_LAT TITLE UNITS | DOUBLE STRING STRING | [nbubbles] - - | Geographic latitude of equatorial arc peak “Geographic latitude of equatorial arc peak” “Degrees” |
| PEAK_VALUE TITLE UNITS | DOUBLE STRING STRING | [nbubbles] - - | 1356 radiance at equatorial arc peak “Value of 1356 data at peak” “Rayleighs” |
| CONFIDENCE TITLE UNITS | DOUBLE STRING STRING | [nbubbles] - - | Confidence of bubble detection “Confidence of bubble detection” “Percent” |
| NMF2 TITLE UNITS | DOUBLE STRING STRING | [nbubbles] - - | NMF2 “nmf2” “electrons per cubic meter” |
| HMF2 TITLE UNITS | DOUBLE STRING STRING | [nbubbles] - - | HMF2 “hmf2” “km” |
| MEAN_BUBBLE_ERROR TITLE UNITS | DOUBLE STRING STRING | [nbubbles] - - | Mean error for tomographic projection within bubble region. “Mean error for projection in bubble region” “electrons per cubic meter” |
| STD_BUBBLE_ERROR TITLE UNITS | DOUBLE STRING STRING | [nbubbles] - - | Standard deviation for tomographic projection within bubble region. “Standard deviation for projection in bubble region” “electrons per cubic meter” |
| MEAN_BUBBLE TITLE UNITS | DOUBLE STRING STRING | [nbubbles] - - | Mean density of bubble region in tomographic projection. “Mean density of bubble region” “electrons per cubic meter” |
| STD_BUBBLE TITLE UNITS | DOUBLE STRING STRING | [nbubbles] - - | Standard deviation of bubble region in tomographic projection. “Standard deviation of bubble region” “electrons per cubic meter” |
| RED_LEAK_CORRECTION RED_LEAK_CORRECTION_TITLE | INT STRING | - - | Correct counts for scattered red light leaking into the detector. “Corrected for red leak light (0/1 – No/Yes).” |

# EDR Ionosphere Data File

PRODUCT VERSION  0100

| Revision | Date | Changes Made |
| --- | --- | --- |
| 1.0.1 | 5/5/2012 | Updated to reflect final content of May 2012 code delivery |
| 1.0.0 | 6/15/2011 | Initial version of three dimensional ionosphere definition table |

The table below details the EDR Ionosphere file.

| Variable Name | Type | Dimensions | Value/Comment |
| --- | --- | --- | --- |
| Global Ionosphere Bubble data file attributes |  |  |  |
| DATA_QUALITY_INDEX | UNSIGNED INTEGER | - | Byte 1 Ionosphere Products Bit # Meaning if set to true - Mev Noise - Mirror Pointing unknown |
| EPSR | FLOAT | - | Parameter for tomographic inversion algorithm |
| LAMBDA | FLOAT | - | Parameter for tomographic inversion algorithm |
| TPARAM | FLOAT | - | Parameter for tomographic inversion algorithm |
| CONF_THRESHOLD | FLOAT | - | Minimum value of the Gaussian pdf function evaluated at the fraction of the difference between nmf2 and the tomographic projection and the error for a pixel to be considered part of a candidate bubble region. |
| EARTHRAD | DOUBLE | - | The radius of the earth used in processing (km). |
| ALTSIZE | INT | - | The number of altitude grids in the reconstruction. |
| LONSIZE | INT | - | The number of longitude grids in the reconstruction. |
| SCANTHRESH | INT | - | The minimum number of scans required in order to complete a three-dimensional reconstruction. |
| ALTRANGE | DOUBLE | - | The altitude range (km) for which the three-dimensional reconstruction is completed. |
| MINALT | DOUBLE | - | The minimum altitude value (km) for which the reconstruction is completed. |
| MINVALALT | DOUBLE | - | The minimum altitude for which a depleted region is detectable. |
| MAXVALALT | DOUBLE | - | The maximum altitude for which a depleted region is detectable. |
| VOLTHRESH | DOUBLE | - | The minimum volume a candidate depleted region has to have in order to be considered a depleted region. |
| SZALAT | DOUBLE | - | Nightside data is determined by sza > SZALAT |
| MAGLATLIMIT | DOUBLE | - | The latitude bound for the geomagnetic reconstruction algorithm. |
| GEOLATLIMIT | DOUBLE | - | The latitude bound for the geographic reconstruction algorithm. |
| SCANSUSED | INTEGER | - | The number of scans used in the reconstruction. |
| STD_DEP_TH | DOUBLE | - | The minimum value of the standard deviation of the electron density inside a candidate depleted region for it to be considered a depleted region |
| DEPTH_TH | DOUBLE | - | The minimum latitudinal depth a candidate depleted region has to have in order to be considered a depleted region |
| Ionosphere Data |  |  |  |
| NDEPS | INT | - | Number of depleted regions detected. |
| TIME TITLE DESCRIPTION | INT STRING STRING | [NDEPS] | Time “Time” “The time associated with the scan nearest to the depleted region centroid. (seconds since start of the day)” |
| DOY | INT | [NDEPS] | Day of year |
| CONFIDENCE TITLE UNITS | DOUBLE STRING STRING | [NDEPS] | Confidence of depleted region detection. “Confidence of depleted region detection.” “percent” |
| CENTROID_LAT TITLE UNITS | DOUBLE STRING STRING | [NDEPS] | Centroid geographic latitude “Centroid geographic latitude” “degrees” |
| CENTROID_LON TITLE UNITS | DOUBLE STRING STRING | [NDEPS] | Centroid geographic longitude “Centroid geographic longitude” “degrees” |
| CENTROID_ALT TITLE UNITS | DOUBLE STRING STRING | [NDEPS] | Centroid geographic altitude “Centroid geographic altitude” “km” |
| MEDIAN_DEP TITLE UNITS | DOUBLE STRING STRING | [NDEPS] | Median electron density of depleted region. “Median electron density of depleted region.” “electrons per cubic cm” |
| STD_DEP TITLE UNITS | DOUBLE STRING STRING | [NDEPS] | Standard deviation of depleted region electron density. “Standard deviation of depleted region”’ “electrons per cubic cm” |
| MEDIAN_DEP_ERROR TITLE UNITS | DOUBLE STRING STRING | [NDEPS] | Median uncertainty for reconstruction in depleted region. “Median uncertainty for reconstruction in depleted region” “electrons per cubic cm” |
| DVOL TITLE UNITS | DOUBLE STRING STRING | [NDEPS] | Volume of depleted region. “Volume of depleted region” “km^3” |
| DEPTH TITLE UNITS | DOUBLE STRING STRING | [NDEPS] | Latitudinal depth of depleted region “Latitudinal depth of depleted region” “degrees” |
| ORIENTATION TITLE UNITS | DOUBLE STRING STRING | [NDEPS] | Orientation of depleted region. “Orientation of depleted region” “degrees” |
| NPEAKS | INT | - | Number of peaks detected. |
| PEAKLATS TITLE UNITS | DOUBLE STRING STRING | [NPEAKS] - - | Geographic latitude of equatorial arc peak “Geographic latitude of equatorial arc peak” “Degrees” |
| PEAKLONS TITLE UNITS | DOUBLE STRING STRING | [NPEAKS] - - | Geographic longitude for peak latitude. “Geographic longitude for peak latitude” “Degrees” |
| PEAKSCONT TITLE | INT STRING | [NPEAKS] - - | Peak contained in data cube indicator “Equatorial arc contained in data cube, 1-yes, 0-no” |
| NMF2 TITLE UNITS | DOUBLE STRING STRING | [NGRIDS] - - | NMF2 for each 2D reconstruction in the data cube. “Maximum number density of electrons” “electrons per cubic meter” |
| HMF2 TITLE UNITS | DOUBLE STRING STRING | [NGRIDS] - - | HMF2 for each 2D reconstruction in the data cube. “Height of the peak F-region electron density” “km” |
| SCANS TITLE | LONG STRING | [NGRIDS] | The scan number of the center scan for each 2D reconstruction in the data cube. “The scan number of the center scan for each 2D reconstruction in the data cube” |
| MAGSTD TITLE UNITS | DOUBLE STRING STRING | [NDEPS] | Standard deviation of depleted region magnetic longitude centroid over depletion latitudes – measures field alignment. “Alignment deviation in depleted region”’ “degrees” |
| ED_CUBE TITLE UNITS | DOUBLE STRING STRING | [L,A, N] | Three-dimensional electron density map. “Three-dimensional electron density map.” “electrons per cubic cm” |
| ED_ERROR TITLE UNITS | DOUBLE STRING STRING | [L,A, N] | Three-dimensional electron density error map. “Three-dimensional electron density error map.” “electrons per cubic cm” |
| ED_REGIONS TITLE UNITS | DOUBLE STRING STRING | [L,A, N] | Three-dimensional electron density region indicator map. “Three-dimensional electron density region indicator map.” |
| DEP_CONF TITLE UNITS | DOUBLE STRING STRING | [L,A, N] | Three dimensional confidence map. “Confidence of depleted region.” “percent” |
| ED_LON TITLE UNITS | DOUBLE STRING STRING | [L,A, N] | Three-dimensional longitude map. “Three dimensional longitude map.” “degrees” |
| ED_LAT TITLE UNITS | DOUBLE STRING STRING | [L,A, N] | Three-dimensional latitude map. “Three-dimensional latitude map.” “degrees” |
| ED_MLON TITLE UNITS | DOUBLE STRING STRING | [L,A, N] | Three-dimensional geomagnetic longitude map. “Three dimensional geomagnetic longitude map.” “degrees” |
| ED_MLAT TITLE UNITS | DOUBLE STRING STRING | [L,A, N] | Three-dimensional geomagnetic latitude map. “Three-dimensional geomagnetic latitude map.” “degrees” |
| ED_ALT TITLE UNITS | DOUBLE STRING STRING | [L,A, N] | Three-dimensional altitude map. “Three-dimensional altitude map.” “km” |
| BUBBLE TITLE UNITS | DOUBLE STRING STRING | [4,X] | The collection of (lon, lat, alt) points that indicate a depleted region. “Depletion area location [lon,lat,z, label]” “[deg, deg, km, label number]” |

# EDR GAIM-LIMB Data File

PRODUCT VERSION  0100

| Revision | Date | Changes Made |
| --- | --- | --- |
| 1.0.0 | 8/8/2011 | Initial Version of GAIM LIMB file definition table |

Note the changes from the SDR-LIMB are the addition of MN subtracted radiances and uncertainties for 1356, and a DQI bit for daylight contaminated data.

The GAIM EDR contains the lowest resolution gridded SSUSI radiances, time, and geolocation information. The raw radoances are taken directly from the SDR.  Other corrections and flags are added to this SDR data, e.g., the estimated contribution from mutual neutralization to the 1356 radiance is removed and the result provided for ingestion into GAIM. Here the cross track dimension (NetCDF name: nCross_G) represented by Mg in the table is the same size as nCross above, but the along track dimension (NetCDF name: nAlong_G) represented by Ng is different.

There is a dimension (NetCDF name: nchan) for the waveband or “color” of the counts and radiance data.  Currently there are 5 colors (121.6, 130.4, 135.6 nm and LBH short and LBH long) and the channel index goes in that order over the range 0-4.  However there is nothing specific to color in SDR processing and if the order changes, the indices will match that specified in the Level 1B description document.

| Variable Name | Type | Dimensions | Value/Comment |
| --- | --- | --- | --- |
| Time and Position Information |  |  |  |
| TIME_GAIM TIME_GAIM.TITLE TIME_GAIM.UNITS | DOUBLE STRING STRING | [Ng] - - | The effective time of each rebinned pixel. “Time of each newly rebinned along track GAIM pixel” “Seconds since the start of the day” |
| TIME_EPOCH_GAIM TIME_EPOCH_GAIM.TITLE TIME_EPOCH_GAIM.UNITS | DOUBLE STRING STRING | [Ng] - - | The effective time of each rebinned pixel. “Time of each newly rebinned along track GAIM pixel” “Epoch Milliseconds” (CDF epoch value) |
| YEAR_GAIM YEAR_GAIM.TITLE | INT STRING | [Ng] - | The year of each rebinned pixel “Year of each newly rebinned GAIM pixel” |
| DOY_GAIM DOY_GAIM.TITLE | INT STRING | [Ng] - | The day of year of each rebinned pixel. “Day of year of each newly rebinned GAIM pixel” |
| ORBIT_GAIM ORBIT GAIM.TITLE | INT String | [Ng] - | Orbit numbers for each grid row. “Orbit number of each newly rebinned along track GAIM pixel” |
| LATITUDE_GAIM LATITUDE_GAIM.TITLE LATITUDE_GAIM.UNITS | FLOAT STRING STRING | [Ng] - - | DMSP spacecraft latitude “S/C geographic latitude at the GAIM grid centers” “degrees” |
| LONGITUDE_GAIM LONGITUDE_GAIM.TITLE LONGITUDE__GAIM.UNITS | FLOAT STRING STRING | [Ng] - | DMSP spacecraft longitude “S/C geographic longitude at the GAIM grid centers” “degrees” |
| ALTITUDE_GAIM ALTITUDE_GAIM.TITLE ALTITUDE_GAIM.UNITS | FLOAT STRING STRING | [Ng] - - | DMSP spacecraft altitude “S/C geographic altitude at the GAIM grid centers” “kilometers” |
| TANGENTPOINT_LATITUDE_GAIM TANGENTPOINT_LATITUDE_GAIM.TITLE TANGENTPOINT_LATITUDE_GAIM.UNITS | FLOAT STRING STRING | [Mg,Ng] - | Rebinned pixel tangent point latitude “Tangent point GAIM geographic latitude” “degrees” |
| TANGENTPOINT_LONGITUDE_GAIM TANGENTPOINT_LONGITUDE_GAIM.TITLE TANGENTPOINT_LONGITUDE_GAIM.UNITS | FLOAT STRING STRING | [Mg,Ng] - | Rebinned pixel tangent point longitude “Tangent point GAIM geographic longitude” “degrees” |
| TANGENTPOINT_ALTITUDE_GAIM TANGENTPOINT_ALTITUDE_GAIM.TITLE TANGENTPOINT_ALTITUDE_GAIM.UNITS | FLOAT STRING STRING | [Mg,Ng] - | Rebinned pixel tangent point altitude “Tangent point GAIM geographic altitude” “kilometers” |
| TANGENTPOINT_SZA_GAIM TANGENTPOINT_SZA_GAIM.TITLE TANGENTPOINT_SZA_GAIM.UNITS | FLOAT STRING STRING | [Mg,Ng] - - | Tangent point solar zenith angle “solar zenith angle for the center of new limb GAIM grid cell.” “degrees” |
| IN_SAA_GAIM IN_SAA__GAIM.TITLE | INT STRING | [Mg,Ng] | Indicator of whether any limb pixels in the bin may have SAA contamination ‘SAA flag: 1 any pixels in GAIM bin were observed while the S/C was in SAA, 0 if not” |
| ACROSSPIXELSIZE_GAIM | INT | - | Number of across track pixels for the rebinned grid (M). |
| ALONGPIXELSIZE_GAIM | INT | - | Number of along track pixels for the rebinned grid (N). |
| EFFECTIVELOOKANGLE_GAIM EFFECTIVELOOKANGLE_GAIM.TITLE EFFECTIVELOOKANGLE_GAIM.UNITS | FLOAT STRING STRING | [Mg,Ng] - - | Effective scan mirror look angles for the rebinned DAY grid (in degrees). “Effective cross track look angle from S/C nadir to GAIM grid tangent points” “Degrees” |
| Re-binned Data |  |  |  |
| LIMBCOUNTSDATA_GAIM LIMBCOUNTSDATA_GAIM.TITLE | FLOAT STRING | [Mg, Ng, 5] - | Rebinned raw pixel counts for the five SSUSI colors “Raw instrument counts in the limb; rebinned to the new GAIM grid” |
| LIMBDECOMP_UNCERTAINTY_GAIM LIMBDECOMP_ UNCERTAINTY_GAIM.TITLE | FLOAT STRING | [Mg, Ng, 5] - | Decompression uncertainty associated with rebinned counts “Decompression errors in the limb; rebinned to the new GAIM grid” |
| EXPOSURE_GAIM EXPOSURE_GAIM.TITLE EXPOSURE_GAIM.UNITS | FLOAT STRING STRING | [Mg, Ng] - - | How many L1B pixel exposures are contained in this grid cell "Number of L1B scan pixels in each limb GAIM grid bin" "Count” |
| SAA_COUNT_GAIM SAA_COUNT_GAIM.TITLE | INT STRING | [Mg, Ng] - | Number of raw pixels in each bin having high 4278 photometer counts (a signature of the South Atlantic Anomaly) “GAIM South Atlantic Anomaly count” |
| Calibrated, background-corrected data |  |  |  |
| LIMB_INTENSITY_GAIM LIMB_INTENSITY_GAIM.TITLE LIMB_INTENSITY_GAIM.UNITS | DOUBLE STRING STRING | [Mg, Ng, 5] - - | Rebinned limb radiances (in Rayleighs) “Imaging mode limb radiance data; rebinned to the new GAIM grid” “Rayleighs” |
| LIMB_RADIANCE_UNCERTAINTY_GAIM LIMB_RADIANCE_UNCERTAINTY_GAIM.TITLE LIMB_RADIANCE_UNCERTAINTY_GAIM.UNITS | DOUBLE STRING STRING | [Mg, Ng, 5] - - | Uncertainty in rebinned limb radiances due to counting statistics and decompression (in Rayleighs) “Limb Radiance error; rebinned to the new GAIM grid” “Rayleighs” |
| LIMB_INTENSITY_MN_SUBTRACTED_GAIM LIMB_INTENSITY_MN_SUBTRACTED_GAIM.TITLE LIMB_INTENSITY_MN_SUBTRACTED_GAIM.UNITS | DOUBLE STRING STRING | [Mg, Ng, 5] - - | Rebinned limb radiances (in Rayleighs) with the estimated 135.6nm contribution from mutual neutralization emission subtracted “Imaging mode MN subtracted limb radiance data; rebinned to the GAIM grid” “Rayleighs” |
| LIMB_RADIANCE_MN_SUBTRACTED_UNCERTAINTY_GAIM LIMB_RADIANCE_MN_SUBTRACTED_UNCERTAINTY_GAIM.TITLE LIMB_RADIANCE_MN_SUBTRACTEDUNCERTAINTY_GAIM.UNITS | DOUBLE STRING STRING | [Mg, Ng, 5] - - | Uncertainty in rebinned limb radiances where the estimated MN contribution to 135.6 nm radiation has been subtracted, due to counting statistics, decompression, and error propagation “Limb MN subtracted Radiance error; rebinned to the GAIM grid” “Rayleighs” |
| LIMB_CALIBRATION_UNCERTAINTY_GAIM LIMB_CALIBRATION_UNCERTAINTY_GAIM.TITLE | DOUBLE STRING | [Mg, Ng, 5] | Calibration uncertainty in rebinned limb radiances (in Rayleighs) “Rebinned limb radiance calibration uncertainty on GAIM grid” |
| DQI_GAIM | INT | [Mg,Ng,5] | Data Quality Indicator for data in bins “Data Quality bitflag: - 0: MeV noise in cell, - 1: SAA - 2: unknown bit position - 3: dayside dayglow region data - 4: unable to subtract Mutual Neutralization component |

# EDR GAIM-DISK Data File

PRODUCT VERSION  0100

| Revision | Date | Changes Made |
| --- | --- | --- |
| 1.0.0 | 8/8/2011 | Initial Version of GAIM-DISK file definition table |

Note the changes from the SDR2-DISK are the addition of MN subtracted radiances and uncertainties for 1356.

The GAIM EDR contains the lowest resolution gridded SSUSI radiances, time, and geolocation information. The raw radoances are taken directly from the SDR.  Other corrections and flags are added to this SDR data, e.g., the estimated contribution from mutual neutralization to the 1356 radiance is removed and the result provided for ingestion into GAIM. In addition to the cross track and along track pixel numbers M and N, we also need S, the number of instrument scans in order to carry forward some L1B information.  (Md,Nd) are the array sizes for the day (cross, along) track arrays and (Mn,Nn) are the array sizes for the night (cross, along) track arrays.

There is a dimension (NetCDF name: nchan) for the waveband or “color” of the counts and radiance data.  Currently there are 5 colors (121.6, 130.4, 135.6 nm and LBH short and LBH long) and the channel index goes in that order over the range 0-4.  However there is nothing specific to color in SDR processing and if the order changes, the indices will match that specified in the Level 1B description document.

| Variable Name | Type | Dimensions |  | Value/Comment |  |
| --- | --- | --- | --- | --- | --- |
| Time and Position Information |  |  |  |  |  |
| TIME_GAIM_DAY TIME GAIM_DAY_TITLE TIME GAIM_DAY_UNITS | DOUBLE STRING STRING | [Nd] - - | The time of each newly re-binned along track day altitude pixel. ‘Time of each newly re-binned along track GAIM pixel’ ‘Seconds since the start of the day’ |  |  |
| YEAR GAIM_DAY YEAR_GAIM_DAY TITLE | INT STRING | [Nd] - | The year of each rebinned day pixel “Year of each newly rebinned for GAIM day pixel” |  |  |
| DOY_GAIM_DAY DOY GAIM_DAY_TITLE | INT STRING | [Nd] - | The day of year of each rebinned day pixel. “Day of year of each newly rebinned for GAIM day pixel” |  |  |
| ORBIT GAIM_DAY ORBIT GAIM_DAY TITLE | INT String | [Nd] - | Orbit numbers for each day grid row. “Orbit number of each newly rebinned for GAIM along track day pixel” |  |  |
| LATITUDE_GAIM_DAY LATITUDE_GAIM_DAY_TITLE LATITUDE_GAIM_DAY_UNITS | FLOAT STRING STRING | [Nd] - - | DMSP S/C Latitude. “S/C geographic latitude, rebinned to the new GAIM day grid” “ degrees” |  |  |
| LONGITUDE_GAIM_DAY LONGITUDE_GAIM_DAY_TITLE LONGITUDE_GAIM_DAY_UNITS | FLOAT STRING STRING | [Nd] - - | DMSP S/C Longitude. “S/C geographic longitude, rebinned to the new GAIM day grid” “degrees” |  |  |
| ALTITUDE_GAIM_DAY ALTITUDE_GAIM_DAY_TITLE ALTITUDE_GAIM_DAY_UNITS | FLOAT STRING STRING | [Nd] - - | DMSP S/C Altitude. “S/C geographic altitude, rebinned to the new GAIM day altitudegrid” “km” |  |  |
| TIME_GAIM_DAY_AURORAL TIME GAIM_DAY_AURORAL _TITLE TIME GAIM_DAY_AURORAL _UNITS | DOUBLE STRING STRING | [Ndda] - - | The time of each newly re-binned along track day altitude pixel. ‘Time of each newly re-binned along track dayside auroral GAIM pixel’ ‘Seconds since the start of the day’ |  |  |
| YEAR GAIM_DAY_AURORAL YEAR_GAIM_DAY_AURORAL TITLE | INT STRING | [Ndda] - | The year of each rebinned day pixel “Year of each newly rebinned for dayside auroral GAIM day pixel” |  |  |
| DOY_GAIM_DAY_AURORAL DOY GAIM_DAY_AURORAL _TITLE | INT STRING | [Ndda] - | The day of year of each rebinned day pixel. “Day of year of each newly rebinned for GAIM dayside auroral pixel” |  |  |
| ORBIT GAIM_DAY_AURORAL ORBIT GAIM_DAY_AURORAL TITLE | INT String | [Ndda] - | Orbit numbers for each day grid row. “Orbit number of each newly rebinned for GAIM along track dayside auroral pixel” |  |  |
| LATITUDE_GAIM_DAY_AURORAL LATITUDE_GAIM_DAY_AURORAL _TITLE LATITUDE_GAIM_DAY_AURORAL _UNITS | FLOAT STRING STRING | [Ndda] - - | DMSP S/C Latitude. “S/C geographic latitude, rebinned to the new GAIM dayside auroral grid” “ degrees” |  |  |
| LONGITUDE_GAIM_DAY_AURORAL LONGITUDE_GAIM_DAY_AURORAL _TITLE LONGITUDE_GAIM_DAY_AURORAL _UNITS | FLOAT STRING STRING | [Ndda] - - | DMSP S/C Longitude. “S/C geographic longitude, rebinned to the new GAIM dayside auroral grid” “degrees” |  |  |
| ALTITUDE_GAIM_DAY_AURORAL ALTITUDE_GAIM_DAY_AURORAL _TITLE ALTITUDE_GAIM_DAY_AURORAL _UNITS | FLOAT STRING STRING | [Ndda] - - | DMSP S/C Altitude. “S/C geographic altitude, rebinned to the new GAIM dayside auroral altitude grid” “km” |  |  |
| TIME_GAIM_NIGHT TIME GAIM_NIGHT_TITLE TIME GAIM_NIGHT_UNITS | DOUBLE STRING STRING | [Nn] - - | The time of each newly re-binned along track night altitude pixel. ‘Time of each newly re-binned for GAIM along track pixel’ ‘Seconds since the start of the day’ |  |  |
| YEAR GAIM_NIGHT YEAR GAIM_NIGHT_TITLE | INT STRING | [Nn] - | The year of each rebinned night pixel “Year of each newly rebinned for GAIM night pixel” |  |  |
| DOY GAIM_NIGHT DOY GAIM_NIGHT_TITLE | INT STRING | [Nn] - | The day of year of each rebinned night pixel. “Day of year of each newly rebinned for GAIM night pixel” |  |  |
| ORBIT GAIM_NIGHT ORBIT GAIM NIGHT_TITLE | INT String | [Nn] - | Orbit numbers for each night grid row. “Orbit number of each newly rebinned for GAIM along track night pixel” |  |  |
| LATITUDE GAIM_NIGHT LATITUDE GAIM_NIGHT_TITLE LATITUDE GAIM_NIGHT_UNITS | FLOAT STRING STRING | [Nn] - - | DMSP S/C Latitude. “S/C Latitude, rebinned to the new GAIM night grid” “degrees” |  |  |
| LONGITUDE GAIM_NIGHT LONGITUDE GAIM_NIGHT_TITLE LONGITUDE GAIM_NIGHT_UNITS | FLOAT STRING STRING | [Nn] - - | DMSP S/C Longitude. “S/C Longitude, rebinned to the new GAO< night grid” “degrees” |  |  |
| ALTITUDE GAIM_NIGHT ALTITUDE GAIM_NIGHT_TITLE ALTITUDE GAIM_NIGHT_UNITS | FLOAT STRING STRING | [Nn] - - | DMSP S/C Altitude. “S/C Altitude, rebinned to the new GAIM night altitude grid” ‘km” |  |  |
| PIERCEPOINT GAIM_NIGHT_LATITUDE PIERCEPOINT GAIM_NIGHT_LATITUDE_TITLE PIERCEPOINT GAIM_NIGHT_LATITUDE_UNITS | FLOAT STRING STRING | [Mn, Nn] - - | Night Piercepoint latitude of each rebinned pixel center “Nightside latitude of the pierce point; rebinned to the new GAIM grid.” ‘degrees” |  |  |
| PIERCEPOINT GAIM_NIGHT_LONGITUDE PIERCEPOINT GAIM_NIGHT_LONGITUDE_TITLE PIERCEPOINT GAIM_NIGHT_LONGITUDE_UNITS | FLOAT STRING STRING | [Mn, Nn] - - | Night Piercepoint longitude of each rebinned pixel center “Nightside longitude of the pierce point; rebinned to the new GAIM grid.” “degrees” |  |  |
| PIERCEPOINT GAIM_NIGHT_ALTITUDE PIERCEPOINT GAIM_NIGHT_ALTITUDE_TITLE PIERCEPOINT GAIM_NIGHT_ALTITUDE _UNITS | FLOAT STRING STRING | [Mn, Nn] - - | Night Piercepoint altitude of each rebinned pixel center “Nightside pierce point altitude for the new GAIM night grid.” “km” |  |  |
| PIERCEPOINT GAIM_NIGHT_SZA PIERCEPOINT GAIM_NIGHT_SZA.TITLE PIERCEPOINT GAIM_NIGHT_SZA.UNITS | FLOAT STRING STRING | [Md, Nd] - - | Night piercepoint solar zenith angle “Nightside solar zenith angle for the center of new GAIM grid cell.” “degrees” |  |  |
| PIERCEPOINT GAIM_DAY_LATITUDE PIERCEPOINT GAIM_DAY_LATITUDE_TITLE PIERCEPOINT GAIM_DAY_LATITUDE_UNITS | FLOAT STRING STRING | [Md, Nd] - | Day piercepoint latitude of each rebinned pixel center “Dayside latitude of the pierce point; rebinned to the new GAIM grid.” “degrees” |  |  |
| PIERCEPOINT GAIM_DAY_LONGITUDE PIERCEPOINT GAIM_DAY_LONGITUDE_TITLE PIERCEPOINT GAIM_DAY_LONGITUDE_UNITS | FLOAT STRING STRING | [Md, Nd] - | Day Piercepoint longitude of each rebinned pixel center “Dayside longitude of the pierce point; rebinned to the new GAIM grid.” “degrees” |  |  |
| PIERCEPOINT GAIM_DAY_ALTITUDE PIERCEPOINT GAIM_DAY_ALTITUDE_TITLE PIERCEPOINT GAIM_DAY_ALTITUDE_UNITS | FLOAT STRING STRING | [Md, Nd] - | Day piercepoint altitude of newly rebinned pixel center “Dayside pierce point altitudes for the new GAIM grid” “km” |  |  |
| PIERCEPOINT GAIM_DAY_SZA PIERCEPOINT GAIM_DAY_SZA.TITLE PIERCEPOINT GAIM_DAY_SZA.UNITS | FLOAT STRING STRING | [Md, Nd] - - | Day piercepoint solar zenith angle “Dayside solar zenith angle for the center of new GAIM grid cell.” “degrees” |  |  |
| PIERCEPOINT GAIM_DAY_AURORAL_LATITUDE PIERCEPOINT GAIM_DAY_AURORAL _LATITUDE_TITLE PIERCEPOINT GAIM_DAY_AURORAL _LATITUDE_UNITS | FLOAT STRING STRING | [Mdda, Ndda] - | Day piercepoint latitude of each rebinned pixel center “Dayside latitude of the pierce point; rebinned to the new GAIM dayside auroral grid.” “degrees” |  |  |
| PIERCEPOINT GAIM_DAY_AURORAL _LONGITUDE PIERCEPOINT GAIM_DAY_AURORAL _LONGITUDE_TITLE PIERCEPOINT GAIM_DAY_AURORAL _LONGITUDE_UNITS | FLOAT STRING STRING | [Mdda, Ndda] - | Day Piercepoint longitude of each rebinned pixel center “Dayside longitude of the pierce point; rebinned to the new GAIM dayside auroral grid.” “degrees” |  |  |
| PIERCEPOINT GAIM_DAY_AURORAL _ALTITUDE PIERCEPOINT GAIM_DAY_AURORAL _ALTITUDE_TITLE PIERCEPOINT GAIM_DAY_AURORAL _ALTITUDE_UNITS | FLOAT STRING STRING | [Mdda, Ndda] - | Day piercepoint altitude of newly rebinned pixel center “Dayside pierce point altitudes for the new GAIM dayside auroral grid” “km” |  |  |
| PIERCEPOINT GAIM_DAY_AURORAL _SZA PIERCEPOINT GAIM_DAY_AURORAL _SZA.TITLE PIERCEPOINT GAIM_DAY_AURORAL _SZA.UNITS | FLOAT STRING STRING | [Mdda, Ndda] - - | Day piercepoint solar zenith angle “Dayside solar zenith angle for the center of new GAIM dayside auroral grid cell.” “degrees” |  |  |
| IN_SAA GAIM_DAY IN_SAA GAIM_DAY_TITLE | INT STRING | [Md,Nd] | Indicator of whether any day pixels in the bin may have SAA contamination ‘SAA GAIM grid cell flag: 1 any pixels in bin were observed while the S/C was in SAA, 0 if not” |  |  |
| IN_SAA GAIM_DAY_AURORAL IN_SAA GAIM_DAY_AURORAL _TITLE | INT STRING | [Mdda,Ndda] | Indicator of whether any day pixels in the bin may have SAA contamination ‘SAA GAIM dayside auroral grid cell flag: 1 any pixels in bin were observed while the S/C was in SAA, 0 if not” |  |  |
| IN_SAA GAIM_NIGHT IN_SAA GAIM_NIGHT_TITLE | INT STRING | [Mn,Nn] | Indicator of whether any night pixels in bin may be contaminated by SAA ‘SAA GAIM grid cell flag: 1 if any binned pixels were observed while S/C was in SAA, 0 if not” |  |  |
| ACROSSPIXELSIZE GAIM_DAY ACROSSPIXELSIZE GAIM_DAY_TITLE ACROSSPIXELSIZE GAIM_DAY_UNITS | FLOAT STRING STRING | [Md] - - | Arclength on the dayside piercepoint surface of across track pixels. “Piercepoint surface arclengths of across track GAIM day pixels" “kilometers” |  |  |
| ALONGPIXELSIZE GAIM_DAY ALONGPIXELSIZE GAIM_DAY_TITLE ALONGPIXELSIZE GAIM_DAY_UNITS | FLOAT STRING STRING | - - - | Arclength on the dayside piercepoint surface of along track pixels "Piercepoint surface arclength of along track GAIM day pixels" “kilometers” |  |  |
| ACROSSPIXELSIZE GAIM_DAY_AURORAL ACROSSPIXELSIZE GAIM_DAY_AURORAL _TITLE ACROSSPIXELSIZE GAIM_DAY_AURORAL _UNITS | FLOAT STRING STRING | [Mdda] - - | Arclength on the dayside piercepoint surface of across track pixels. “Piercepoint surface arclengths of across track GAIM dayside auroral pixels" “kilometers” |  |  |
| ALONGPIXELSIZE GAIM_DAY_AURORAL ALONGPIXELSIZE GAIM_DAY_AURORAL _TITLE ALONGPIXELSIZE GAIM_DAY_AURORAL _UNITS | FLOAT STRING STRING | - - - | Arclength on the dayside piercepoint surface of along track pixels "Piercepoint surface arclength of along track GAIM dayside auroral pixels" “kilometers” |  |  |
| ACROSSPIXELSIZE GAIM_NIGHT ACROSSPIXELSIZE GAIM_NIGHT_TITLE ACROSSPIXELSIZE GAIM_NIGHT_UNITS | FLOAT STRING STRING | [Mn] - - | Arclength on the nightside piercepoint surface of across track pixels. “Piercepoint surface arclengths of across track GAIM night pixels" “kilometers” |  |  |
| ALONGPIXELSIZE GAIM_NIGHT ALONGPIXELSIZE GAIM_NIGHT_TITLE ALONGPIXELSIZE GAIM_NIGHT_UNITS | FLOAT STRING STRING | - - - | Arclength on the nightside Piercepoint surface of along track pixels. "Piercepoint surface arclength of along track GAIM night pixels" “kilometers” |  |  |
| EFFECTIVELOOKANGLE GAIM_DAY EFFECTIVELOOKANGLE GAIM_DAY_TITLE EFFECTIVELOOKANGLE GAIM_DAY_UNITS | FLOAT STRING STRING | [M,N] - - | Effective scan mirror look angles for the rebinned DAY grid (in degrees). “Effective cross track look angle to day GAIM grid centers” “Degrees” |  |  |
| EFFECTIVELOOKANGLE GAIM_DAY_AURORAL EFFECTIVELOOKANGLE GAIM_DAY_AURORAL _TITLE EFFECTIVELOOKANGLE GAIM_DAY_AURORAL _UNITS | FLOAT STRING STRING | [Mdda,Ndda] - - | Effective scan mirror look angles for the rebinned DAY grid (in degrees). “Effective cross track look angle to dayside auroral GAIM grid centers” “Degrees” |  |  |
| EFFECTIVELOOKANGLE GAIM_NIGHT EFFECTIVELOOKANGLE GAIM_NIGHT_TITLE EFFECTIVELOOKANGLE GAIM_NIGHT_UNITS | FLOAT STRING STRING | [M,N] - - | Effective scan mirror look angles for the rebinned NIGHT grid (in degrees). “Effective cross track look angle to night GAIM grid centers” “Degrees” |  |  |
| Re-binned Data |  |  |  |  |  |
| DISKCOUNTSDATA GAIM_DAY DISKCOUNTSDATA GAIM_DAY_TITLE DISKCOUNTSDATA GAIM_DAY_UNITS | FLOAT STRING STRING | [Md, Nd,5] - - |  | Raw pixel data that was re-binned onto the new dayside grid. “Dayside uncorrected decompressed counts in the disk; rebinned to the new GAIM grid.” “Uncorrected decompressed counts” |  |
| DISKDECOMP_ UNCERTAINTY GAIM _DAY DISKDECOMP_ UNCERTAINTY GAIM_DAY_TITLE DISKDECOMP_ UNCERTAINTY GAIM_DAY_UNITS | FLOAT STRING STRING | [Md, Nd,5] - - |  | Decompression error uncertainty that was re-binned onto the new grid. “Dayside Auroral decompression uncertainty for the disk; rebinned to the new grid.” “Uncorrected decompressed counts” |  |
| DISKCOUNTSDATA GAIM_DAY_AURORAL DISKCOUNTSDATA GAIM_DAY_AURORAL _TITLE DISKCOUNTSDATA GAIM_DAY_AURORAL _UNITS | FLOAT STRING STRING | [Mdda, Ndda,5] - - |  | Raw pixel data that was re-binned onto the new dayside grid. “Dayside uncorrected decompressed counts in the disk; rebinned to the new GAIM grid.” “Uncorrected decompressed counts” |  |
| DISKDECOMP_ UNCERTAINTY GAIM _DAY_AURORAL DISKDECOMP_ UNCERTAINTY GAIM_DAY_AURORAL _TITLE DISKDECOMP_ UNCERTAINTY GAIM_DAY_AURORAL _UNITS | FLOAT STRING STRING | [Mdda, Ndda,5] - - |  | Decompression error uncertainty that was re-binned onto the new grid. “Dayside Auroral decompression uncertainty for the disk; rebinned to the new grid.” “Uncorrected decompressed counts” |  |
| EXPOSURE_DAY_GAIM EXPOSURE_DAY_GAIM.TITLE EXPOSURE_DAY_GAIMUNITS | FLOAT STRING STRING | [Md,Nd,5] - - |  | How many L1B pixel exposures are contained in this grid cell "Number of L1B scan pixels in each dayside GAIM grid bin" "count” |  |
| EXPOSURE_DAY_GAIM_AURORAL EXPOSURE_DAY_GAIM_AURORAL.TITLE EXPOSURE_DAY_GAIM_AURORAL UNITS | FLOAT STRING STRING | [Mdda,Ndda,5] - - |  | How many L1B pixel exposures are contained in this grid cell "Number of L1B scan pixels in each dayside auroral GAIM grid bin" "count” |  |
| DISKCOUNTSDATA GAIM_NIGHT DISKCOUNTSDATA GAIM_NIGHT_TITLE DISKCOUNTSDATA GAIM_NIGHT_UNITS | FLOAT STRING STRING | [Mn, Nn,5] - - |  | Raw pixel data that was re-binned onto the new nightside grid. “Nightside uncorrected decompressed counts in the disk; rebinned to the new GAIM grid.” “Uncorrected decompressed counts” |  |
| DISKDECOMP_UNCERTAINTY GAIM_NIGHT DISKDECOMP_ UNCERTAINTY GAIM _NIGHT_TITLE DISKDECOMP_ UNCERTAINTY GAIM_NIGHT_UNITS | FLOAT STRING STRING | [Mn, Nn,5] - - |  | Decompression error uncertainty that was re-binned onto the new grid. “Nightside decompression uncertainty for the disk; rebinned to the new GAIM grid.” “Uncorrected decompressed counts” |  |
| EXPOSURE_NIGHT_GAIM EXPOSURE_NIGHT_GAIM.TITLE EXPOSURE_NIGHT._GAIMUNITS | FLOAT STRING STRING | [Mn,Nn,5] - - |  | How many L1B pixel exposures are contained in this grid cell "Number of L1B scan pixels in each nightside GAIM grid bin" "Count” |  |
| SAA_COUNT GAIM_DAY_AURORAL SAA_COUNT GAIM_DAY_AURORAL _TITLE | INT STRING | [Mdda, Ndda] - |  | Number of raw pixels in each day bin having high 4278 photometer counts (a signature of the South Atlantic Anomaly) “South Atlantic Anomaly count GAIM dayside auroral grid” |  |
| SAA_COUNT GAIM_DAY SAA_COUNT GAIM_DAY_TITLE | INT STRING | [Md, Nd] - |  | Number of raw pixels in each day bin having high 4278 photometer counts (a signature of the South Atlantic Anomaly) “South Atlantic Anomaly count GAIM day grid” |  |
| SAA_COUNT GAIM_NIGHT SAA_COUNT GAIM_NIGHT TITLE | INT STRING | [Mn, Nn] - |  | Number of raw pixels in each night bin having high 4278 photometer counts (a signature of the South Atlantic Anomaly) “South Atlantic Anomaly count GAIM night grid” |  |
| Calibrated, background-corrected data |  |  |  |  |  |
| DISK_INTENSITY GAIM_NIGHT DISK_INTENSITY GAIM_NIGHT_TITLE DISK_INTENSITY GAIM_NIGHT_UNITS | DOUBLE STRING STRING | [Mn, Nn, 5] - - |  |  | Re-calibrated disk radiances, re-binned onto the new grid. “Nightside Imaging Mode Disk Radiance data re-binned to new GAIM grid” “Rayleighs” |
| DISK_INTENSITY_MN_SUBTRACTED GAIM_NIGHT DISK_INTENSITY_MN_SUBTRACTED GAIM_NIGHT_TITLE DISK_INTENSITY_MN_SUBTRACTED GAIM_NIGHT_UNITS | DOUBLE STRING STRING | [Mn, Nn, 5] - - |  |  | Re-calibrated disk radiances, re-binned onto the new grid. “Nightside Imaging Mode Disk Radiance data re-binned to new GAIM grid” “Rayleighs” |
| DISK_INTENSITY GAIM_DAY DISK_INTENSITY GAIM_DAY_TITLE DISK_INTENSITY GAIM_DAY_UNITS | DOUBLE STRING STRING | [Md, Nd, 5] - |  |  | Re-calibrated disk radiances, re-binned onto the new grid. “Dayside Imaging Mode Disk Radiance data re-binned to new GAIM grid” “Rayleighs” |
| DISK_INTENSITY GAIM_DAY_AURORAL DISK_INTENSITY GAIM_DAY_AURORAL _TITLE DISK_INTENSITY GAIM_DAY_AURORAL _UNITS | DOUBLE STRING STRING | [Mdda, Ndda, 5] - |  |  | Re-calibrated disk radiances, re-binned onto the new grid. “Dayside AURORAL Imaging Mode Disk Radiance data re-binned to new GAIM grid” “Rayleighs” |
| DISK_RADIANCE_UNCERTAINTY GAIM_NIGHT DISK_RADIANCE_UNCERTAINTY GAIM_NIGHT .TITLE DISK_RADIANCE_UNCERTAINTY GAIM_NIGHT .UNITS | DOUBLE STRING STRING | [Mn, Nn, 5] - - |  |  | Uncertainty in disk values due to counting statistics and decompression, re-binned onto the new grid. “Nightside disk Counting Statistical Error; rebinned to new GAIM grid” “Rayleighs” |
| DISK_RADIANCE_MN_SUBTRACTED_ UNCERTAINTY GAIM_NIGHT DISK_RADIANCE_MN_SUBTRACTED _UNCERTAINTY GAIM_NIGHT .TITLE DISK_RADIANCE_MN_SUBTRACTED _UNCERTAINTY GAIM_NIGHT .UNITS | DOUBLE STRING STRING | [Mn, Nn, 5] - - |  |  | Uncertainty in disk values due to counting statistics and decompression, re-binned onto the new grid. “Nightside disk Counting Statistical Error; rebinned to new GAIM grid” “Rayleighs” |
| DISK_RADIANCE_UNCERTAINTY GAIM_DAY DISK_RADIANCE_UNCERTAINTY GAIMDAY_TITLE DISK_RADIANCE_UNCERTAINTY GAIM_DAY_UNITS | DOUBLE STRING STRING | [Md, Nd, 5] - - |  |  | Uncertainty in disk values due to counting statistics and decompression, re-binned onto the new grid. “Dayside disk Counting Statistical Error; rebinned to new GAIM grid” “Rayleighs” |
| DISK_RADIANCE_UNCERTAINTY GAIM_DAY_AURORAL DISK_RADIANCE_UNCERTAINTY GAIMDAY_AURORAL _TITLE DISK_RADIANCE_UNCERTAINTY GAIM_DAY_AURORAL _UNITS | DOUBLE STRING STRING | [Mdda, Ndda, 5] - - |  |  | Uncertainty in disk values due to counting statistics and decompression, re-binned onto the new grid. “Dayside auroral disk Counting Statistical Error; rebinned to new GAIM grid” “Rayleighs” |
| DISK_RECTIFIED_INTENSITY GAIM_NIGHT DISK_RECTIFIED_INTENSITY GAIM_NIGHT_TITLE DISK_RECTIFIED_INTENSITY GAIM_NIGHT_UNITS | DOUBLE STRING STRING | [Mn, Nn, 5] - |  |  | Re-calibrated and rectified disk radiances, re-binned onto the new grid. “Nightside Imaging Mode Disk Radiance data – corrected for background and look angle, re-binned to new GAIM grid” “Rayleighs” |
| DISK_RECTIFIED_INTENSITY GAIM_DAY DISK_RECTIFIED_INTENSITY GAIM_DAY_TITLE DISK_RECTIFIED_INTENSITY GAIM_DAY_UNITS | DOUBLE STRING STRING | [Md, Nd, 5] - |  |  | Re-calibrated and rectified disk radiances, re-binned onto the new grid. “Dayside Imaging Mode Disk Radiance data – corrected for background and look angle, re-binned to new GAIM grid” “Rayleighs” |
| DISK_RECTIFIED_INTENSITY GAIM_DAY_AURORAL DISK_RECTIFIED_INTENSITY GAIM_DAY_AURORAL _TITLE DISK_RECTIFIED_INTENSITY GAIM_DAY_AURORAL _UNITS | DOUBLE STRING STRING | [Mdda, Ndda, 5] - |  |  | Re-calibrated and rectified disk radiances, re-binned onto the new grid. “Dayside Auroral Imaging Mode Disk Radiance data – corrected for background and look angle, re-binned to new GAIM grid” “Rayleighs” |
| DISK_RECTIFIED_RADIANCE_UNCERTAINTY_ GAIM_NIGHT DISK_RECTIFIED_RADIANCE_UNCERTAINTY_ GAIM_NIGHT_TITLE DISK_RECTIFIED_RADIANCE_UNCERTAINTY_ GAIM_NIGHT_UNITS | DOUBLE STRING STRING | [Mn, Nn,5] - - |  |  | Uncertainty in rectified disk values due to counting statistics and decompression, re-binned onto the new grid. “Nightside rectified disk Counting Statistical Error; rebinned to new GAIM grid” “Rayleighs” |
| DISK_RECTIFIED_RADIANCE_UNCERTAINTY_ GAIM_DAY DISK_RECTIFIED_RADIANCE_UNCERTAINTY_ GAIM_DAY_TITLE DISK_RECTIFIED_RADIANCE_UNCERTAINTY_ GAIM_DAY_UNITS | DOUBLE STRING STRING | [Md, Nd, 5] - - |  |  | Uncertainty in rectified disk values due to counting statistics and decompression, re-binned onto the new grid. “Dayside rectified disk Counting Statistical Error; rebinned to new GAIM grid” “Rayleighs” |
| DISK_RECTIFIED_RADIANCE_UNCERTAINTY_ GAIM_DAY_AURORAL DISK_RECTIFIED_RADIANCE_UNCERTAINTY_ GAIM_DAY_AURORAL _TITLE DISK_RECTIFIED_RADIANCE_UNCERTAINTY_ GAIM_DAY_AURORAL _UNITS | DOUBLE STRING STRING | [Mdda, Ndda, 5] - - |  |  | Uncertainty in rectified disk values due to counting statistics and decompression, re-binned onto the new grid. “Dayside Auroral rectified disk Counting Statistical Error; rebinned to new GAIM grid” “Rayleighs” |
| DISK_CALIBRATION_UNCERTAINTY GAIM_NIGHT DISK_CALIBRATION_UNCERTAINTY_ GAIM_NIGHT_TITLE | DOUBLE STRING | [Mn, Nn, 5] - |  |  | Disk calibration uncertainty, re-binned onto the new grid. “Nightside Disk Calibration Error; rebinned to new grid” |
| DISK_CALIBRATION_UNCERTAINTY GAIM_DAY_AURORAL DISK_CALIBRATION_UNCERTAINTY GAIM_DAY_AURORAL _TITLE | DOUBLE STRING | [Mdda, Ndda, 5] - |  |  | Disk calibration uncertainty, re-binned onto the new grid. “Dayside Auroral Disk Calibration Error; rebinned to new GAIM grid” |
| DISK_CALIBRATION_UNCERTAINTY GAIM_DAY DISK_CALIBRATION_UNCERTAINTY GAIM_DAY_TITLE | DOUBLE STRING | [Md, Nd, 5] - |  |  | Disk calibration uncertainty, re-binned onto the new grid. “Dayside Disk Calibration Error; rebinned to new GAIM grid” |
| DQI_DAY_GAIM DQI_DAY.TITLE | SHORT STRING | [Md,Nd,5] - |  |  | Data Quality Indicator for data in dayside GAIM grid bins “Dayside GAIM Data Quality bitflag: - 0:MeV noise in cell, - 1: SAA, - 2: unknown mirror position” |
| DQI_DAY_AURORAL _GAIM DQI_DAY_AURORAL.TITLE | SHORT STRING | [Mdda,Ndda,5] - |  |  | Data Quality Indicator for data in dayside GAIM grid bins “Dayside Auroral GAIM Data Quality bitflag: - 0:MeV noise in cell, - 1: SAA, - 2: unknown mirror position” |
| DQI_NIGHT_GAIM DQI_NIGHT.TITLE | SHORT STRING | [Mn,Nn,5] - |  |  | Data Quality Indicator for data in nightside GAIM grid bins “Nightside GAIM Data Quality bitflag: - 0:MeV noise in cell, - 1: SAA, - 2: unknown mirror position - 3: LBH Thresh exceeded” - 4: Unable to correct for MN |