SSUSI LEVEL 1B FILE

APLVersion 1.11.1

Data Product version 0111

Change Log

| #### Revision | #### Date | Changes Made |
| --- | --- | --- |
| 1.11.1 | 1/7/11 | Fixed SLIT_WIDTH variable name (had SLIT, but variable is actually SLIT_WIDTH is the name) |
| 1.11.0 | 8/27/10 | Added Red Leak correction variables |
| 1.10.1 | 9/15/09 | Added bit definition for mirror positions unknown in the per scan Data Quality Index |
| 1.10.0 | 4/30/09 | Added SAA background counts and new data quality indicators. Removed some Aerospace L1B fields that were never implemented. Corrected some minor typos. |
| 1.9.0 | 10/4/08 | Reset numbering for full orbit L1B. Now RA, dec, and star_location fields are filled in. SOFTWARE_VERSION_NUMBER and CALIBRATION PERIOD_VERSION global attributes have been added. |
| 1.7.2 | 2/6/08 | Fixed typo, DISKCOUNTSDATA. LIMBCOUNTSDATA are O/I Uncorrected counts. |
| 1.7.1 | 3/20/07 | SOLAR_ZENITH_ANGLE was in L1B file but not documented. |
| 1.7.0 | 2/7/07 | Changed SCAN_TYPE to SCAN_MODE |
| 1.6.2 | 1/18/07 | Removed “*” from calibration flag variables as they are already in the L1Bs now. |
| 1.6.1 | 1/16/07 | Moved geophysical parameters to globals and made geophysical index names consistent (index_time). (note, technically this change should have caused the minor revision to change, but since no 0106 files have been written, there is no chance of a conflict.) |
| 1.6.0 | 1/12/07 | “Uncommented” geophysical and variables so they are now part of the file. |
| 1.5.4 | 11/30/2006 | Fixed some minor typos. |
| 1.5.3 | 11/29/06 | Added AFWA example to geophysical info section |
| 1.52 | 11/6/06 | Fixed typo in description of DISK_COUNTS_MINUS_BG and LIMB_COUNTS_MINUS_BG |
| 1.5.1 | 10/16/06 | Fixed index order. Separated global attributes into a separate table, removed ECEF look vectors. |
| 1.5.0 | 10/2/06 | Added Title and Units to many fields, added DMSP_NADIR_ECI, DMSP_SOLAR_ECI, DMSP_SOLAR_LAT, and DMSP_SOLAR_LON. Removed DMSP_COORDS_ECEF and LIMB,DISK PIXELERROR terms. Added fields for backgrounds subtracted (1216, 1304, long, dark0 and the value of counts_minus_BG, (un-IO corrected) DISK and LIMB COUNTSDATA with ERROR terms. |
| 1.4.1 | 9/13/06 | Added color index lookup table |
| 1.4.0 | 8/21/06 | Removed TIME_EPHEM and 1SEC values, since they are not useful. Added DMSP_COORDS_TIME for the array of 1 second cadence times used to generate the ephemeris in the DMSP_ tagged variables. Removed photometer times; instead use the PHOTOMETER DMSP TIME OFFSET parameter relating the 1 second times of photometer measurements to DMSP_COORDS_TIME, Added new data product version number = MMmm; MM = data format document major revision, mm = data format document minor revision. |
| 1.3.1 | 8/11/06 | Changed description of time field to indicate that it is the time when scan pointing is nadir. |
| 1.3.0 | 7/21/2006 | Artificially upping data product version numbers to 0013 because there are existing versions of L1B files with version 0012 and 0011 around. The 0013 marker will clearly indicate that this is a new version. Also we will call the version of this file 1.3.0 to correspond to this data product version |
| 1.0 | 7/21/2006 | Denoted TIME_EPHEM as being used. Removed GEOID_MODEL_DESCRIPTION since it was redundant with GEOID_MODEL_USED. |
| 0.9 | 07/19/2006 | Added NODAL_CROSSING_EPOCH to global attributes; Added data product version number that goes into files |
| 0.8 | 02/08/2006 | Put * on fields not supported for 1st AFWA delivery and an – in front of variables that are implemented |
| 0.7 | 01/25/2006 | Initial version of data files based on Aerospace version 1.7 of the L1B document |

The SSUSI Level 1B file contains both limb and disk reduced scan imaging mode data.  It does not contain any spectrograph mode data.  The files are generated using calibration and geolocation routines contained in the APL L1B Generation IDL program.  Each Level 1B file contains a single quarter orbit of data generated from the quarter orbit prep files generated by the reformatter C++ processing program.  Thus, a Level 1B file is constructed from a single quarter orbit Prep File generated by the operational RSDR-to-Prep File code (dependent on when data is actually downlinked to the ground).  Each scan consists of 22 seconds of data. In the descriptions that follow it is assumed that there are N scans of data in a quarter orbit.  Each scan consists of 24 limb integration steps with 8 spatial pixels and 132 disk integration steps with 16 spatial pixels. Therefore, there are 24+132=156 total readouts of the five SSUSI colors every 22-second scan. Note that in the definitions given below the step by step variable sizes are listed as N*22 to indicate that an array is a vector, not a matrix, and that the length of the vector is the product of the number of scans in a given file and the number of seconds in a scan.

The five SSUSI “colors” in image files are really bandpass filters centered on certain ultra-violet wavelengths of interest.  The color indices are mapped to the wavebands as indicated in the table below.

| SSUSI Colors |  |
| --- | --- |
| Color Array Index | Bandpass name |
| 0 | Lyman nanometers) |
| 1 | OI 130.4 nanometers |
| 2 | OI 135.6 nanometers |
| 3 | Lyman-Birge Hopfield 1 (LBH short) |
| 4 | Lyman-Birge-Hopfield 2 (LBH long) |

Those parameters not generated by the current build of the level 1B software are marked with a ‘*’.  They will not appear in the file, but they may be included in a later version of the file.  NetCDF attributes associated with variables are listed together with the variable and are indented.

The Aerospace version of the L1B processing code distinguishes between a “full” and reduced or “subset” version of  L1B files  to be generated. In practice the APL operational code makes files most like the L1B Subset file. The variables in the Aerospace full L1B that are being considered for the APL L1B are marked with a ‘+’ symbol in the following table. The APL files will likely not ever include the variables that only appear in the Aerospace  “full” L1B file.

# L1B Global Attributes

These are the attributes that apply to all data in the file.

| Variable Name | Type | Dimensions | Value/Comment |
| --- | --- | --- | --- |
| FILENAME | STRING | - | e.g.,”SSUSIF16_Av0011r002_2005248REV09722DOAE.image_L1B” “” |
| MISSION | STRING | - | e.g., “F16” |
| DATA_PRODUCT_TYPE | STRING | - | “Level1B Imaging Data” |
| SOURCE | STRING | [#files] | Names of the Prep File(s) used to generate this file |
| DATA_PRODUCT_VERSION | STRING | - | e.g., “0105” – the version of this document without dots in the name |
| DATA_PRODUCT_REVISION | STRING | - | e.g., “002” |
| SOFTWARE_VERSION_NUMBER | STRING | - | Software release sequence number, e.g. “009” |
| SOFTWARE_VERSION | STRING | - | e.g., “1.0.1” |
| SOFTWARE_NAME | STRING | - | “APL Prep to Level1B” |
| CALIBRATION_VERSION_PHOTOMETER | STRING | - | e.g., “001.0” |
| CALIBRATION_TABLES_NAMES | STRING | - | ‘;’ separated list of names of calibration files that were used. |
| CALIBRATION_TABLES_CREATED | STRING | - | ‘;’ separated list of calibration file creation times. |
| CALIBRATION_PERIOD_VERSION | STRING | - | Calibration period plus a count of which calibration version this is., e.g. “D0016” for Cal period D, and V03R11 is the 16th F16 calibration file. |
| DESCRIPTION | STRING | - | e.g., “Level1B Reduced Scan Imaging Mode Data” |
| COMMENT | STRING | - | e.g., “This file needs to include DQIs” |
| HISTORY | STRING | - | e.g., “No revisions” |
| DATE_GENERATED | STRING | - | e.g., “20052489161552UT” |
| STARTING_TIME | STRING | - | e.g., “20052472345500UT” |
| STOPPING_TIME | STRING | - | e.g., “20052480012111UT” |
| STARTING_ORBIT_NUMBER | STRING | - | e.g., “09722” |
| STOPPING_ORBIT_NUMBER | STRING | - | e.g., “09722” |
| NODAL_CROSSING_EPOCH | DOUBLE | - | The CDF EPOCH of the Nodal crossing data in the GWC ephemeris from the Prep file. e.g., “63273228090000.” |
| NODAL_DAY | INTEGER | - | The day of month of the Nodal Crossing |
| NODAL_MONTH | INTEGER | - | The month of the Nodal Crossing |
| NODAL_YEAR | INTEGER | - | The year of the Nodal Crossing |
| PIERCEPOINT_COMMENT | STRING | - | “Pierce point calculations use a reference geoid and a specified pierce point altitude” |
| TANGENTPOINT_COMMENT | STRING | - | “Tangent points quantities are calculated for all LOS at the true tangent point not the PIERCEPOINT_ALTITUDE” |
| GEOID_MODEL_USED | STRING | - | “WGS84” |
| *EPHEMERIS_CODE | STRING | - | e.g., “SGP4 VERSION 1.1” |
| *TLE_LINE1 | STRING | - | First line of the TLE |
| *TLE_LINE2 | STRING | - | Second line of the TLE |
| *TLE_SOURCE | STRING | - | Origin of TLE file |
| *TLE_DATE | STRING | - | Date of TLE file creation |
| *TLE_FILE_NAME | STRING | - | Name of TLE file as stored at APL |
| *EPHEMERIS_CREATION_DATE | STRING | - | e.g., “12/25/2004 23:59” |
| INSTRUMENT_MODE | STRING | - | e.g., “Imaging” |
| SCAN_MODE | STRING | - | e.g., “Reduced” |
| Geophysical information (if available) |  |  |  |
| GEOPHYSICAL_INFO_UPDATE | STRING | - | e.g., “20010418:0000” |
| F10_7_81_DAY | STRING | - | e.g., “ 0.000000” |
| F10_7_DAILY | STRING | - | e.g., “ 0.000000” |
| F10_7_SOURCE | STRING | - | e.g., “DYN_PARAMS_01108.TXT” |
| KP_3_HOUR | STRING | - | e.g., “ 2.00000” |
| KP_DAILY | STRING | - | e.g., “ 2.00000” |
| KP_AP_SOURCE | STRING | - | e.g., “DYN_PARAMS_01108.TXT” |
| AP_DAILY | STRING | - | e.g., “ 2.00000” |

# L1B Variables

In this table N is the number of scans in the file.

| Variable Name | Type | Dimensions | Value/Comment |
| --- | --- | --- | --- |
| Time information follows. Note that the time information is written at two resolutions: the one-second resolution of the DMSP_COORDS_TIME variable and the 22-second resolution of the TIME variable. |  |  |  |
| TIME TIME_TITLE | DOUBLE STRING | N] - | The nadir time of each scan in seconds since the start of the day. “Nadir Time of each scan” |
|  |  | - |  |
| *TIME_EPOCH_DISK TIME_EPOCH_DISK_TITLE TIME_EPOCH_DISK_UNITS | DOUBLE STRING STRING | [132,N] - - | The time of each disk pixel. “Time for each disk pixel” “Milli-seconds since 01-Jan-0000 00:00:00.00 (CDF epoch value)” |
| *TIME_EPOCH_LIMB TIME_EPOCH_LIMB_TITLE TIME_EPOCH_LIMB_UNITS | DOUBLE STRING STRING | [132,N] - - | The time of each limb pixel. “Time for each limb pixel” “Milli-seconds since 01-Jan-0000 00:00:00.00 (CDF epoch value)” |
| The ‘Z-bits’ parameters are included here. |  |  |  |
| COSCROSSINGANGLE | FLOAT | [N] | The cosine of crossing angle of subsatellite track with respect to local meridian. See p. 25 of DMSP Data Specification (IS-YD-821) Rev C. |
| COSLUNARAZIMUTH | FLOAT | [N] | Azimuth of moon relative to direction of satellite flight. However, this parameter seems to have been improperly scaled as it has a value greater than 1. It also exhibits discrete “stair step” behavior. |
| COSSOLARAZIMUTH | FLOAT | [N] | Azimuth of sun relative to direction of satellite flight. It has a value of close to 1 for F16, which doesn’t seem quite right. |
| EPHEMCLOCKPT1 | INT | [N] | Divided by 16 to get a time-like variable in units of seconds that rolls over every 512 seconds. This variable has zeros every ~53 seconds. |
| EPHEMCLOCKPT2 | INT | [N] | Multiply by 8 and then add (EPHEMCLOCKPT1 mod 128) divided by 16 to get the time in elapsed seconds since 0000Z (I think this is the right algorithm). This variable has zeros every ~53 seconds. |
| HEIGHTEARTHRADIUSRATIO | FLOAT | [N] | Ratio of altitude of spacecraft above subsatellite point at mean sea level to the Earth’s radius (R = 6378.145 km). This has a value of ~4400. |
| LUNARELEVATION | FLOAT | [N] | Elevation of moon with respect to local orbit tangent plane (in degrees). |
| LUNARPHASEANGLE | FLOAT | [N] | Angle between lunar and solar position vectors. Units should be degrees, but this has an integer value between 5 and 7, so it does not seem right. |
| SOLARELEVATION | FLOAT | [N] | Elevation of sun with respect to local orbit tangent plane (in degrees). |
| SUBPOINTLAT | FLOAT | [N] | Geodetic latitude of subsatellite point, WGS-72 (degrees). The values are very close to but not quite the same as those in LAT_1. |
| SUBPOINTLONG | FLOAT | [N] | Longitude of subsatellite point, WGS-72 (degrees, from –180 to 180). The values are very close to but not quite the same as those in LONGITUDE.. |
| LATITUDE | FLOAT | [N] | Pre-computed GWC ephemeris: latitude (degrees) at a 22-second (per scan) cadence. The corresponding time array is TIME. Previously named LAT_1. |
| LONGITUDE | FLOAT | [N] | Pre-computed GWC ephemeris: longitude (degrees, 0 to 360) at a 22-second (per scan) cadence. The corresponding time array is TIME. Previously named LONG_1. |
| ALTITUDE | FLOAT | [N] | Pre-computed GWC ephemeris: altitude (km) at a 22-second (per scan) cadence. The corresponding time array is TIME. The values exhibit discrete steps of 1 or 2 km. Previously named ALT_1. |
| JULDAY | INT | [N] | Precomputed GWC ephemeris: Julian day. Previously named JULDAY_1. |
| SATH | FLOAT | [N] | Precomputed GWC ephemeris: ?. Not sure what this is, but it runs from 0 to 360 degrees within one file, so it looks like an orbit angle. Previously named SATH_1. |
| Information in the “DMSP” fields is derived from smoothing and filtering the GWC ephemeris. Later, an ephemeris derived from integration of TLEs may be inserted into these DMSP fields in a later version of the file. |  |  |  |
| DMSP_LATITUDE DMSP_LATITUDE_TITLE DMSP_LATITUDE_UNITS | FLOAT STRING STRING | [N,22] - - | DMSP latitude. For first delivery, using smoothed and filtered GWC ephem data. "Geodetic latitude of the DMSP spacecraft at 1 second resolution" "Degrees” |
| DMSP_LONGITUDE DMSP_LONGITUDE_TITLE DMSP_LONGITUDE_UNITS | FLOAT STRING STRING | [N,22] - - | DMSP east longitude. For first delivery, using smoothed and filtered GWCephem data. "Geodetic longitude of the DMSP spacecraft at 1 second resolution" "Degrees” |
| DMSP_RADIAL_DISTANCE DMSP_RADIAL_DISTANCE_TITLE DMSP_RADIAL_DISTANCE_UNITS | FLOAT STRING STRING | [N,22] - - | DMSP radial distance. For first delivery, using smoothed and filtered GWCephem data. “Radial distance of the DMSP spacecraft from center of earth at 1 second resolution” “km” |
| DMSP_ALTITUDE DMSP_ALTITUDE_UNITS DMSP_ALTITUDE_UNITS | FLOAT STRING STRING | [N,22] - - | DMSP altitude. For first delivery, using smoothed and filtered GWC ephem data. "Altitude of the DMSP spacecraft from the surface of the earth at 1 second resolution" “km” |
| DMSP_COORDS_ECI DMSP_COORDS_ECI_TITLE DMSP_COORDS_ECI_UNITS | FLOAT STRING STRING | [N,3,22] - - | DMSP location in ECI coordinates. For first delivery, derived using smoothed and filtered GWCephem data. "ECI coordinates of the DMSP spacecraft at 1 second resolution" “km” |
| DMSP_NADIR_ECI DMSP_NADIR_ECI_TITLE DMSP_NADIR_ECI_UNITS | FLOAT STRING STRING | [N,3,22] - - | DMSP nadir vector in ECI coordinates. For first delivery, derived using smoothed and filtered GWCephem data. "ECI coordinates of the DMSP spacecraft nadir position at 1 second resolution” “km” |
| DMSP_SOLAR_ECI DMSP_SOLAR_ECI_TITLE DMSP_SOLAR_ECI_UNITS | FLOAT STRING STRING | [N,3,22] - - | Solar location in ECI coordinates. For first delivery, based smoothed and filtered GWCephem data. "ECI position of the sun at 1 second resolution" “km” |
| DMSP_SOLAR_LAT DMSP_SOLAR_LAT_TITLE DMSP_SOLAR_LAT_UNITS | FLOAT STRING STRING | [N,22] - - | Subsolar latitude. For first delivery, derived using smoothed and filtered GWCephem data. "Subsolar latitude at 1 second resolution" “degrees” |
| DMSP_SOLAR_LON DMSP_SOLAR_LON_TITLE DMSP_SOLAR_LON_UNITS | FLOAT STRING STRING | [N,22] - - | Subsolar longitude. For first delivery, derived using smoothed and filtered GWCephem data. "Subsolar longitude at 1 second resolution" “degrees” |
| DMSP_COORDS_TIME DMSP_COORDS_TIME_TITLE DMSP_COORDS_TIME_UNITS | FLOAT STRING STRING | [N,22] - - | 1 second time steps used to generate the “DMSP_” tagged coordinates "Time corresponding to each DMSP coordinate at 1 second resolution “Seconds” |
| SSUSI instrument parameters |  |  |  |
| DETECTOR_USED DETECTOR_TITLE DETECTOR_VALID_RANGE | STRING STRING | [N] - [2] | Has a value of 129 on day 37, 2004. e.g., “Detector Number 1” |
| SLIT_WIDTH SLIT_WIDTH:TITLE SLIT_WIDTH:VALID_RANGE | STRING STRING | [N] - [2] | Has a value of 129 on day 37, 2004. e.g., “Slit width, 0=wide, 1=medium, 2=narrow” |
| SIS data fields |  |  |  |
| DARKCOUNTPIXEL DARKCOUNTPIXEL_TITLE | INT STRING | [N] - | “Dark Count Pixel” |
| DISKPIXELDATA DISKPIXELDATA_TITLE DISKPIXELDATA_UNITS | FLOAT STRING STRING | [N,132,16,5] - - | The raw instrument counts in the disk “Imaging Mode Disk Pixel Data” “O/I Corrected Decompressed Counts” |
| LIMBPIXELDATA LIMBPIXELDATA_TITLE LIMBPIXELDATA_UNITS | FLOAT STRING STRING | [N,24,8,5] - - | The raw instrument counts in the limb “Imaging Mode Limb Pixel Data” “O/I Corrected Decompressed Counts” |
| DISKCOUNTSDATA DISKCOUNTSDATA_TITLE DISKCOUNTSDATA_UNITS | FLOAT STRING STRING | [N,132,16,5] - - | The raw instrument counts in the disk “Imaging Mode Disk Pixel Data” “O/I UNCorrected Decompressed Counts” |
| LIMBCOUNTSDATA LIMBCOUNTSDATA_TITLE LIMBCOUNTSDATA_UNITS | FLOAT STRING STRING | [N,24,8,5] - - | The raw instrument counts in the limb “Imaging Mode Limb Pixel Data” “O/I UNCorrected Decompressed Counts” |
| DISKCOUNTSERROR DISKCOUNTSERROR_TITLE DISKCOUNTSERROR_UNITS | FLOAT STRING STRING | [N,132,16,5] - - | The decompression error for the disk pixels in counts. “Decompression Error” “Counts” |
| LIMBCOUNTSERROR LIMBCOUNTSRROR_TITLE LIMBCOUNTSERROR_UNITS | FLOAT STRING STRING | [N,24,8,5] - - | The decompression error for the limb pixels in counts. “Decompression Error” “Counts” |
| DISKDETECTORRATIO DISKDETECTORRATIO_TITLE | FLOAT STRING | [N,132] - | The output/input count ratio for each disk scan step multiplied by 64. “O/I ratio data for each mirror step per scan” |
| LIMBDETECTORRATIO LIMBDETECTORRATIO_TITLE | FLOAT STRING | [N,24] - | The output/input count ratio for each limb scan step multiplied by 64. “O/I ratio data for each mirror step per scan” |
| PULSEHEIGHTDATA | FLOAT | [N,22] |  |
| FLIGHT_SW_VERSION | INT | [N] | Value imported from PREP file. |
| FLIGHT_SW_COLORTABLE_VERSION | INT | [N] |  |
| DARKCOUNT | INT | [N] | The value of the dark count pixel for a scan. |
| BACKGROUNDCOUNT | INT | [N] | The value of the background count pixel for a scan. |
| MIRRORSTARTPOSITION | INT | [N] |  |
| NADIRPOSITION | INT | [N] |  |
| FAULTSTATUS | INT | [N] |  |
| PREVCOMMAND1 | INT | [N] |  |
| PREVCOMMAND2 | INT | [N] |  |
| SISSTATUS | INT | [N] |  |
| DETECTORCONFIG | INT | [N] |  |
| DETECTORHIGHVOLTAGE | INT | [N] |  |
| SISRADIATORTEMP | INT | [N] |  |
| SCANMIRRORTEMP | INT | [N] |  |
| DET1HIGHVOLTAGE | INT | [N] |  |
| DET2HIGHVOLTAGE | INT | [N] |  |
| FPE1TEMP | INT | [N] |  |
| FPE2TEMP | INT | [N] |  |
| SMBOARDTEMP | INT | [N] |  |
| Information for the photometers |  |  |  |
| PHOTOMETER427COUNT | LONG | [N,22] |  |
| PHOTOMETER629COUNT | LONG | [N,22] |  |
| PHOTOMETER630COUNT | LONG | [N,22] |  |
| FILTER427HEATER | INT | [N] |  |
| FILTER629HEATER | INT | [N] |  |
| FILTER630HEATER | INT | [N] |  |
| PHOTOMETERSTATUS | INT | [N] |  |
| FILTER427SETTEMP | INT | [N] |  |
| FILTER629SETTEMP | INT | [N] |  |
| FILTER630SETTEMP | INT | [N] |  |
| FILTER427TEMP | INT | [N] |  |
| FILTER629TEMP | INT | [N] |  |
| FILTER630TEMP | INT | [N] |  |
| PHVPSTEMP | INT | [N] |  |
| ILLUMSENSOR1 | INT | [N] |  |
| ILLUMSENSOR2 | INT | [N] |  |
| PHOTOMETER427HIGHVOLTAGE | INT | [N] |  |
| PHOTOMETER629HIGHVOLTAGE | INT | [N] |  |
| PHOTOMETER630HIGHVOLTAGE | INT | [N] |  |
| Calibration parameters |  |  |  |
| CALIBRATION_TABLES | INT | - | The number of calibration files that were used in generating this file. |
| DISK_RESPONSIVITIES DISK_RESPONSIVITIES_TITLE | FLOAT STRING | [2,3,156,16,5] - | The responsivity of the SSUSI instrument for the disk from the first calibration file that was used. “Disk Responsivities” |
| LIMB_RESPONSIVITIES LIMB_RESPONSIVITIES_TITLE LIMB RESPONSIVITIES UNITS | FLOAT STRING STRING | [2,3,24,8,5] - | The responsivity of the SSUSI instrument for the limb (1st cal file). “Limb Responsivities” “Cts/sec/pixel/Rayleigh” |
| DISK_INTEGRATION_TIME DISK_INTEGRATION_TIME_TITLE DISK_INTEGRATION_TIME_UNITS | FLOAT STRING STRING | - - - | The integration time for a single disk pixel measurement (1st cal file). “Disk Integration Time” “Seconds” |
| LIMB_INTEGRATION_TIME LIMB_INTEGRATION_TIME_TITLE LIMB_INTEGRATION_TIME_UNITS | FLOAT STRING STRING | - - - | The integration time for a single limb pixel measurement (1st cal file). “Limb Integration Time” “Seconds” |
| DARK_INTEGRATION_TIME DARK_INTEGRATION_TIME_TITLE DARK_INTEGRATION_TIME_UNITS | FLOAT STRING STRING | - - - | The integration time for the SSUSI dark pixel (1st cal file). “Dark Integration Time” “Seconds” |
| DISK_DARK_MASKS DISK_DARK_MASKS_TITLE DISK_DARK_MASKS_UNITS | FLOAT STRING STRING | [2,3,16,5] - - | Dark count mask for the disk (1st cal file). “Disk Dark Mask” “Counts” |
| LIMB_DARK_MASKS LIMB_DARK_MASKS_TITLE LIMB_DARK_MASKS_UNITS | FLOAT STRING STRING | [2,3,8,5] - - | Dark count mask for the limb (1st cal file). “Limb Dark Mask” “Counts” |
| DISK_1216_BACKGROUND_MASK DISK_1216_BACKGROUND_MASK_TITLE | FLOAT STRING | [2,3,16,5] - | 1216 scattered light mask for the disk (1st cal file). “Disk 1216 Background Mask” |
| LIMB_1216_BACKGROUND_MASK LIMB_1216_BACKGROUND_MASK_TITLE | FLOAT STRING | [2,3,8,5] - | 1216 scatter light mask for the limb (1st cal file). “Limb 1216 Background Mask” |
| DISK_1304_BACKGROUND_MASK DISK_1304_BACKGROUND_MASK_TITLE | FLOAT STRING | [2,3,16,5] - | 1304 scattered light mask for the disk (1st cal file). “Disk 1304 Background Mask” |
| LIMB_1304_BACKGROUND_MASK LIMB_1304_BACKGROUND_MASK_TITLE | FLOAT STRING | [2,3,8,5] - | 1304 scatter light mask for the limb (1st cal file). “Limb 1304 Background Mask” |
| DISK_LONG_BACKGROUND_MASK DISK_LONG_BACKGROUND_MASK_TITLE | FLOAT STRING | [2,3,16,5] - | Long wavelength scattered light mask for the disk (1st cal file). “Long Wavelength Background Mask” |
| DISK_CALIBRATION_ERROR_RATIO DISK_CAL_ERROR_RATIO_TITLE | FLOAT STRING | [2,3,156,16,5] - | Calibration uncertainty for the disk (1st cal file). “Disk Calibration Uncertainty” |
| LIMB_CALIBRATION_ERROR_RATIO LIMB_CAL_ERROR_RATIO_TITLE | FLOAT STRING | [2,3,24,8,5] - | Calibration uncertainty for the limb (1st cal file). “Limb Calibration Uncertainty” |
| DARK_COUNT_CORRECTION DARK_COUNT_CORRECTION_TITLE | INT STRING | - - | Correct counts for dark counts. “Corrected for Dark counts (1-Yes, 0-No)” |
| SCATTER_LIGHT_1216_CORRECTION SCATTER_LIGHT_1216_CORRECTION _TITLE | INT STRING | - - | Correct counts for 1216 scattered lights. “Corrected for 1216 scattered light (1-Yes, 0-No)” |
| SCATTER_LIGHT_1304_CORRECTION SCATTER_LIGHT_1304_CORRECTION _TITLE | INT STRING | - - | Correct counts for 1304 scattered lights. “Corrected for 1304 scattered light (1-Yes, 0-No)” |
| OVERLAP_1304_1356_CORRECTION OVERLAP_1304_1356_CORRECTION _TITLE | INT STRING | - - | Correct counts for 1304/1356 overlap. “Corrected for 1304/1356 overlap (1-Yes, 0-No)” |
| LONGWAVE_SCATTER_CORRECTION LONGWAVE_SCATTER_CORRECTION _TITLE | INT STRING | - - | Correct counts for long-wave scattered light. “Corrected for long-wave scattered light (1-Yes, 0-No)” |
| RED_LEAK_CORRECTION RED_LEAK_CORRECTION .TITLE | INT STRING | - - | Correct counts for leaked red light. “Corrected for red light leak (1-Yes, 0-No)” |
| Calibrated, background-corrected data |  |  |  |
| DISK_RADIANCEDATA_INTENSITY DISK_RADIANCEDATA_INTENSITY_TITLE DISK_RADIANCEDATA_INTENSITY_UNITS | FLOAT STRING STRING | [N,132,16,5] - - | The calibrated disk pixel radiances “Imaging Mode Disk Radiance Data – corrected for background” “Rayleighs” |
| LIMB_RADIANCEDATA_INTENSITY LIMB_RADIANCEDATA_INTENSITY_TITLE LIMB_RADIANCEDATA_INTENSITY_UNITS | FLOAT STRING STRING | [N,24,8,5] - - | The calibrated disk pixel radiances “Imaging Mode Limb Radiance Data – corrected for background” “Rayleighs” |
| Error Tracking Variables |  |  |  |
| DISK_CALIBRATIONERROR DISK_CALIBRATIONERROR_TITLE DISK_CALIBRATIONERROR_UNITS | FLOAT STRING STRING | [N,132,16,5] - - | Disk pixel calibration uncertainty “Disk Calibration Error” “Rayleighs” |
| DISK_BG_DARK DISK_BG_DARK_TITLE DISK_BG_DARK_UNITS | FLOAT STRING STRING | [N,132,16,5] - - | Dark background for the disk. “Disk Background - Dark” “Counts” |
| DISK_BG_1216 DISK_BG_1216_TITLE DISK_BG_1216_UNITS | FLOAT STRING STRING | [N,132,16,5] - - | 1216 background for the disk “Disk Background – 1216” “counts” |
| DISK_BG_1304 DISK_BG_1304_TITLE DISK_BG_1304_UNITS | FLOAT STRING STRING | [N,132,16,5] - - | 1304 background for the disk. “Disk Background – 1304” “counts” |
| DISK_BG_LONG DISK_BG_LONG_TITLE DISK_BG_LONG_UNITS | FLOAT STRING STRING | [N,132,16,5] - - | Long wavelength background for the disk. “Disk Background – long” “counts” |
| DISK_BG_SAA DISK_BG_SAA_TITLE DISK_BG_SAA_UNITS | FLOAT STRING STRING | [N,132,16,5] - - | SAA background for the disk. “Disk Background - SAA” “Counts” |
| DISK_BG_RED DISK_BG_RED.TITLE DISK_BG_RED.UNITS | FLOAT STRING STRING | [N,132,16,5] - - | Red leak light background for the disk. “Disk Background – Red Leak” “Counts” |
| DISK_COUNTS_MINUS_BG DISK_COUNTS_MINUS_BG_TITLE DISK_COUNTS_MINUS_BG_UNITS | FLOAT STRING STRING | [N,132,16,5] - - | Remaining counts after all background subtractions for the disk. “Disk Counts after all background subtractions” “counts” |
| DISK_COUNTERROR_TOTAL DISK_COUNTERROR_TOTAL_TITLE DISK_COUNTERROR_TOTAL_UNITS | FLOAT STRING STRING | [N,132,16,5] - - | Total uncertainty in disk pixels from all statistical sources “Disk Statistical Error from all sources (excluding any calibration bias)” “Rayleighs” |
| LIMB_CALIBRATIONERROR LIMB_CALIBRATIONERROR_TITLE LIMB_CALIBRATIONERROR_UNITS | FLOAT STRING STRING | [N,24,8,5] - - | Limb pixel calibration uncertainty “Limb Calibration Error” “Rayleighs” |
| LIMB_BG_DARK LIMB_BG_DARK_TITLE LIMB_BG_DARK_UNITS | FLOAT STRING STRING | [N,24,8,5] - - | Dark background for the limb.. “Limb Background - Dark” “Counts” |
| LIMB_BG_1216 LIMB_BG_1216_TITLE LIMB_BG_1216_UNITS | FLOAT STRING STRING | [N,24,8,5] - - | 1216 background for the limb “Limb Background – 1216” “counts” |
| LIMB_BG_1304 LIMB_BG_1304_TITLE LIMB_BG_1304_UNITS | FLOAT STRING STRING | [N,24,8,5] - - | 1304 background for the limb. “Limb Background – 1304” “counts” |
| LIMB_BG_LONG LIMB_BG_LONG_TITLE LIMB_BG_LONG_UNITS | FLOAT STRING STRING | [N,24,8,5] - - | Long wavelength background for the limb. “Limb Background – long” “counts” |
| LIMB_BG_SAA LIMB_BG_SAA_TITLE LIMB_BG_SAA_UNITS | FLOAT STRING STRING | [N,24,8,5] - - | SAA background for the limb.. “Limb Background - SAA” “Counts” |
| LIMB_COUNTS_MINUS_BG LIMB_COUNTS_MINUS_BG_TITLE LIMB_COUNTS_MINUS_BG_UNITS | FLOAT STRING STRING | [N,24,8,5] - - | Counts remaining after all background are subtracted for the limb. “Limb Counts after all background subtractions” “counts” |
| LIMB_COUNTERROR_TOTAL LIMB_COUNTERROR_TOTAL_TITLE LIMB_COUNTERROR_TOTAL_UNITS | FLOAT STRING STRING | [N,24,8,5] - - | Total uncertainty in limb pixels from all statistical sources “Limb Statistical Error from all sources (excluding any calibration bias)” “Rayleighs” |
| DQI_TOTAL_SCAN DQI_TOTAL_SCAN_TITLE DQI_TOTAL_SCAN_UNITS | UNSIGNED SHORT STRING STRING | [N] - - | Bit Meaning if set to true 7: MeV Noise Present 6: unused 5: Pointing Unknown (mirror position unknown) 4-0 unused “Data Quality Indicator – per scan” “N/A” |
| DQI_COLOR_SCAN DQI_COLOR_SCAN_TITLE DQI_COLOR_SCAN_UNITS | UNSIGNED SHORT STRING STRING | [N,5] - - | Bit Meaning if set to true 7 – 0 unused “Data Quality Indicator – per color and scan” “N/A” |
| Pixel by pixel geolocation information. Note that both day and night pierce point locations are calculated for ALL disk pixels. |  |  |  |
| DISK_SCAN_TIMES DISK_SCAN_TIMES_TITLE DISK_SCAN_TIMES_UNITS | FLOAT STRING STRING | [132] - - | Time for each mirror step in a disk scan. “Time for each mirror step in a disk scan, relative to start of scan” “Seconds” |
| LIMB_SCAN_TIMES LIMB_SCAN_TIMES_TITLE LIMB_SCAN_TIMES_UNITS | FLOAT STRING STRING | [24] - - | Time for each mirror step in a limb scan. “Time for each mirror step in a limb scan, relative to start of scan” “Seconds” |
| SCAN_MOTOR_OFFSETS SCAN_MOTOR_OFFSETS_TITLE SCAN_MOTOR_OFFSETS_UNITS | INT STRING STRING | [N] - - | The scan motor offsets detected for each scan. “Scan motor offsets relative to default scan.” “Steps” |
| DISK_SCAN_ANGLES DISK_SCAN_ANGLES_TITLE DISK_SCAN_ANGLES_UNITS | FLOAT STRING STRING | [3,132] - - | Disk cross track angle for each mirror step in a scan. “Cross track angle for each mirror step in a disk scan” “Degrees” |
| LIMB_SCAN_ANGLES LIMB_SCAN_ANGLES_TITLE LIMB_SCAN_ANGLES_UNITS | FLOAT STRING STRING | [3,24] - - | Limb cross track angle for each mirror step in a scan. “Cross track angle for each mirror step in a limb scan” “Degrees” |
| DISK_PIXEL_ANGLES DISK_PIXEL_ANGLES_TITLE DISK_PIXEL_ANGLES_UNITS | FLOAT STRING STRING | [2,16] - - | Angular offset of each along-track (along-slit) disk pixel. “Along track pixel angle (disk)” “Degrees” |
| LIMB_PIXEL_ANGLES LIMB_PIXEL_ANGLES_TITLE LIMB_PIXEL_ANGLES_UNITS | FLOAT STRING STRING | [2,8] - - | Angular offset of each along-track (along-slit) limb pixel. “Along track pixel angle (limb)” “Degrees” |
| DISK_PIXEL_OFFSETS DISK_PIXEL_OFFSETS_TITLE DISK_PIXEL_OFFSETS_UNITS | FLOAT STRING STRING | [3,16] - - | Corrections to disk scan angles for each along-track (along-slit) disk pixel. “Along track pixel scan angle correction (disk)” “Degrees” |
| LIMB_PIXEL_OFFSETS LIMB_PIXEL_OFFSETS_TITLE LIMB_PIXEL_OFFSETS_UNITS | FLOAT STRING STRING | [3,8] - - | Corrections to disk scan angles for each along-track (along-slit) limb pixel. “Along track pixel scan angle correction (limb)” “Degrees” |
| DISK_LOOK_VECTOR_ECI DISK_LOOK_VECTOR_ECI_COMMENT | FLOAT STRING | [N,132,16,3] - | Disk look vectors in the ECI frame. “Unit look vector for the disk from s/c in ECI frame” |
| LIMB_LOOK_VECTOR_ECI LIMB_LOOK_VECTOR_ECI_COMMENT | FLOAT STRING | [N,24,8,3] - | Limb look vectors in the ECI frame. “Unit look vector from s/c to the limb in ECI frame” |
| PIERCEPOINT_DAY_ALTITUDE PIERCEPOINT_DAY_ALTITUDE_COMMENT | FLOAT STRING | - - | 150 km reference altitude “Dayside reference altitude for all pierce point location calculations” |
| PIERCEPOINT_DAY_LATITUDE PIERCEPOINT_DAY_LATITUDE_TITLE PIERCEPOINT_DAY_LATITUDE_UNITS | FLOAT STRING STRING | [N,132,16] - - | “Latitude of Pierce Point” “Geographic coordinates, degrees” |
| PIERCEPOINT_DAY_LONGITUDE PIERCEPOINT_DAY_LONGITUDE_TITLE PIERCEPOINT_DAY_LONGITUDE_UNITS | FLOAT STRING STRING | [N,132,16] - - | “Longitude of Pierce Point” “Geographic coordinates, degrees” |
| PIERCEPOINT_NIGHT_ALTITUDE PIERCEPOINT_NIGHT_ALTITUDE _COMMENT | FLOAT STRING | - - | 350 km reference altitude “Nightside reference altitude for all pierce point location calculations” |
| PIERCEPOINT_NIGHT_LATITUDE PIERCEPOINT_NIGHT_LATITUDE_TITLE PIERCEPOINT_NIGHT_LATITUDE_UNITS | FLOAT STRING STRING | [N,132,16] - - | “Latitude of Pierce Point” “Geographic coordinates, degrees” |
| PIERCEPOINT_NIGHT_LONGITUDE PIERCEPOINT_NIGHT_LONGITUDE _TITLE PIERCEPOINT_NIGHT_LONGITUDE _UNITS | FLOAT STRING STRING | [N,132,16] - - | “Longitude of Pierce Point” “Geographic coordinates, degrees” |
| TANGENTPOINT_LATITUDE TANGENTPOINT_LATITUDE_TITLE TANGENTPOINT_LATITUDE_UNITS | FLOAT STRING STRING | [N,24,8] - - | “Latitude of Tangent Point” “Geographic coordinates, degrees” |
| TANGENTPOINT_LONGITUDE TANGENTPOINT_LONGITUDE_TITLE TANGENTPOINT_LONGITUDE_UNITS | FLOAT STRING STRING | [N,24,8] - - | “Longitude of Tangent Point” “Geographic coordinates, degrees” |
| TANGENTPOINT_ALTITUDE TANGENTPOINT_ALTITUDE_TITLE TANGENTPOINT_ALTITUDE_UNITS | FLOAT STRING STRING | [N,24,8] - - | “Altitude of the true tangent point” “Kilometers” |
| RA RA_TITLE RA_UNITS | FLOAT STRING STRING | [N,24,8] - - | “Right Ascension” “Decimal degrees” |
| DEC DEC_TITLE DEC_UNITS | FLOAT STRING STRING | [N,24,8] - - | “Declination” “Decimal degrees (90 to –90)” |
| STAR STAR_TITLE | SHORT STRING | [N] | Scans that may be contaminated with stellar emissions. “Star passage likely in limb scan” |
| STAR_LOCATION STAR_LOCATION_TITLE | SHORT STRING | [N,24,8] - | Pixels that may be contaminated with stellar emission. “Probable star locations in limb scan” |
| DISK_SOLAR_ZENITH_ANGLE DISK_SOLAR_ZENITH_ANGLE.TITLE DISK_SOLAR_ZENITH_ANGLE.UNITS | FLOAT STRING STRING | [N,132,16] - - | Angle from disk pixel zenith to sun “Disk Solar Zenith Angle” “degrees” |
| LIMB_SOLAR_ZENITH_ANGLE LIMB_SOLAR_ZENITH_ANGLE.TITLE LIMB_SOLAR_ZENITH_ANGLE.UNITS | FLOAT STRING STRING | [N,24,8] - - | Angle from limb pixel zenith to sun “Limb Solar Zenith Angle” “degrees” |
| Photometer Data |  |  |  |
| LAT_PHOTOMETER | FLOAT | [N,22] | Latitude of each photometer radiance |
| LON_PHOTIMETER | FLOAT | [N,22] | Longitude of each photometer radiance |
| ALT_PHOTOMETER | FLOAT | [N,22] | Altitude of the photometer for each radiance |
| PHOTOMETER_DMSP_TIME_OFFSET PHOTOMETER_DMSP_TIME_OFFSET TITLE PHOTOMETER_DMSP_TIME_OFFSET_UNITS | FLOAT STRING STRING | - - - | Scan start time offset from DMSP_COORDS_TIME " Offset between ephemeris time and photometer scan start" “Seconds” |
| PHOTOMETER427_RADIANCE | FLOAT | [N,22] | Calibrated Radiances from the 427 nm photometer in Rayleighs |
| PHOTOMETER629_RADIANCE | FLOAT | [N,22] | Calibrated Radiances from the 629 nm photometer in Rayleighs |
| PHOTOMETER630_RADIANCE | FLOAT | [N,22] | Calibrated Radiances from the 630 nm photometer in Rayleighs |
| PHOTOMETER427_VARIANCE | FLOAT | [N,22] | Variances of the 427 nm photometer radiances in Rayleighs**2 |
| PHOTOMETER629_VARIANCE | FLOAT | [N,22] | Variances of the 629 nm photometer radiances in Rayleighs**2 |
| PHOTOMETER630_VARIANCE | FLOAT | [N,22] | Variances of the 630 nm photometer radiances in Rayleighs**2 |