| variable name | description | units | resolu-tion | range | plot range | fill data | notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| date | year and day of year | – | 1 sec | – | – | n/a | year and day of year in yyyyddd format |
| time | time of day since midnight UT | seconds | 1 sec | 0 - 86399 | 0 - 86399 | n/a | time of the data in seconds of the day |
| mlat | magnetic latitude | degrees | 1 sec | -90. – +90. | -90. – +90. | n/a | magnetic latitude of field line at s/c mapped to 120 km |
| mlt | magnetic local time | hour and decimal hour | 1 sec | 0.0 – 23.99 | 0.0 – 23.99 | n/a | magnetic longitude of field line at s/c mapped to 120 km |
| glat | geographical latitude | degrees | 1 sec | -90. – +90. | -90. – +90. | n/a | geographic latitude of s/c |
| glon | geographic east longitude | degrees | 1 sec | 0.00 – 359.99 | 0.00 – 359.99 | n/a | geographic east longitude of s/c |
| sza | solar zenith angle | degree | 1 sec | 0.0 – 180.0 | 0.0 – 180.0 | n/a | angle between the sun-earth line and the s/c-center of earth line |
| alt | altitude | km | 1 sec | 820. – 870. | 820. – 870. | n/a | altitude of the s/c above earth’s surface |
| vx | median filtered and smoothed ion flow in the ram direction | m/s | 1 sec | -2000. – +2000. | -2000. – +2000. | -999999. | median filtered and smoothed ion flow in the ram direction from the fits to the RPA curve |
| vxqual | quality flag for Vx data | numerical flags | 1 sec | 1 – 5 | 1 – 5 | n/a | 1-good 2-fair 3-caution 4-bad 5-uncertain |
| vxrms | root mean square of the RPA fit | unitless | 1 sec | 0-999 | 0-99 | -999999. | root mean square of the fit to the RPA curve giving an indication of the quality of the RPA data for this sweep |
| corrpaqual | overall quality flag for the RPA data | numerical flags | 1 sec | 1 – 5 | 1 – 5 | n/a | 1-good 2-fair 3-caution 4-bad 5-uncertain |
| rpainfo | flag for RPA RPAsweep direction | – | 1 sec | 1 – 2 | 1 – 2 | -999999. | 1-retarding potential increasing 2- retarding potential decreasing |
| vy | ion flow in the horizontal crosstrack direction | m/s | 1 or 2 s depending on mode | -2800. – +2800. | -2000. – +2000. | -999999. | crosstrack horizontal ion flow from IDM, positive is in the sunward direction regardless of the orientation of the orbit, data taken in two modes: normal and slow (see nmbpts) |
| vyqual | quality flag for Vy data | numerical flags | 1 or 2 s depending on mode | 1 – 9 | 1 – 9 | n/a | 1-good 2-fair 3-caution 4-bad 5-uncertain 6-good (F17) 7-fair (F17) 8-caution (F17) 9-caution (sun glint or high-energy particles) |
| vyrms | standard deviation of the Vy flow | m/s | 1 or 2 s depending on mode | 0. – +2800. | 0. – +2800. | -999999. | standard deviation of the six Vy samples taken per second in the normal mode, fill data in slow mode |
| vz | ion flow in the vertical crosstrack direction | m/s | 1 or 2 s depending on mode | -2800. – +2800. | -2000. – +2000. | -999999. | crosstrack vertical ion flow from IDM, positive is in the upward direction, data taken in two modes: normal and slow (see nmbpts) |
| vzqual | quality flag for Vz data | numerical flags | 1 or 2 s depending on mode | 1 – 9 | 1 – 9 | n/a | 1-good 2-fair 3-caution 4-bad 5-uncertain 6-good (F17) 7-fair (F17) 8-caution (F17) 9-caution (sun glint or high-energy particles) |
| vzrms | standard deviation of the Vz flow | m/s | 1 or 2 s depending on mode | 0. – +2800. | 0. – +2800. | -999999. | standard deviation of the six Vz samples taken per second in the normal mode, fill data in slow mode |
| nmbpt | number of data points in the Vy – Vz averages | – | 1 sec | 1 or 6 | 1 or 6 | -999999. | the IDM operates in two modes: normal mode takes six horizontal and six vertical samples per second, slow mode is for low density conditions where only one sample is taken each second and alternates between directions |
| idmqual | overall quality flag for the IDM data | numerical flags | 1 or 2 s depending on mode | 1 – 9 | 1 – 9 | n/a | 1-good 2-fair 3-caution 4-bad 5-uncertain 6-good (F17) 7-fair (F17) 8-caution (F17) 9-caution (sun glint or high-energy particles) |
| scvel | spacecraft velocity | m/s | 1 sec | ~ 7400 – 7500 | ~7400 – 7500 | -999999. | orbital velocity of the spacecraft |
| temp | ion temperature | deg K | 1 sec | 500. – 20000. | 500. – 5000. | -999999. | temperature of the ions in the plasma calculated from the fit to the RPA curve |
| tempqual | Quality flag for the ion temperature | numerical flags | 1 sec | 1 – 5 | 1 – 5 | n/a | 1-good 2-fair 3-caution 4-bad 5-uncertain |
| pot | Potential of the SSIES instruments relative to the plasma ground | volts | 1 sec | -3. – +2. | -2. – +1. | -999999. | all the SSIES instruments are held near the plasma ground potential, nominally about -1 V relative to the plasma, this value is calculated from the RPA curve fit, shown as “SC Potential” on the plots |
| dens | ion density calculated from the RPA | Ions/cc | 1 sec | 0 – 107 | 200 – 107 | -999999. | Ion density of the plasma calculated from the fit to the RPA curve |
| densqual | quality flag for the RPA calculated ion density | numerical flags | 1 sec | 1 – 5 | 1 – 5 | n/a | 1-good 2-fair 3-caution 4-bad 5-uncertain |
| frach | fractional amount of H+ ions in plasma | – | 1 sec | -0.1 – 1.05 | 0.0 – 1.05 | -999999. | fractional amount of the plasma that is H+ based on the fitting of the RPA curve, because of uncertainties the value can exceed 1.0 |
| frachqual | Quality flag for fractional H+ | numerical flags | 1 sec | 1 – 5 | 1 – 5 | n/a | 1-good 2-fair 3-caution 4-bad 5-uncertain |
| frache | fractional amount of He+ ions in plasma | – | 1 sec | -0.1 – 1.05 | 0.0 – 1.05 | -999999. | fractional amount of the plasma that is He+ based on the fitting of the RPA curve, because of uncertainties the value can exceed 1.0 |
| frachequal | Quality flag for fractional He+ | numerical flags | 1 sec | 1 – 5 | 1 – 5 | n/a | 1-good 2-fair 3-caution 4-bad 5-uncertain |
| fraco | fractional amount of O+ ions in plasma | – | 1 sec | -0.1 – 1.05 | 0.0 – 1.05 | -999999. | fractional amount of the plasma that is O+ based on the fitting of the RPA curve, because of uncertainties the value can exceed 1.0 |
| fracoqual | Quality flag for fractional O+ | numerical flags | 1 sec | 1 – 5 | 1 – 5 | n/a | 1-good 2-fair 3-caution 4-bad 5-uncertain |
| bx | north component of Earth’s magnetic field (IGRF) | nT | 1 sec | -50000 – +50000 | -50000 – +50000 | -999999. | northward component of the Earth’s magnetic field at the spacecraft’s location calculated by the IGRF model |
| by | east component of Earth’s magnetic field (IGRF) | nT | 1 sec | -50000 – +50000 | -50000 – +50000 | -999999. | eastward component of the Earth’s magnetic field at the spacecraft’s location calculated by the IGRF model |
| bz | vertical downward component of Earth’s magnetic field (IGRF) | nT | 1 sec | -50000 – +50000 | -50000 – +50000 | -999999. | vertical downward component of the Earth’s magnetic field at the spacecraft’s location calculated by the IGRF model |
| ductdens | ion density from the scintillation meter | ions/cc | 1 sec | 0 – 107 | 200 – 107 | -999999. | the average of the 24 Hz samples of the ion density by the scintillation meter, because of its large aperture this is considered the best meaure of the ion density |
| dmdens | Ion density from the IDM | ions/cc | 1 sec | 0 – 107 | 200 – 107 | -999999. | the ion density based on the collector current in the IDM, because some or all of the light ions are excluded this density should always be equal to or less than the other ion densities |
| te | electron temperature | deg K | 4 sec | 500 – 10000 | 500 – 10000 | -999999. | the electron temperature based on an onboard calculation of the Langmuir probe sweep which occurs once every 4 s |
| rpaground | potential difference between SSIES instruments and spacecraft ground | Volts | 1 sec | -25 – +25 | -25 – +25 | -999999. | the DMSP spacecraft charges negative relative to the plasma ground so SSIES is electrically insolated and driven positive relative to spacecraft ground to keep the instrument near the plasma ground |
| dmhrough | roughness parameter of Vy | – | 1 sec | 0 – 1 | 0 – 1 | -999999. | roughness parameter of Vy defined as (delta Vy / averaged Vy) |
| dmvrough | roughness parameter of Vz (delta Vz / averaged Vz) | – | 1 sec | 0 – 1 | 0 – 1 | -999999. | roughness parameter of Vz defined as (delta Vz / averaged Vz) |
| ductdensrough | roughness parameter of SM density | – | 1 sec | 0 – 1 | 0 – 1 | -999999. | roughness parameter of scintillation meter ion density defined as (delta Ni/ averaged Ni) |
| corvelx | corotation velocity of ionosphere in the ram direction | m/s | 1 sec | -100 – 0 | -100 – 0 | -999999. | the ram component (x) of the corotation speed of the ionosphere at the spacecraft’s location |
| corvely | corotation velocity of ionosphere in the horizontal crosstrack direction | m/s | 1 sec | -600 – +600 | -600 – +600 | -999999. | the crosstrack horizontal component (y) of the corotation speed of the ionosphere at the spacecraft’s location |
| corvelz | corotation velocity of ionosphere in the vertical direction | m/s | 1 sec | 0 | 0 | -999999. | the crosstrack vertical component (z) of the corotation speed of the ionosphere at the spacecraft’s location, since DMSP is in a circular orbit, this value is zero |
| vxraw | original unfiltered Vx flow | m/s | 1 sec | -2000 – +2000 | -2000 – +2000 | -999999. | the unfiltered Vx flow values calculated by the RPA fitting routine |
| dens0v | ion density measured from the RPA at 0 volts | ions/cc | 1 sec | 0 – 107 | 0 – 107 | -999999. | during each RPA sweep there is a point where the repeller voltage is 0 V so all the ions reach the collector, this is the ion density based on that measurement |
| ebm | high energy particles currents detected by the RPA | Amps | 2 sec | -2 x 10-9 – +2 x 10-9 | -2 x 10-9 – +2 x 10-9 | -999999. | once every 2 s the RPA repeller voltage is set so high that only high energy electrons or ions reach the collector. A large current indicates high-energy particles or photo-electron production (a sun-glint). |