Readme file for the DAILI Data Sets

James Hecht

Data Archive for the NASA DAILI CubeSat Mission:

NASA Award Number: 80NSSC18K0973 (2023)

1. Introduction

The DAILI (Daily Atmospheric and Ionospheric Limb Imager) mission utilized a 6U CubeSat, to measure the airglow on the Earth’s limb day and night from about 140 to 300 km in two colors; the OI (630 nm) redline and part of the O2A (0,0) band near 760 nm. The ratio of these colors, obtained when the atmosphere is sunlit, can provide absolute O2 density. The theoretical basis for this is found in a GRL paper that describes this technique and that uses data from the NRL/Aerospace RAIDS experiment that was on the ISS (International Space Station) in 2009/2010 (Hecht et al., 2015).  DAILI was designed to obtain the same measurements but from a CubeSat. DAILI was launched to the ISS on December 2021 and was deployed from the ISS into an ISS-like orbit at 410 km on 01/26/22. DAIL’s reentered in late June 2022.

The data in these files (either IDL save sets or NCDF files) are from this mission and the purpose of this readme file is to (a) provide an overview of the DAILI mission to provide some context to the reader, (b) provide some examples of the DAILI data, (c) describe the analysis procedure (and provide some examples), and (d) list the array names of that included in the saved datasets. We note that an overview of the DAILI mission was presented at the fall 2022 AGU meeting in Chicago and the slides below are from that presentation.

For this archive the data are in ncdf files. (The IDL save set archive is saved at zenodo-https://doi.org/10.5281/zenodo.8122347 ). All, except for March 3. Include the raw and processed daytime data as described below. On March 3 no processed data are available at this time (July 6 2023). The raw data however allow for future processing.

2. Overview of the DAILI Mission

Figure 1 provides an overview of the experiment.

Figure 1. An overview of the DAILI CubeSat Mission.

Figure 2 provides some background as well as a schematic of DAILI viewing the limb.

Figure 2. Background for the DAILI CubeSat mission.

Figure 3 has 5 images. The top image presents the optical layout showing the deployed sunshade, the first fold mirror (that is super-polished to reduce scattering) and the two beam splitters that direct the two colors onto two separate portions of the E2V CCD. Not shown is the shutter (a clear glass element at the end of the sunshade and before the first mirror) that kept the instrument clean until it was deployed a few days after DAILI was ejected from the ISS. In the left column are two images of DAILI;(top) on the bench before delivery) and (bottom) looking down the throat of the sunshade. DAILI was painted with a special Nano black paint that reduced scattering. In the right column are two images taken from the initial testing in the laboratory: (Top) A calibration chart imaged  onto the full (1024 x 256) CCD. with the signal through the 765 (630) nm channel shown on the left (right).  (Bottom) A line plot slice through the top image that was used to spatially calibrate the image. As noted on the bottom of the chart on-orbit images were binned down to 256-horizontal by 16-vertical pixels.

Figure 4 shows DAIL being I deployed from the ISS on 01/26/22.

Figure 3. Some images of the DAILI hardware, an optical layout, and some first light da

Figure 4. The Deployment of DAILI from the ISS.

Before fully discussing Figure 5 we describe the general storage of the data. On March 18,2022 DAILI obtained 867 images. There is an array in the ncdf and IDL save set files called raw_image that has 256 by 16 by 867 elements. From Figure 4 the on-orbit binned CCD has 256 horizontal h-pixels and 16 vertical v-pixels. Thus, there is an element in the array for each of the 867 images that have 256 CCD h-pixels that are in the altitude direction, and 16 CCD v-pixels that are in the cross-track direction. The plot on the left of Figure 5 shows a slice at v- pixel 8 (where the first pixel is 0) through all of the 256 h-pixels. This slice is for the 81st image (row number 80 in this column of the array where   row 0 is the first image). As noted, the region between the baffle vanes is between h-pixels 9 and 58, for the 765 nm channel, and (189 pixels to the right) between h-pixels 198 to 247, for the 630 nm channel. To facilitate analysis (and because none of the pointing data are useful for the interior pixels) almost all the remaining arrays listed in Appendix 4 have 59 x 16 x n   elements where n is the number of images on a selected = day.  The 59   rows of the first column are the altitude direction h-pixels and the 16 rows of the second column are the cross-track direction v-pixels. Each of the two data arrays are labeled as either 765 or 630, referring to their respective colors. Since they each have identical altitudes there are 59 altitude rows In the tangent point arrays that apply to both channels.  All of the provided arrays are listed in Appendix 4.

Figure 5. Examples of on-orbit DAILI data

Now we describe Figure 5 that presents some on-orbit data. The plot on the left therefore is a plot of raw data (0:255,8,80) for March 18, 2022. The value at pixel 150 is 4167. The figure is annotated, and it is noted that region between the vane is from pixels 9 to 58. There is an array called tpaltitude that has, for day March 18, 59 x 16 x 867 elements and that gives the tangent point altitude. Here element (9,8,80) is at 136.54 km and element (58,8,80) is at 308.49 km.  During the day there is considerable 765 nm signal and even though the vane edge is at pixel 8 the optics are such that the signal still increases until the illumination on the first fold mirror decreases enough to significantly attenuate the signal.  Thus, there is still signal between pixels 0 and 8 in altitude direction. There are also some well-contained glints at the edge of each vane.  The tangent point solar zenith angle is given in the array tpsolarzen   and for element (9,8,80) its value is 60.4. The UT time array, time, only has 867 elements and its value for element 80 in seconds is 11335 or 3.1 UT in hours.

The plot on the right is image 80 on March 13 but at twilight where the solar zenith angle is less than 90 degrees. The 765 signal is greatly attenuated although the 630 nm layer is still illuminated by the sun.

3. Calibration of DAILI Data

To proceed further one needs the instrument calibration. The calibration to determine the responsivity in (counts/s)/Rayleigh was done in two ways; on the ground before delivery and on orbit by imaging the star SIRIUS.

- Star Calibration to determine responsivity of the DAILI sensor

- In the 765(630) channel 741(690) Analog to Digital Units (ADUs) were measured in a 5 second exposure when the CCD was imaging in the full 1024 x 256 mode. There was an offset of about one binned pixel in the vertical mode.

- Sirius flux (w/cm2-s-micron)

630 nm  -  9.28e-12. = 3.11e7 photons/s/cm2-micron

765 nm  -  5.01e-12. = 1.93e7 photons/s/cm2-micron

- Adjustment for Filter FWHM at 0-degree incidence

765 nm  ~(0.001584) microns FWHM. Thus, (1.93e7)*(1.584e-3 microns)= 30571.2 ph/s-cm2

630 nm  ~(0.001636) microns FWHM. Thus, (3.11e7)*(1.636e-3 microns)= 50879.6 ph/s-cm2

- Adjustment for FOV per pixel (in microradians)

765 nm FOV is 577.78 microradians

30571.2/(577.78e-6)^2=9.15772e10 ph/s-cm2-sr

630 nm  FOV is 579.07 microradians

50879.6/(579.07e-6)^2=1.5173e11 ph/s-cm2-sr

- Convert to Rayleighs

765 nm.9.15772e10/(1.e6/4pi)= 1.15079e6 R

630 nm 1.55508e11/(1.e6/4pi)= 1.90674e6 R

- Responsivity

a. 765 nm channel

1.150793e6/(741.ADU/5.sec) or 7765.13 R/ADU per s per pixel

0.00012878071 ADU/R for 1 s integration and one pixel

For a 5 s 64-binned pixel 0.041209828 ADU/R or

24.27 R/ADU

b. 630 nm channel

1.9067398e6/(690./5s)=13816.96 R/ADU per s per pixel

or 7.2374845e-5 ADU/R for 1 s integration and one pixel

For a 5s 64-binned pixel 0.023159951ADU/R or

43.18 R/ADU

- Ratio of sensitivity between 765 and 630 channel (in units R/ADU) is 1.78.  When comparing the inverse (R/ADU) the ratio is 0.56199

- Ground Calibration

This was done through the full DAILI f/5 optics using a NIST calibrated extended source.  Measurements indicated that at f/5 that the 630(765) nm filters broadened to FWHM values of 1.727 and 1.659 nm respectively.

- ADUs (Analog to Digital Units) measured from the source in the lab.

765(630) 2420(695) ADUs although it is estimated that atmospheric O2 absorption would reduce the measured ADU value at 765 nm by 1%.

- For a 5s exposure and 64-binned pixels the result in R/ADU was 23.195(43.77) R/ADU values a few percent higher than the star cal.

- Ratio of 765 to 630 is 0.5756 and that is 1.024 time larger implying the lab calibration would infer ~2.5% more O2

- Summary

We will use the star calibration in the analysis presented here but note the possibility of a few percent error in the derived O2.

4. Fraction of the O2A band in the bandpass.

Following the references and discussion in Hecht et al. (2015) we estimated the fraction of the band as function of temperature and for a nominal MSIS T profile converted that to altitude. These numbers are provided in the Appendix. Following Hecht et al. (2015) we note that there is a decrease at 150 km (in the band fraction) of about 2.5% between a very quiet and disturbed day. Thus, future archives could take this into account

5. Band Contamination

Hecht et al. (2015) also note that there can be some out of band contamination. Specifically, there is some N2 First positive (3,1) band contamination of the 765 nm channel and some N2+ Meinel (5,1) band contamination of the 630 nm channel. The latter is less than 1% at 150 km and the fraction decreases at higher altitudes. For the 765 channel the contamination is between 2 and 3 %. The 765 to 630 nm ratio therefore should be reduced bay 1 to 2 %. But that is not done in this version of the archive.

6. Abel Inversion

To continue with the analysis even one needs to convert the altitude dependent limb viewing measurements to altitude dependent volume emission rates. Following Hecht et al. (2015) we use an ABEL transform with a software package developed for that RAIDS data analysis.  Wikipedia gives a good overview (https://en.wikipedia.org/wiki/Abel_transform) and the forward transform (eqns 1 and 1a) and inverse transform (eqn 2) are given above. Here F(y) is the integrated signal per area seen at a given tangent height y, above the center of the Earth, and f(r) is the volume emission rate. If F is in Rayleighs then the value -10f(r) is in photons/cm3/s. There are several issues with recovering f(r) from measurements of F. (1) There is a singularity when y is equal to r. (2) A derivative is involved. (3) The results at r depend on data above r. To avoid the first issue, we used a software routine (written in IDL by Dr. Paul Straus and given in the appendix) that evaluates the integral but avoids the singularity. That algorithm was used for the RAIDS project and for the study cited earlier (Hecht et al., 20105).

We also looked at other ways of evaluating eqn 2 by direct integration. This is discussed for example on the pyABEL website (https://pyabel.readthedocs.io/en/latest/readme_link.html) and in the associated paper (Hickstein et al., 2019).The discussion on the direct integration method suggests integrating the first cell that contains the singularity analytically

(see https://pyabel.readthedocs.io/en/latest/_modules/abel/direct.html#direct_transform)

and then integrating the rest by any preferred integration scheme. We also divided the first cell into 500 equally spaced areas (excluding the one with the singularity and integrated that area.

Figure 6. Plots to validate the Abel Inversion Algorithms (see text)

To check the validity of all these approaches we applied the inverse transform (eqn 2) for sample measured 765 and 630 profiles and used the forward transform (eqn 1/1a) on that result to reconstruct the original limb view data.  The plots in Figure 6 show the results for the sample 765 and 630 nm limb profiles (black) compared to the reconstructed limb profiles (red and blue) as well as the intermediate derived volume emission rates. For the latter we show for the region of interest (135 to 300 km). As can be seen the different methods are comparable and all reproduce the original profile. When looked at in detail the difference in the reconstructed 765 (630) profiles differ by less than 2 (1) percent or less. The 765 (630) VERs derived from the IDL code are smaller (greater) than the pyABEL method by less than ~4(~1) percent. The mean difference in the ratio is 3% with the IDL result being less than that amount.Finally, we note that to convert the above volume emission rates into units of cts/cm3/s we need to multiply by the calibrations given earlier.

7. Appendices

There are four Appendices. Note that in Appendix 4 the arrays in the ncdf and IDL saveset files are described. Many of these arrays are processed data. But note that the raw data are found for each image in the array raw_image. This array has 256 x 16 x n elements where n is the number of images in the file.  Also, for convenience the two colors are extracted from the raw image and given in two arrays, cntimage_630 or cntimage_765. Each of these arrays have 59 by 16 b  n elements and there are associated altitude, latitude, and longitude arrays that provide the tangent point of each observation.

Appendix 1

Appendix 1 provides the values for the plot on the left-hand side of slide 5. This found in the array, as noted in the discussion above, called raw_image that has (256,16,867) elements as these data are from raw_image (0:255,8,81) for the March 18, 2022, data found in the IDL save set or in the ncdf file.

4271.00      4866.00      5650.00      5991.00      6209.00      6201.00      5983.00      5676.00      5247.00      4988.00      4856.00      4759.00      4618.00      4545.00

4495.00      4445.00      4385.00      4392.00      4339.00      4342.00      4316.00      4302.00      4299.00      4253.00      4224.00      4242.00      4227.00      4179.00

4197.00      4262.00      4219.00      4204.00      4207.00      4201.00      4174.00      4183.00      4218.00      4161.00      4162.00      4198.00      4148.00      4160.00

4200.00      4198.00      4173.00      4206.00      4170.00      4174.00      4173.00      4186.00      4174.00      4165.00      4179.00      4135.00      4163.00      4168.00

4169.00      4195.00      4187.00      4252.00      4404.00      4543.00      4650.00      4703.00      4740.00      4765.00      4756.00      4767.00      4685.00      4597.00

4607.00      4468.00      4369.00      4336.00      4257.00      4183.00      4184.00      4186.00      4149.00      4158.00      4155.00      4152.00      4182.00      4121.00

4162.00      4127.00      4165.00      4195.00      4173.00      4163.00      4128.00      4147.00      4169.00      4142.00      4150.00      4174.00      4149.00      4154.00

4147.00      4163.00      4157.00      4143.00      4142.00      4145.00      4136.00      4205.00      4115.00      4154.00      4115.00      4148.00      4125.00      4180.00

4145.00      4154.00      4172.00      4172.00      4255.00      4131.00      4179.00      4141.00      4123.00      4150.00      4144.00      4148.00      4154.00      4160.00

4135.00      4148.00      4154.00      4153.00      4139.00      4145.00      4148.00      4173.00      4128.00      4151.00      4139.00      4152.00      4177.00      4141.00

4163.00      4149.00      4121.00      4158.00      4127.00      4161.00      4141.00      4150.00      4125.00      4148.00      4167.00      4171.00      4186.00      4160.00

4163.00      4163.00      4139.00      4135.00      4152.00      4164.00      4187.00      4159.00      4177.00      4148.00      4191.00      4187.00      4173.00      4182.00

4190.00      4174.00      4194.00      4129.00      4250.00      4177.00      4180.00      4238.00      4281.00      4218.00      4219.00      4253.00      4313.00      4223.00

4279.00      4268.00      4268.00      4278.00      4300.00      4377.00      4379.00      4560.00      4804.00      5006.00      5203.00      5404.00      5426.00      5476.00

5630.00      5550.00      5464.00      5555.00      5520.00      5526.00      5557.00      5585.00      5642.00      5625.00      5626.00      5648.00      5658.00      5648.00

5636.00      5604.00      5621.00      5612.00      5520.00      5514.00      5474.00      5445.00      5377.00      5308.00      5274.00      5223.00      5181.00      5143.00

5057.00      5038.00      5031.00      4942.00      4930.00      4867.00      4854.00      4792.00      4800.00      4720.00      4665.00      4629.00      4621.00      4655.00

4580.00      4584.00      4546.00      4477.00      4500.00      4486.00      4495.00      4477.00      4461.00      4430.00      4414.00      4421.00      4449.00      4589.00

4631.00      4662.00      4733.00      4775.00

Appendix 2

This provides the band fraction of the O2A (0,0) band that passes through the DAILI 765 filter. This was calculated for a quite time MSIS atmosphere.

Altitude.    frac765

95.8503     0.163641

100.554     0.162330

105.225     0.169155

109.863     0.178460

114.468     0.182059

119.041     0.179035

123.581     0.173482

128.087     0.167759

132.561     0.162475

137.001     0.157711

141.408     0.153500

145.781     0.149809

150.121     0.146570

154.427     0.143809

158.699     0.141389

162.938     0.139254

167.142     0.137377

171.313     0.135723

175.449     0.134255

179.551     0.132967

183.619     0.131834

187.653     0.130824

191.652     0.129921

195.616     0.129113

199.546     0.128389

203.441     0.127751

207.302     0.127175

211.127     0.126653

214.918     0.126179

218.674     0.125760

222.395     0.125378

226.080     0.125031

229.731     0.124721

233.346     0.124440

236.926     0.124181

240.470     0.123944

243.979     0.123737

247.453     0.123545

250.891     0.123367

254.294     0.123209

257.661     0.123062

260.992     0.122924

264.288     0.122801

267.548     0.122691

270.772     0.122588

273.960     0.122491

277.112     0.122405

280.228     0.122327

283.309     0.122254

286.353     0.122181

289.362     0.122110

292.334     0.122040

295.270     0.121970

298.170     0.121902

301.034     0.121834

303.862     0.121767

306.653     0.121701

309.409     0.121636

312.128     0.121571

Temperature (K) profile for this atmosphere

ALTITUDE        90.0000  KM       TEMPERATURE         194.800

ALTITUDE        95.0000  KM       TEMPERATURE         178.400

ALTITUDE        100.000  KM       TEMPERATURE         172.800

ALTITUDE        105.000  KM       TEMPERATURE         190.200

ALTITUDE        110.000  KM       TEMPERATURE         231.100

ALTITUDE        115.000  KM       TEMPERATURE         291.500

ALTITUDE        120.000  KM       TEMPERATURE         364.800

ALTITUDE        125.000  KM       TEMPERATURE         426.200

ALTITUDE        130.000  KM       TEMPERATURE         476.800

ALTITUDE        135.000  KM       TEMPERATURE         521.700

ALTITUDE        140.000  KM       TEMPERATURE         561.600

ALTITUDE        145.000  KM       TEMPERATURE         597.000

ALTITUDE        150.000  KM       TEMPERATURE         628.600

ALTITUDE        155.000  KM       TEMPERATURE         656.500

ALTITUDE        160.000  KM       TEMPERATURE         681.400

ALTITUDE        165.000  KM       TEMPERATURE         703.600

ALTITUDE        170.000  KM       TEMPERATURE         723.200

ALTITUDE        175.000  KM       TEMPERATURE         740.700

ALTITUDE        180.000  KM       TEMPERATURE         756.300

ALTITUDE        185.000  KM       TEMPERATURE         770.200

ALTITUDE        190.000  KM       TEMPERATURE         782.500

ALTITUDE        195.000  KM       TEMPERATURE         793.400

ALTITUDE        200.000  KM       TEMPERATURE         803.200

ALTITUDE        205.000  KM       TEMPERATURE         811.900

ALTITUDE        210.000  KM       TEMPERATURE         819.600

ALTITUDE        215.000  KM       TEMPERATURE         826.500

ALTITUDE        220.000  KM       TEMPERATURE         832.700

ALTITUDE        225.000  KM       TEMPERATURE         838.200

ALTITUDE        230.000  KM       TEMPERATURE         843.000

ALTITUDE        235.000  KM       TEMPERATURE         847.400

ALTITUDE        240.000  KM       TEMPERATURE         851.300

ALTITUDE        245.000  KM       TEMPERATURE         854.700

ALTITUDE        250.000  KM       TEMPERATURE         857.800

ALTITUDE        255.000  KM       TEMPERATURE         860.500

ALTITUDE        260.000  KM       TEMPERATURE         863.000

ALTITUDE        265.000  KM       TEMPERATURE         865.200

ALTITUDE        270.000  KM       TEMPERATURE         867.100

ALTITUDE        275.000  KM       TEMPERATURE         868.900

ALTITUDE        280.000  KM       TEMPERATURE         870.400

Appendix 3

This appendix provides the IDL software code used in the analysis.

FUNCTION AbelInvert, r, I

;

; Calculates the Abel inverse transform of I(r).  Here r is the radius of the tangent

; altitude.

;

if (r(0) gt r(Num(r)-1)) then begin   ; Put data into ascending order

r2 = reverse(r)

I2 = reverse(I)

endif $

else begin

r2 = r

I2 = I

endelse

Ip = deriv(r2, I2)

Ans = fltarr(Num(r))

for j=0, Num(r2)-2 do begin

ztan2  = r2(j:*)

Ip2    = Ip(j:*)

AvgIp  = (Ip2(1:*) + Ip2)/2

Fact   = ztan2 + sqrt(ztan2^2 - ztan2(0)^2)

Ans(j) = total(AvgIp*alog(Fact(1:*)/Fact))

endfor

if (r(0) gt r(Num(r)-1)) then Ans = reverse(Ans)

return, -10*Ans/!pi

END

FUNCTION Num, array

;

; Same as n_elements, but shorter!

;

return, n_elements(array)

END

FUNCTION SumJLimb, j, z, zMax

;

; Calculates the integral of a volume emission rate (j(z)) along ;the line of sight in a

; limb-viewing geometry for the first zMax elements of z

;Assumes spherical symmetry, an optically thin emission, and that j is zero

; above the maximum z-value.  z (in km) must be in ascending ;order.  j is in

; photons/cc/sec.  Answer in rayleighs.

;

; The z/zMax convention is designed to allow for the possibility that the altitude of the

; observer is substantially above the altitude of strong ;emissions that are falling off

; expontially with altitude.  In such a case the interesting altitude range of the airglow

; intensities ends below the observer altitude. However, to ;avoid errors at the top end of

; the range of interest, j(z) must be integrated to higher ;altitudes (approximately 3

; scale heights above the maximum altitude of interest

;To do this, provide j(z) over this larger altitude range and ;set zMax to the

; index of the maximum altitude of interest.

;

COMMON SUMJLIMBMEMORY, Factors, Oldz, OldzMax

if (IsArray(where((z(1:*)-z) le 0))) then begin

print, "SumJLimb: ERROR - ordering of altitude array invalid!"

stop

endif

if (IsVoid(zMax)) then zMax = Num(z) - 1

Re = 6370.0			; Earth radius (km)

m  = Num(j)

CalcFactors = IsVoid(Factors)

if (~CalcFactors) then CalcFactors = (Num(Oldz) ne Num(z)) or (OldzMax ne zMax)

if (~CalcFactors) then CalcFactors = IsArray(where(Oldz ne z))

if ( CalcFactors) then begin

Factors = fltarr(m,zMax)

for ind=0, zMax-1 do Factors(ind:*,ind) = -2*sqrt((z(ind:*)+Re)^2 - (z(ind)+Re)^2)

Oldz    = z

OldzMax = zMax

endif

dj = deriv(z, j)

I  = fltarr(zMax)

for ind=0, zMax-1 do begin

kernal = Factors(ind:*,ind)*dj(ind:*)

I(ind) = Integrate(z(ind:*), kernal) + 2*j(m-1)*sqrt((z(m-1)+Re)^2 - (z(ind)+Re)^2)

endfor

return, I/10.0

END

FUNCTION Integrate, x, y

;

; Integral of a function (unevenly spaced data points ok, but x can only be

; increasing).

;

return, total((y(1:*) + y(0:num(y)-2))*(x(1:*) - x(0:num(y)-2)))/2.0

END

FUNCTION Deriv, x, y

;+

; NAME:

;	DERIV

;

; PURPOSE:

;	Perform numerical differentiation using 3-point, Lagrangian

;	interpolation.

;

; CATEGORY:

;	Numerical analysis.

;

; CALLING SEQUENCE:

;	Dy = Deriv(Y)	 	;Dy(i)/di, point spacing = 1.

;	Dy = Deriv(X, Y)	;Dy/Dx, unequal point spacing.

;

; INPUTS:

;	Y:  Variable to be differentiated.

;	X:  Variable to differentiate with respect to.  If omitted, unit

;	    spacing for Y (i.e., X(i) = i) is assumed.

;

; OPTIONAL INPUT PARAMETERS:

;	As above.

;

; OUTPUTS:

;	Returns the derivative.

;

; COMMON BLOCKS:

;	None.

;

; SIDE EFFECTS:

;	None.

;

; RESTRICTIONS:

;	None.

;

; PROCEDURE:

;	See Hildebrand, Introduction to Numerical Analysis, Mc Graw

;	Hill, 1956.  Page 82.

;

; MODIFICATION HISTORY:

;	Written, DMS, Aug, 1984.

;   Modified to handle unevenly spaced points correctly, PRS, Nov, 1999.

;-

;

on_error,2              ;Return to caller if an error occurs

n = Num(x)

if (n lt 3) then message, 'Parameters must have at least 3 points'

if (n_params(0) ge 2) then begin

if (n ne Num(y)) then message,'Vectors must have same size'

;	d = float(shift(y,-1) - shift(y,1))/(shift(x,-1) - shift(x,1))		;

;	d[0] = (-3.0*y[0] + 4.0*y[1] - y[2])/(x[2]-x[0])					; Old code

;	d[n-1] = (3.*y[n-1] - 4.*y[n-2] + y[n-3])/(x[n-1]-x[n-3])			;

x0 = shift(x,-1)

x2 = shift(x, 1)

y0 = shift(y,-1)

y2 = shift(y, 1)

d  = float(y0*(x-x2))/((x0-x )*(x0-x2)) + y*(1.0/(x-x0) + 1.0/(x-x2)) + $

float(y2*(x-x0))/((x2-x0)*(x2-x ))

d[ 0 ] = y[0]*(1.0/(x[0]-x[2]) + 1.0/(x[0]-x[1])) + $

float(y[1]*(x[0]-x[2]))/((x[1]-x[0])*(x[1]-x[2])) + $

float(y[2]*(x[0]-x[1]))/((x[2]-x[0])*(x[2]-x[1]))

d[n-1] = float(y[n-3]*(x[n-1]-x[n-2]))/((x[n-3]-x[n-2])*(x[n-3]-x[n-1])) + $

float(y[n-2]*(x[n-1]-x[n-3]))/((x[n-2]-x[n-3])*(x[n-2]-x[n-1])) + $

y[n-1]*(1.0/(x[n-1]-x[n-3]) + 1.0/(x[n-1]-x[n-2]))

end $

else begin

d = (shift(x,-1) - shift(x,1))/2.

d[ 0 ] = (-3.0*x[ 0 ] + 4.0*x[ 1 ] - x[ 2 ])/2.

d[n-1] = ( 3.0*x[n-1] - 4.0*x[n-2] + x[n-3])/2.

endelse

return, d

END

FUNCTION IsVoid, x

return, (n_elements(x) eq 0)

END

FUNCTION IsArray, x

return, ((size(x))(0) ge 1)

END

Appendix 4

This Appendix provides the titles for the data stored in idl save sets at zenodo.

Appendix 5 repeats most of these for the ncdf files

This provides an annotated list of the arrays contained in the save set or nc files. Note that the examples data as listed are from March 18, 2022 and where specified file image 80, where image 0 is the first image. The annotations in red help explain the data names.

TITLE           STRING    'DAILI_MAIN_L1_OUTPUTFILE'

MISSION         STRING    'DAILI'

DATA_PRODUCT_TYPE

STRING    'LEVEL 1'

DATA_SOURCE     STRING    'DAILI MOC'

SOFTWARE_VERSION

STRING    'DAILI_MAIN_VERSION =       4.10000'

INPUT_SOFTWARE_NAME

STRING    './LEVEL01A'

ORBIT_FILENAME  STRING    'L0_DAILI_Mar_18_2022_02-28-25_final_tag_2022-03-18_V9.sav'

DATE_GENERATED  STRING    'Thu Jan 26 18:30:09 2023'

DATA_START_TIME STRING    'Fri Mar 18 02:28:25 2022'

DATA_END_TIME   STRING    'Fri Mar 18 23:59:55 2022'

ORBIT_NUMBER    STRING    'TBD'

EXPOSURES_TOTAL LONG               867

INDICES_OF_PROCESSED_DAYSIDE_IMAGES;The first 5 are 57,58,59,60,61.

LONG      Array[259]

NUMBER_DAYTIME_IMAGES_PROCESSED

LONG               233

NUMBER_NIGHTSIDE_IMAGES

LONG               461

NUMBER_TWILIGHT_IMAGES

LONG               173

PROCESSED_IMAGES_THIS_FILE

STRING 'DAYSIDE PROCESSING FROM IMAGE NUMBER 68 TO 846'

INDICES_OF_DAYSIDE_IMAGES;The first five are 68,69,70,71,72

LONG Array[233]

INDICES_OF_NIGHTSIDE_IMAGES

LONG Array[461];The first five are 0,1,2,3,4

INDICES_OF_TWILIGHT_IMAGES

LONG Array[173];The first five are 45,46,47,48,49

INDICES_OF_GOOD_POINTING

LONG   867

FLAG          INT  Array[2, 867]

FLAG_INFO     STRING    'VALUE IN OTH COLUMN IS 0=DAYSIDE, 1= TWILIGHT, 2 = NIGHTSIDE.VALUE IN 1ST COLUMN IS POINTING WITH 0 = OK, 1 = QUESTIONABLE'. ;flag(*,80)= 0 0

MONTH         STRING Array[867]; month(80)=03

DAY           STRING Array[867]; day(80)=18

YEAR          STRING    '2022'

DATE          STRING Array[867];date(80)=2022077

DATE_UNITS    STRING    'YYYYDDD'

L0_INPUT_SOFTWARE_NAME

STRING    'L0_DAILI_Mar_18_2022_02-28-25_final_tag_2022-03-18_V8.sav'

L1_SOFTWARE_VERSION

LONG                 4

L1_SOFTWARE_NAME

STRING    'DAILI_MAIN.PRO'

MODE          INT       Array[867] ;Not Defined

TIME_UT (s)   DOUBLE    Array[867] ;time_ut(80) 11335.011 s

at beginning of collect

TIME_UT_MISSING_VALUE

FLOAT     -999.000

TIME_UTEND    DOUBLE    Array[867];time_utend(80)=11340.011

;end of collect

TIME_UTEND_MISSING_VALUE

FLOAT     -999.000

TPDN          INT       Array[867] ;tpdn(80)=0 TPDN identifies an image as daytime(0), twilight(1) or night(2)

DWELL           INT       Array[867];dwell(80)=5

DWELL_LONG_NAME STRING    'Seconds, accumulation period'

IMAGECOR_630    FLOAT     Array[59, 16,867];

imagecor_630(*,8,80)=1491.00 For the 630 data a constant value of 4000 is subtracted from the raw counts

IMAGECOR_630_LONG_NAME

STRING    'Superpixel counts (ADUs) per dwell period for each superpixel vertical (59) and horizontal (16) in the 6300 A channel'

RAY_IMAGECOR_630

FLOAT     Array[59, 16, 867]; ray_imagecor_630(9,8,80)=imagecor_630 times the cal (R/ADU)

RAY_IMAGECOR_630_LONG_NAME

STRING    'Fully corrected image in Rayleighs'-

RAY_IMAGECOR_630_MISSING_VALUE

STRING    '-999.000   '

IMAGECOR_765    FLOAT     Array[59, 16, 867]

IMAGECOR_765_LONG_NAME

STRING    'Superpixel counts per dwell period for each superpixel IN 765'

IMAGECOR_765_MISSING_VALUE

STRING    '-999.000'

RAY_IMAGECOR_765

FLOAT     Array[59, 16, 867]

RAY_IMAGECOR_765_LONG_NAME

STRING    'Image after dark count, scattered light correction and responsivity calibration, Imagecor_765 times cal(R/ADU)/bandfrac(given in Appendix)'

RAY_IMAGECOR_765_MISSING_VALUE

STRING    '-999.000'

AVG_630         FLOAT     Array[59, 4, 867] ;avg_630(9,2,80)=65210.2

AVG_630_LONG_NAME

STRING    'BRIGHTNESS IN RAYLEIGHS, AVERAGE OF 3 CONTIGUOUS ALTITUDE PROFILES'

;The 16 vertical profiles are divided into superpixels as follows:(0)is 3,4,5,(1)is 6,7,8,(2) is 9,10,11,(3)is 12,13,14.

AVG_630_MISSING_VALUE

STRING    '-999.000'

AVG_765        FLOAT     Array[59, 4, 867]

;avg_765(9,2,80)=127338.

AVG_765_LONG_NAME

STRING    'BRIGHTNESS IN RAYLEIGHS, AVERAGE OF 3 CONTIGUOUS PROFILES'

AVG_765_MISSING_VALUE

STRING    '-999.000'

FINVER_630     FLOAT     Array[59, 4, 867]

finver_630(9,2,80)=101.220

FINVER_630_LONG_NAME

STRING    'FINAL 630 VOLUME EMISSION RATE ALTITUDE PROFILE - CM-3S-1'

FINVER_630_MISSING_VALUE

STRING    '-999.000'

FINVER_765      FLOAT     Array[59, 4, 867]

finver_765(9,2,80)= 1344.90

FINVER_765_LONG_NAME

STRING    'FINAL 765 VOLUME EMISSION RATE ALTITUDE PROFILE - CM-3S-1'

FINVER_765_MISSING_VALUE

STRING    '-999.000'

RATIO           FLOAT     Array[59, 4, 867]

ratio(9,2,80)=13.2870

RATIO_LONG_NAME STRING    'Ratio of volume emission rates ALTITUDE PROFILE (FINVER_765/FINVER_630)'

RATIO_MISSING_VALUE

STRING    '-999.000'

O2D_RESULT      FLOAT     Array[59, 4, 867]

o2d_result(9,2,80)=7.30657e+09

O2D_RESULT_LONG_NAME

STRING    'Derived O2 density profile from RATIO'; and using the conversion following Hecht et al.,[2015] and the constants given below

O2D_RESULT_MISSING_VALUE

STRING    '-999.000'

O2D_AURIC       FLOAT     Array[100]

o2d_auric(44)=5.69500e+09 value from a quiet day auric model   O2D_AURIC_LONG_NAME

STRING    'AURIC MODEL O2 DENSITY '

AURIC_ALT_DEN   FLOAT     Array[100]

auric_alt_den(44)=137.710

AURIC_ALT_DEN_LONG_NAME

STRING    'ALTITUDE ARRAY (km) FOR O2D_AURIC'

TPALTITUDE      FLOAT     Array[59, 16, 867]

tpaltitude(9,8,80)=136.814

TPALTITUDE_LONG_NAME

STRING    'Tangent Point Altitude of LOS in km'

TPALTITUDE_MISSING_VALUE

STRING    '-999.000'

TPLATITUDE      FLOAT     Array[59, 16, 867]

tplatitude(9,8,80)=-50.0433

TPLATITUDE_LONG_NAME

STRING    'Tangent Point Latitude'

TPLATITUDE_MISSING_VALUE

FLOAT          -999.000

TPLONGITUDE     FLOAT     Array[59, 16, 867]

tplongitude(9,8,80)=3.1192

TPLONGITUDE_LONG_NAME

STRING    'Tangent Point longitude'

TPLONGITUDE_MISSING_VALUE

FLOAT          -999.000

TPSOLARZEN      FLOAT     Array[59, 16, 867]

tpsolarzen(9,8,80)=60.4724

TPSOLARZEN_LONG_NAME

STRING    'Tangent Point Solar zenith angle'

TPSOLARZEN_MISSING_VALUE

FLOAT          -999.000

TPSOLARLT       FLOAT     Array[59, 16, 867]

tpsolarlt(9,8,80)= 9.35656

TPSOLARLT_LONG_NAME

STRING    'Tangent Point Solar Local Time'

TPSOLARLT_MISSING_VALUE

FLOAT          -999.000

SCALTITUDE      DOUBLE    Array[867]

sclatitude(80)=-51.652682

SCALTITUDE_LONG_NAME

STRING    'Spacecraft altitude in km'

SCALTITUDE_MISSING_VALUE

FLOAT          -999.000

SCLONGITUDE    DOUBLE    Array[867]

sclongitude(80)=119.07874

SCLONGITUDE_LONG_NAME

STRING    'Spacecraft longitude in degrees'

SCLONGITUDE_MISSING_VALUE

FLOAT          -999.000

INSTRUMENT_STATUS

STRING    Array[867];Not Used in this version

CNTIMAGE_630   FLOAT     Array[59, 16, 867]

cntimage_630(9,8,80)=5491.00

CNTIMAGE_630_MISSING_VALUE

FLOAT          -999.000

CNTIMAGE_630_LONG_NAME

STRING    ' raw Super pixel cnt/dwell before any

dark subtraction'

CNTIMAGE_765   FLOAT     Array[59, 16, 867]

cntimage_765(9,8,80)=4988.00

CNTIMAGE_765_LONG_NAME

STRING    'raw Super Pixel cnt/dwell before any dark subtraction'

CNTIMAGE_765_MISSING_VALUE

FLOAT          -999.000

RAW_IMAGE      INT       Array[256, 16, 867]; see Appendix 1

RAW_IMAGE_LONG_NAME

STRING    'Full image cnt/dwell'

RAW_IMAGE_MISSING_VALUE

FLOAT.    -999.000

MOONSEPANGLE   DOUBLE    Array[867]

moonsepangle(80)=62.456960

MOONSEPANGLE_LONG_NAME

STRING    'Angles between the Moon and the LOS in degrees'

SUNLOSANGLE    DOUBLE    Array[867]

SUNLOSANGLE_LONG_NAME

STRING    'Angle between Sun and LOS in degrees'

SUNLOSANGLE_MISSING_VALUE

FLOAT     -999.000

; following are constants needed to turn the ratio into O2D see Hecht et al (2015).GRL paper and references therein

A630           FLOAT        0.00910000

A630_LONG_NAME STRING    'EINSTEIN COEFF FOR O1D EMISSION'

TEMP.          FLOAT           600.000

TEMP_LONG_NAME STRING    'TEMPERATURE USED IN MOLECULARA OXYGEN CALCULATION'

KAPPA          FLOAT       3.61680e-11

KAPPA_LONG_NAME STRING    'KAPPA VALUE O2 + O1D REACTION RATE'

BETA1           FLOAT          0.800000

BETA1_LONG_NAME STRING    'EFFICIENCY OF PRODUCTING SINGLET SIGMA STATE OF O2'

F0              FLOAT          0.600000

F0_LONG_NAME    STRING    ' EFFIENCIES OF POPULATING THE V=0 OR 1 STATE'

F1              FLOAT          0.400000

F1_LONG_NAME    STRING    'EFFIENCIES OF POPULATING THE V=0 OR 1 STATE'

AV              FLOAT         0.0919000

AV_LONG_NAME    STRING    'EINSTEIN COEFF FOR EMISSION OF UPPER V STATE'

A00             FLOAT         0.0876000

A00_LONG_NAME   STRING    'EINSTEIN COEFF FOR RADIATING IN 0-0 TRANSITION'

A11_LONG_NAME   STRING    'EINSTEIN COEFF FOR RADIATION IN 1-1 TRANSITION'

A11             FLOAT         0.0700000

APPENDIX 5

Header information for ncdf files

Appendix 4 provides a more extensive list than we saved for the ncdf files. Refer to that list for the definitions. Below we refer to March 13,2022 where there were 689 files. For March,18,2022 there are 867 files and examples of the data are in Appendix 4

MONTH           INT       Array[689]

DAY             INT       Array[689]

YEAR            INT       Array[689]

DATE            LONG      Array[689]

MODE            INT       Array[689]

TIME_UT         FLOAT     Array[689]

TIME_UT_END     FLOAT     Array[689]

DWELL           INT       Array[689]

CNTIMAGE_630    FLOAT     Array[59, 16, 689]

CNTIMAGE_765    FLOAT     Array[59, 16, 689]

RAW_IMAGE       FLOAT     Array[256, 16, 689]

; Next two are redundent as they are the same as raw image

; They were not in the IDL save set

DAILI_RAW_630   FLOAT     Array[256, 16, 689]

DAILI_RAW_765   FLOAT     Array[256, 16, 689]

;

INDICES_OF_DAYSIDE_PROCESSED_IMAGES

INT       Array[689]

INDICES_OF_NIGHTSIDE_IMAGES

INT       Array[689]

INDICES_OF_TWILIGHT_IMAGES

INT       Array[689]

INDICES_OF_GOOD_POINTING

INT       Array[689]

FLAG            INT       Array[2, 689]

IMAGECOR_630    FLOAT     Array[59, 16, 689]

IMAGECOR_765    FLOAT     Array[59, 16, 689]

RAY_IMAGECOR_630

FLOAT     Array[59, 16, 689]

RAY_IMAGECOR_765

FLOAT     Array[59, 16, 689]

AVG_630         FLOAT     Array[59, 4, 689]

AVG_765         FLOAT     Array[59, 4, 689]

FINVER_630      FLOAT     Array[59, 4, 689]

FINVER_765      FLOAT     Array[59, 4, 689]

RATIO           FLOAT     Array[59, 4, 689]

O2D_RESULT      FLOAT     Array[59, 4, 689]

TPALTITUDE      FLOAT     Array[59, 16, 689]

TPALTITUDEEND   FLOAT     Array[59, 16, 689]

TPLATITUDE      FLOAT     Array[59, 16, 689]

TPLONGITUDE     FLOAT     Array[59, 16, 689]

TPSOLARZEN      FLOAT     Array[59, 16, 689]

TPSOLARLT       FLOAT     Array[59, 16, 689]

SCALTITUDE      FLOAT     Array[689]

SCLATITUDE      FLOAT     Array[689]

SCLONGITUDE     FLOAT     Array[689]

SCSOLARZEN      FLOAT     Array[689]

MOONSEPANGLE    FLOAT     Array[689]

SUNLOSANGLE     FLOAT     Array[689]

EARTH_SUN       FLOAT       1.49500e+08

A630            FLOAT        0.00910000

TEMP            FLOAT           600.000

KAPPA           FLOAT       3.61680e-11

BETA1           FLOAT          0.800000

F0              FLOAT          0.600000

F1              FLOAT          0.400000

AV              FLOAT         0.0919000

A00             FLOAT         0.0876000

A11             FLOAT         0.0700000

TPDN            INT       Array[689]

8. References

Hecht, J. H., Christensen, A. B., Yee, J.-H., Crowley, G., Bishop, R. L., Budzien, S. A., Stephan, A. W., and Evans, J. S. (2015),  A new technique for remote sensing of O2 density from 140 to 180 km, Geophys. Res. Lett.,  42,  233– 240, doi:10.1002/2014GL062355.

Hickstein, Daniel, D.,  Stephen T. Gibson, Roman Yurchak, Dhrubajyoti D. Das, Mikhail Ryazanov, A direct comparison of high-speed methods for the numerical Abel transform,

Rev Sci Instrum 90, 065115 (2019) https://doi.org/10.1063/1.5092635